REACH – Registration, Evaluation and Authorisation of Chemicals

PRESIDENCY INTRODUCTION

This is the UK Presidency’s proposal for an amended text of the draft Regulation. It is based closely

on discussions in the Ad Hoc Working Party on Chemicals and among Ministers at the Council,

along with the many proposals recorded in the footnote documents on the Titles examined so far.

Some of the amendments have already been presented in earlier Presidency documents. It takes the

form of proposed revisions to the Commission’s proposal1 for Titles I (scope), II (registration), III

(data-sharing), IV (information in the supply chain), V (downstream users), VI (evaluation), VII

(authorisation), Title VIII (restrictions), Title IX (Agency), Title XII (competent authorities) and

Title XIII (enforcement). Any proposed revisions to Title X (classification and labelling), Title XI

(access to information) and Title XIV (transitional and final provisions) shall be made after the

meeting of the Working Party of 5 and 6 September. Proposed revisions are also made to Annexes

I, Ia, Ib (relating to Chemical Safety Reports and Safety Data Sheets), II, III (scope), IV – IX

(information requirements), XI (downstream user obligations), XIV (dossiers) and XV (socio-

economic analysis). No revisions to the other Annexes will be proposed by the Presidency.

The main factor in our considerations has been ensuring that the proposal fulfils the aims of

REACH as described in the Commission’s White Paper2, in particular the need to ensure improved

protection of environment and health whilst maintaining EU competitiveness.

1 See document COM (2003)644 final.

2 See document COM (2001)88 final.

OVERVIEW OF THE MAIN ELEMENTS

Title I – Scope and definitions

.. Consolidation in Title I of the scope and exemptions. Clarification of certain

exemptions (eg. waste, food and related substances).

.. Exemptions from Registration - Annex II is not amended but should be considered for

review once REACH comes into force. Clarification of Annex III exemptions,

particularly in relation to natural substances such as ores and minerals.

.. Proposed amendments to the Definitions (Article 3), although this Article remains open

pending further discussion of the remaining Titles.

Title II – Registration

1. Substances in articles

.. Minor modifications only to Article 6(1) (substances intentionally released).

.. Problems regarding Article 6(2) could be resolved by imposing an obligation on

producers and importers of articles to notify certain substances (those agreed under Title

VII as meeting the criteria for authorisation) where they are contained in articles above

a certain level.

2. Information to be submitted at registration

.. Registrants should be able to apply use and exposure categories voluntarily.

.. Quality assurance of the registration dossier by a competent third party to be voluntary.

3. Multiple registrants for one substance

.. All manufacturers or importers of the same substance should submit certain parts of the

registration dossier jointly.

.. Inclusion of a mechanism to allow differences of opinion to be expressed in relation to

these parts of the dossier without preventing a substance from being registered.

4. Information submitted depending on tonnage

.. Low volume substances (those manufactured or imported between 1-10 tonnes): where

a substance in this tonnage range meets simple criteria highlighting it as potentially of

concern, Annex V information to be provided. In other cases, only information that is

available to the registrant would need to be provided.

.. Criteria for the above are established in an Annex. These aim to be clear, simple for

industry to apply and relate to the information known about a substance’s properties,

uses and likely exposure.

.. As Annex V will only apply to a limited number of substances in this tonnage range, it

is proposed to include additional information requirements in relation to acute toxicity,

biodegradation and algal toxicity.

.. One test for reproductive toxicity is proposed for Annex VI.

.. No significant changes in requirements are proposed for Annexes VII and VIII.

.. Annexes IV – IX include technical revisions based, in particular, on the outcome of

discussions at expert level and in the Working Party under the Netherlands and

Luxembourg Presidencies.

.. The Presidency is not proposing to amend the testing methods in Annex X as they are

agreed at OECD level and uncontroversial. The Presidency is however minded to

provide for these test methods to be adopted through comitology instead.

5. Phase-in substances

.. Inclusion in the first phase of registration of substances that are potentially persistent,

bioaccumulative and toxic (PBT) or very persistent, very bioaccumulative (vPvB) – in

practice those classified as R50-53 under 67/548/EEC – and manufactured or imported

over 100 tonnes.

Title III – Data-sharing

.. Potential registrants required to share non-animal data (as well as animal data).

.. The sharing of costs to be agreed, in the majority of cases, amongst potential registrants

themselves in a fair, proportionate and non-discriminatory way, in particular in relation

to SMEs.

.. In cases where the sharing of costs cannot be resolved amongst potential registrants, a

clear and unambiguous provision to assign costs equally is included.

.. Two phases for pre-registration of phase-in substances, as proposed by the Commission.

During these phases, the publication of an Agency list of pre-registered substances

would trigger a submission of available hazard data by potential registrants who have

not yet pre-registered.

Title IV – Information in the supply chain

.. Inclusion of a requirement for safety data sheets to be provided for substances that are

persistent, bioaccumulative and toxic or very persistent and very bioaccumulative.

.. Clarification of the role of distributors in ensuring that information flows through the

supply chain.

.. Changes to Annexes I and Ia based, in particular, on discussions under the Luxembourg

Presidency.

Title V – Downstream Users

.. Annex Ib (Chemical Safety Assessments for Preparations) to be deleted.

.. The Presidency is not proposing to introduce a minimum tonnage below which

downstream users need not conduct a chemical safety assessment. However, the

Presidency recognises this is a key issue.

Title VI – Evaluation

1. Dossier evaluation

.. Responsibility for dossier evaluation (both in terms of testing proposals and compliance

checks) transferred to the Agency. The Agency will be able to decide how best to

discharge these obligations, including the possibility of using external sources as

provided by Article 83.

.. A minimum number of compliance checks to be performed. These to focus (although

not exclusively) on dossiers where disagreements are highlighted amongst registrants,

the quality of the dossier has not been independently verified or, for 1-10 tonne

substances, where the information specified in Annex V has not been submitted.

2. Substance evaluation

.. A single EU-wide rolling plan for substance evaluation prepared by the Agency with

input from the Member States.

.. Member State competent authorities to perform the evaluation and prepare draft

decisions. Member State competent authorities open to use expert institutes to perform

the evaluation.

Title VII – Authorisation

1. Scope

.. Scope of authorisation as in articles 54 (a) to (e) of the Commission proposal.

Clarification of the wording of Article 54(f).

2. List of substances meeting the authorisation criteria

.. A list of substances agreed as meeting the criteria of Article 54 to be published by the

Agency rather than in a separate Annex for increased flexibility and simplicity.

3. Granting of authorisations

.. Authorisations should be granted where a substance is adequately controlled.

.. In order to encourage the development of safer substitutes, all authorisations should lay

down time-limited review periods and should be subject, in most cases, to monitoring.

Title VIII – Restrictions

.. Inclusion of a transition period after REACH comes into force to allow Member States

to update existing national legislation.

.. Clarifications are proposed to Annexes XIV and XV.

Title VIIIa (new) – Fees

.. A separate Title is introduced to clarify when and how fees may be charged by the

Agency. The fee structure will be specified in a Commission Regulation.

Title IX – The Agency

.. Changes are proposed based, in particular, on the discussion in COREPER on 15 July

2005 including:

– one representative per Member State on the Management Board.

– clarification of the procedures for appeal.

– specification that the language regime of the Agency should be according to

Regulation 1/58.

– deletion of the reference to the seat of the Agency.

Title XII – Competent Authorities

.. Clarifications of the text are proposed.

Title XIII – Enforcement

.. Clarification of the sanctions regime to be established by Member States.

3 OJ C

4 OJ C

5 OJ C

REGULATION OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL

concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals

(REACH), establishing a European Chemicals Agency and amending Directive

1999/45/EC and Regulation (EC) No 850/2004 on Persistent Organic Pollutants

THE EUROPEAN PARLIAMENT AND THE COUNCIL OF THE EUROPEAN UNION,

Having regard to the Treaty establishing the European Community, and in particular

Article 95 thereof,

Having regard to the proposal from the Commission3,

Having regard to the opinion of the European Economic and Social Committee4,

Acting in accordance with the procedure laid down in Article 251 of the Treaty5,

Whereas:

{…}

HAVE ADOPTED THIS REGULATION:

6 As set out in the Communication from the Commission on the precautionary principle,

COM(2000) 1 final.

7 OJ L 159, 29.6.1996, p. 1.

TITLE I

GENERAL ISSUES

CHAPTER 1

AIM, SCOPE AND APPLICATION

Article 1

Aim and scope

1. The purpose of this Regulation is to ensure a high level of protection of health and

the environment as well as the free circulation of substances on the internal market.

1bis. This Regulation lays down provisions on substances and preparations within the

meaning of Article 3. These provisions shall apply to the manufacture, placing on the

market or use of such substances on their own, in preparations or in articles and to

the placing on the market of preparations, if so stated.

2. [deleted]

3. This Regulation is based on the principle that it is up to manufacturers, importers and

downstream users to ensure that they manufacture, place on the market or use such

substances that do not adversely affect human health or the environment. Its

provisions are underpinned by the precautionary principle6.

Article 2

Application

1. This Regulation shall not apply to:

(a) radioactive substances within the scope of Council Directive 96/29/Euratom7;

(b) substances, on their own, in a preparation or in an article, which are subject to

customs supervision, provided that they do not undergo any treatment or

processing, and which are in temporary storage, or in a free zone or free

warehouse with a view to re-exportation, or in transit;

(c) non-isolated intermediates;

(d) the carriage of dangerous substances and dangerous substances in dangerous

preparations by rail, road, inland waterway, sea or air.

8 OJ L 183, 29.6.1989, p. 1.

9 OJ L 131, 5.5.1998, p. 11.

10 OJ L 311, 28.11.2001, p. 1.

11 OJ L 311, 28.11.2001, p. 67.

1bis. Waste as defined in Council Directive 75/442/EEC and the amendments thereto, is

not a substance, preparation or article within the meaning of Article 3 of this

Regulation.

2. This Regulation shall apply without prejudice to Community workplace legislation,

including:

(a) Council Directive 89/391/EEC8;

(b) Directive 90/394/EEC;

(c) Council Directive 98/24/EC9;

3. The provisions of Title II, V, VI and VII shall not apply to the extent that a substance

is used:

(a) in medicinal products for human or veterinary use within the scope of

Regulation (EEC) No 2309/93, Directive 2001/82/EC of the European

Parliament and of the Council10 and Directive 2001/83/EC of the European

Parliament and of the Council11;

(b) in food or feedingstuffs according to Regulation (EC) No 178/2002 of the

European Parliament and of the Council including used:

(i) as a food additive in foodstuffs within the scope of Council Directive

89/107/EEC;

(ii) as a flavouring in foodstuffs within the scope of Council Directive

88/388/EEC and Commission Decision 1999/217/EC;

(iii) as an additive in feedingstuffs within the scope of Regulation (EC) No

1831/2003 of the European Parliament and of the Council ;

(iv) in animal nutrition within the scope of Council Directive 82/471/EEC.

4. The following shall be exempted from Title II, V and VI:

(a) substances included in Annex II, about which sufficient information is known

that they are considered to cause minimum risk;

(b) substances covered by Annex III, for which registration is deemed

inappropriate or unnecessary and their exemption from this Title does not

prejudice the aims of this Regulation;

(c) substances on their own or in preparations, registered in accordance with Title

II, exported from the Community by an actor in the supply chain and

re-imported into the Community by the same or another actor in the same

supply chain who shows that:

(i) the substance being re-imported is the same as the exported substance;

(ii) he has been provided with the information in accordance with Articles 30

and 31 relating to the exported substance.

(d) Substances, on their own, in preparations or in articles, which have been

registered in accordance with Title II and which are recovered in the

Community if:

(i) the substance that results from the recovery process is the same as the

substance that has been registered in accordance with Title II; and

(ii) the establishment undertaking the recovery has available the information

required by Articles 29 or 30 relating to the substance that has been

registered in accordance with Title II.

5. On-site isolated intermediates or transported isolated intermediates shall be exempted

from Chapter 2 of Title II, with the exception of Article 7, and Chapter 3 of Title II,

without prejudice to Chapters 4, 5 and 6, and Title VII.

6. The provisions of Title VI shall not apply to polymers.

CHAPTER 2

DEFINITIONS

Article 3

Definitions

For the purposes of this Regulation:

1. Substance means a chemical element and its compounds in the natural state or

obtained by any manufacturing process, including any additive necessary to preserve

its stability and any impurity deriving from the process used, but excluding any

solvent which may be separated without affecting the stability of the substance or

changing its composition;

2. Preparation means a mixture or solution composed of two or more substances;

3. Article means an object composed of one or more substances or preparations which

during production is given a specific shape, surface or design determining its end use

function to a greater degree than its chemical composition does;

4. Polymer means a substance consisting of molecules characterised by the sequence of

one or more types of monomer units. Such molecules must be distributed over a

range of molecular weights wherein differences in the molecular weight are primarily

attributable to differences in the number of monomer units. A polymer comprises the

following:

(a) a simple weight majority of molecules containing at least three monomer units

which are covalently bound to at least one other monomer unit or other

reactant;

(b) less than a simple weight majority of molecules of the same molecular weight.

In the context of this definition a ‘monomer unit’ means the reacted form of a

monomer substance in a polymer;

5. Registrant means the manufacturer or the importer or the producer or importer of an

article submitting a registration for a substance;

6. Manufacturing means production or extraction of substances in the natural state;

7. Manufacturer means any natural or legal person established within the Community

who manufactures a substance within the Community;

8. Import means the physical introduction into the customs territory of the Community;

9. Importer means any natural or legal person established within the Community who is

responsible for import;

10. Placing on the market means supplying or making available, whether in return for

payment or free of charge, to a third party. Import shall be deemed to be placing on

the market;

11. Downstream user means any natural or legal person established within the

Community, other than the manufacturer or the importer, who uses a substance,

either on its own or in a preparation, in the course of his industrial or professional

activities. A distributor or a consumer is not a downstream user. A re-importer

exempted pursuant to Article 4(2)(c) shall be regarded as a downstream user;

12. Use means any processing, formulation, consumption, storage, keeping, treatment,

filling into containers, transfer from one container to another, mixing, production of

an article or any other utilisation;

13. Distributor means any natural or legal person established within the Community,

including a retailer, who only stores and places on the market a substance, on its own

or in a preparation, for third parties;

14. Intermediate means a substance that is manufactured for and consumed in or used for

chemical processing in order to be transformed into another substance (hereinafter

called synthesis):

(a) non-isolated intermediate means an intermediate that during synthesis is not

intentionally removed (except for sampling) from the equipment in which the

synthesis takes place. Such equipment includes the reaction vessel, its ancillary

equipment, and any equipment through which the substance(s) pass(es) during

a continuous flow or batch process as well as the pipework for transfer from

one vessel to another for the purpose of the next reaction step, but it excludes

tanks or other vessels in which the substance(s) are stored after the

manufacture;

12 OJ L 259, 15.10.1979, p. 10.

13 OJ L 154, 5.6.1992, p. 1.

(b) on-site isolated intermediate means an intermediate not meeting the criteria of

a non-isolated intermediate and where the manufacture of the intermediate and

the synthesis of (an)other substance(s) from that intermediate take place on the

same site, operated by one more legal entities;

(c) transported isolated intermediate means an intermediate not meeting the

criteria of a non-isolated intermediate and transported between or supplied to

other sites;

15. Site means a single location, in which, if there is more than one manufacturer of

(a) substance(s), certain infrastructure and facilities are shared;

16. Actors in the supply chain means all manufacturers and/or importers and/or

downstream users;

17. [deleted]18. [deleted]19. Competent authority means the authority or authorities

or bodies established by the Member States to carry out the obligations arising from

this Regulation;

20. Phase-in substance means a substance which, over the 15 years preceding the entry

into force of this Regulation, meets at least one of the following criteria:

(a) it was manufactured in or imported into the Community, or the countries

acceding to the European Union on 1 May 2004, by a manufacturer or importer

and is listed in the European Inventory of Existing Commercial Chemical

Substances (EINECS);

(b) it was manufactured in the Community, or in the countries acceding to the

European Union on 1 May 2004, but not placed on the market by the

manufacturer or importer;

(c) it was placed on the market in the Community, or in the countries acceding to

the European Union on 1 May 2004, and between 18 September 1981 and 31

October 1993 inclusive it was also placed on the market by the manufacturer or

importer and was considered as having been notified in accordance with the

first indent of Article 8 (1) of Directive 67/548/EEC, as amended by Directive

79/831/EEC12, but does not meet the definition of a polymer set out in

Directive 67/548/EEC, as amended by Directive 92/32/EEC13;

provided the manufacturer or importer has documentary evidence of this.

21. Notified substance means a substance for which a notification has been submitted

and which could be placed on the market in accordance with Directive 67/548/EEC;

22. Product and process orientated research and development means any scientific

development related to product development, the further development of a substance,

on its own, in preparations or in articles in the course of which pilot plant or

production trials are used to develop the production process and/or to test the fields

of application of the substance;

23. Scientific research and development means any scientific experimentation, analysis

or chemical research carried out under controlled conditions in a volume less than 1

tonne per year;

24. Registrant’s own use means an industrial or professional use by the registrant;

25. Identified use means a use of a substance on its own or in a preparation, or a use of a

preparation, that is intended by an actor in the supply chain, including his own use,

or that is made known to him in writing by an immediate downstream user and that is

covered in the safety data sheet communicated to the downstream user concerned;

26. [deleted]

26a. Full study report means a complete and comprehensive description of the activity

performed to generate the information. This covers the complete scientific paper as

published in the literature describing the study performed or the full report prepared

by the test house describing the study performed;

27. Robust study summary means a detailed summary of the objectives, methods, results

and conclusions of a full study report providing sufficient information to make an

independent assessment of the study minimising the need to consult the full study

report;

27a. Study summary means a summary of the objectives, methods, results and conclusions

of a full study report providing sufficient information to make an assessment of the

relevance of the study hazard assessment;

28. Per year means per calendar year unless stated otherwise;

29. Restriction means any condition for or prohibition of the manufacture, use or placing

on the market;

30. Supplier of a substance or a preparation means any manufacturer, importer,

downstream user or distributor placing on the market a substance, on its own or in a

preparation, or a preparation;

31. Recipient of a substance or a preparation means a downstream user or a distributor

being supplied with a substance or a preparation;

32. Small and Medium Enterprise (SME) means small and medium-sized enterprises

according to the definition contained in Commission Recommendation concerning

the definition of micro, small and medium-sized enterprises;

33. Exposure scenario means the set of conditions that describe how the substance is

manufactured or used during its life-cycle and how the manufacturer or importer

controls, or recommends downstream users to control, exposures of humans and the

environment. These exposure scenarios may cover one specific process or use or

several processes or uses as appropriate;

34. Use and exposure category means an exposure scenario covering a wide range of

processes or uses;

35. Substances which occur in nature means a substance as such, unprocessed or

processed only by manual, mechanical or gravitational means; by dissolution, in

water, by flotation, or by heating solely to remove water, or which is extracted from

air by any means;

36. Not chemically modified substance means a substance whose chemical structure

remains unchanged, even if it has undergone a chemical process or treatment, for

instance to remove impurities.

TITLE II

REGISTRATION OF SUBSTANCES

CHAPTER 1

SCOPE

[deleted]

Article 4

Scope

[deleted]

CHAPTER 2

GENERAL OBLIGATION TO REGISTER AND

INFORMATION REQUIREMENTS

Article 4a

No data, no market

Subject to Article 21, substances shall not be manufactured in the Community or imported

unless they have been registered in accordance with the relevant provisions of this Title.

Article 5

General obligation to register substances on their own or in preparations

1. Save where this Regulation provides otherwise, any manufacturer of a substance,

either on its own or in a preparation, in quantities of 1 tonne or more per year shall

submit a registration to the Agency.

Save where this Regulation provides otherwise, any importer of a substance, either

on its own or in one or more preparation(s), in quantities of 1 tonne or more per year

shall submit a registration to the Agency.

2. For monomers that are used as on-site isolated intermediates or transported isolated

intermediates, Articles 15 and 16 shall not apply.

3. Any manufacturer or importer of a polymer shall submit a registration to the Agency

for the monomer substance(s) or any other substance(s), that have not already been

registered by an actor up the supply chain, if both the following conditions are met:

(a) the polymer consists of 2% weight by weight (w/w) or more of such monomer

substance(s) or other substance(s) in the form of monomeric units and

chemically bound substance(s);

(b) the total quantity of such monomer substance(s) or other substance(s) makes up

1 tonne or more per year.

4. A submission for registration shall be accompanied by the fee in accordance with

Title VIIIa.

Article 6

General obligation to register substances in articles

1. Any producer or importer of articles shall submit a registration to the Agency for any

substance contained in those articles, if all the following conditions are met:

(a) the substance is present in those articles in quantities totalling over 1 tonne per

producer or importer per year;

(b) the substance meets the criteria for classification as dangerous in accordance

with Directive 67/548/EEC;

(c) the substance is intended to be released under normal and reasonably

foreseeable conditions of use.

A submission for registration shall be accompanied by the fee in accordance with

Title VIIIa.

2. Any producer or importer of articles shall notify the Agency, in accordance with

paragraph 3, if a substance meets the criteria in Article 54 and is identified according

to Article 56(1), if all the following conditions are met:

(a) the substance is present in those articles in quantities totalling over 1 tonne per

producer or importer per year;

(b) the substance is present in those articles above a concentration of 0.1% weight

by weight (w/w).

(c) [deleted]

(d) [deleted]

3. The information to be notified shall include the following:

(a) the identity and contact details of the producer or importer as specified in

section 1 of Annex IV, with the exception of the use sites;

(b) the registration number(s) referred to in Article 18 (1), if available;

(c) the identity of the substance(s) as specified in section 2 of Annex IV;

(d) the classification of the substance as specified in sections 4.1 and 4.2 of Annex

IV;

(e) a brief description of the use(s) of the substance in the article as specified in

section 3.5 of Annex IV;

(f) the tonnage range of the substance, such as 1-10 tonnes, 10-100 tonnes and so

on.

4. The Agency may take decisions requiring producers or importers of articles to

register, in accordance with this Title, any substance contained in those articles and

notified in accordance with paragraph 3. The Agency shall set a deadline for

submission of the registration.

A submission for registration shall be accompanied by the fee in accordance with

Title VIIIa.

5. Paragraphs 1 to 4 shall not apply to substances that have already been registered for

that use.

6. Paragraphs 1 and 4 shall apply 3 months after the deadline specified in Article 21(3).

Paragraphs 2 and 3 shall apply 6 months after a substance is identified according to

Article 56(1), provided that 6 months have elapsed from the deadline specified in

Article 21(1).

7. Any measures for the implementation of paragraphs 1 to 6 shall be adopted in

accordance with the procedure referred to in Article 130(3).

Article 6a

Only representative of a non-Community manufacturer

1. A natural or legal person established outside the Community who manufactures a

substance on its own, in preparations or in articles, formulates a preparation or

produces an article that is imported into the Community may by mutual agreement

appoint a natural or legal person established in the Community to fulfil, as his only

representative, the obligations on importers under this Title.

2. The representative shall also comply with all other obligations of importers under

this Regulation. To this end, he shall have a sufficient background in the practical

handling of substances and the information related to them and, without prejudice to

Article 33, he shall keep available and up-to-date information on quantities imported

and customers sold to, as well as information on the supply of the latest update of the

safety data sheet.

3. If a representative is appointed in accordance with paragraphs 1 and 2, the

non-Community exporter shall inform the importer(s) within the same supply chain

of the appointment. These importers shall be regarded as downstream users for the

purposes of this Regulation.

Article 7

Exemption from the general obligation to register

for product and process orientated research and development

(PPORD)

1. Articles 4a, 5, 6, 15, 16 and 19 shall not apply for a period of five years to a

substance manufactured in the Community or imported for the purposes of product

and process orientated research and development by a manufacturer or importer, by

himself or in co-operation with listed customers and in a quantity which is limited to

the purpose of product and process orientated research and development.

2. For the purpose of paragraph 1, the manufacturer or importer shall notify the Agency

of the following information:

(a) the identity of the manufacturer or importer as specified in section 1 of Annex

IV;

(b) the identity of the substance, as specified in section 2 of Annex IV;

(c) the classification of the substance as specified in section 4 of Annex IV,if any;

(d) the estimated quantity as specified in section 3.1 of Annex IV;

(e) the list of customers referred to in paragraph 1, including their names and

addresses; and

(f) sufficient information on the research and development programme to enable

the Agency to take informed decisions under paragraphs 4 and 7.

The notification shall be accompanied by the fee in accordance with Title VIIIa.

The period set out in paragraph 1 shall begin at receipt of the notification at the

Agency.

3. The Agency shall check the completeness of the information supplied by the notifier

and Article 18(2) shall apply adapted as necessary. The Agency shall assign a

number to the notification and a notification date, which shall be the date of receipt

of the notification at the Agency, and shall forthwith communicate that number and

date to the manufacturer or importer concerned. The Agency shall also communicate

this information to the competent authority of the Member State(s) concerned.

4. The Agency may decide to impose conditions with the aim of ensuring that the

substance or the preparation or article in which the substance is incorporated will be

handled only by staff of listed customers as referred to in paragraph 2(e) in

reasonably controlled conditions, in accordance with the requirements of legislation

for the protection of workers and the environment, and will not be made available to

the general public at any time either on its own or in a preparation or article and that

remaining quantities will be re-collected for disposal after the exemption period.

5. In the absence of any indication to the contrary, the manufacturer or importer of the

substance may manufacture or import the substance not earlier than two weeks after

the notification.

6. The manufacturer or importer shall comply with any conditions imposed by the

Agency in accordance with paragraph 4.

7. The Agency may decide to extend the five-year exemption period by a further

maximum of five years or, in the case of substances to be used exclusively in the

development of medicinal products for human or veterinary use, for a further

maximum of 10 years, upon request if the manufacturer or importer can demonstrate

that such an extension is justified by the research and development programme.

8. The Agency shall forthwith communicate any draft decisions to the competent

authorities of each Member State in which the manufacture, import or product and

process orientated research takes place.

When taking decisions as provided for in paragraphs 4 and 7, the Agency shall take

into account any comments made by such competent authorities.

9. The Agency and the competent authorities of the respective Member States shall

always keep confidential the information submitted in accordance with paragraphs 1

to 8.

10. An appeal may be brought, in accordance with Articles 87, 88 and 89, against

Agency decisions under paragraphs 4 and 7.

Article 8

Substances in plant protection and biocidal products

[deleted]

Article 9

Information to be submitted for general registration purposes

A registration required by Article 5 or by Article 6(1) or (4) shall include all the following

information:

(a) a technical dossier including:

(i) the identity of the manufacturer(s) or importer(s) as specified in section 1 of

Annex IV;

(ii) the identity of the substance as specified in section 2 of Annex IV;

(iii) information on the manufacture and use(s) of the substance as specified in

section 3 of Annex IV; this information shall represent all the registrant’s

identified use(s). This information may be based, if the registrant deems

appropriate, on use and exposure categories;

(iv) the classification and labelling of the substance as specified in section 4 of

Annex IV;

(v) guidance on safe use of the substance as specified in Section 5 of Annex IV;

(vi) study summaries of the information derived from the application of Annexes V

to IX;

(vii) robust study summaries of the information derived from the application of

Annexes V to IX, if required under Annex I;

(viia) a statement as to whether or not the quality of data under (iii), (iv), (vi) and

(vii) has been assessed by a competent third-party;

(viii) [deleted]

(ix) proposals for testing where required by the application of Annexes V to IX;

(ixa) for substances in quantities of 1 to 10 tonnes, exposure information as specified

in section 6 of Annex IV;

(x) [deleted]

The registrant shall be in legitimate possession of or have permission to refer to the

full study report summarised under (vi) and (vii) for the purpose of registration,

unless the Agency has provided the registrant with the summaries or robust study

summaries under Article 25(5).

(b) a chemical safety report when required under Article 13.

Article 10

Joint submission of data by multiple registrants

1. When a substance is intended to be manufactured in the Community by one or more

manufacturers and/or imported by one or more importers, the following shall apply.

As regards the information specified in Article 9(a)(iv), (vi), (vii) and (ix), and a

statement under Article 9(a)(viia) as regards the information specified in Article 9(a)

(iv), (vi) and (vii), this shall first be submitted by the one manufacturer or importer

acting with the agreement of the other assenting manufacturer(s) or importer(s) (“the

lead registrant”). If a registrant disagrees with the lead registrant on the selection of

the information specified in Article 9(a)(iv), (vi), (vii) and/or (ix), he shall submit an

explanation as to the nature of the disagreement, along with any accompanying

information that he considers relevant, when subsequently submitting his registration

dossier.

Each registrant shall subsequently submit separately the information specified in

Article 9(a)(i), (ii), (iii) and (ixa), and a statement under Article 9(a)(viia) as regards

the information specified in Article 9(a)(iii).

The registrants may decide themselves whether to submit the information specified

in Article 9(a)(v) and (b) separately or whether one manufacturer or importer is to

submit this information on behalf of the others.

2. Each manufacturer or importer need only comply with paragraph 1 for items of

information specified in Article 9(a)(iv), (vi), (vii) and (ix) that are required for the

purposes of registration within his tonnage band in accordance with Article 11.

3. A submission for registration shall be accompanied by the fee in accordance with

Title VIIIa.

Article 11

Information to be submitted depending on tonnage

1. The technical dossier referred to in Article 9 (a) shall include under points (vi) and

(vii) of that provision all physicochemical, toxicological and ecotoxicological

information that is relevant and available to the registrant and as a minimum the

following:

(a) the information specified in Annex V for substances manufactured or imported

in quantities of 1 tonne or more per year per manufacturer or importer and

meeting the criteria in Annex Ic;

(b) the information specified in Annexes V and VI for substances manufactured or

imported in quantities of 10 tonnes or more per year per manufacturer or

importer;

(c) the information specified in Annexes V and VI and testing proposals for the

provision of the information specified in Annex VII for substances

manufactured or imported in quantities of 100 tonnes or more per year per

manufacturer or importer;

(d) the information specified in Annexes V and VI and testing proposals for the

provision of the information specified in Annexes VII and VIII for substances

manufactured or imported in quantities of 1 000 tonnes or more per year per

manufacturer or importer.

2. As soon as the quantity of a substance per manufacturer or importer that has already

been registered reaches the next tonnage threshold, the manufacturer or importer

shall inform the Agency immediately of the additional information he would require

under paragraph 1. Article 24 paragraphs (4) - (6) shall apply.

Article 12

General requirements for generation of information on intrinsic properties of substances

1. Information on intrinsic properties of substances may be generated by means other

than tests, in particular through the use of qualitative or quantitative structure-activity

relationship models or from information from structurally related substances

(grouping), provided that the conditions set out in Annex IX are met. Testing in

accordance with Annex VII and VIII may be omitted where justified by information

on exposure and implemented risk management measures as specified in Annex

IX(3).

2. Where tests on substances are required to generate information on intrinsic properties

of substances, they shall be conducted in accordance with the test methods laid down

in Annex X or other international test methods recognised by the Agency as being

equivalent.

Information on intrinsic properties of substances may be generated in accordance

with other test methods provided that the conditions set out in Annex IX are met.

3. Laboratory tests and analyses shall be carried out in compliance with the principles

of good laboratory practice provided for in Directive 2004/10/EC or other

international standard recognised as being equivalent by the Agency and with the

provisions of Directive 86/609/EEC, if applicable.

4. If a substance has already been registered, a new registrant shall be entitled to refer

to the study summaries or robust study summaries, for the same substance submitted

earlier, provided that he can show that the substance that he is now registering is the

same as the one previously registered, including the degree of purity and the nature

of impurities, and that the previous registrant(s) has given permission to refer to the

full study reports for the purpose of registration.

5. However, a new registrant shall not refer to such studies in order to provide the

information required in section 2 of Annex IV.

Article 13

Chemical safety report and

duty to apply and recommend risk reduction measures

1. Without prejudice to Article 4 of Directive 98/24/EC, a chemical safety assessment

shall be performed and a chemical safety report completed for all substances subject

to registration in accordance with this Chapter if the registrant manufactures or

imports such a substance in quantities of 10 tonnes or more per year.

The chemical safety report shall document the chemical safety assessment which

shall be conducted in accordance with paragraphs 2 to 7 and with Annex I for either

each substance on its own or in a preparation or a group of substances.

2. A chemical safety assessment in accordance with paragraph 1 need not be performed

for a substance which is present in a preparation if the concentration of the substance

in the preparation is less than the lowest of any of the following:

(a) the applicable concentrations defined in the table of Article 3(3) of

Directive 1999/45/EC;

(b) the concentration limits given in Annex I to Directive 67/548/EEC;

(c) the concentration limits given in Part B of Annex II to Directive 1999/45/EC;

(d) the concentration limits given in Part B of Annex III to Directive 1999/45/EC;

(e) the concentration limits given in an agreed entry in the classification and

labelling inventory established under Title X;

(f) 0.1% weight by weight (w/w), if the substance meets the criteria in Annex XII.

14 OJ L 40, 11.2.1989, p. 38.

15 OJ L 262 , 27.9.1976, p. 169.

3. A chemical safety assessment of a substance shall include the following steps:

(a) human health hazard assessment;

(b) physicochemical hazard assessment;

(c) environmental hazard assessment;

(d) PBT and vPvB assessment.

4. If, as a result of carrying out steps (a) to (d) of paragraph 3, the manufacturer or

importer concludes that the substance meets the criteria for classification as

dangerous in accordance with Directive 67/548/EEC or is assessed to be a PBT or

vPvB, the chemical safety assessment shall include the following additional steps:

(a) the generation of exposure scenario(s) (or the identification of relevant use and

exposure categories if appropriate);

(a bis) exposure assessment;

(b) risk characterisation.

The exposure scenarios (where appropriate the use and exposure categories),

exposure assessment and the risk characterisation shall address all identified uses of

the manufacturer or importer.

5. The chemical safety report need not include consideration of the risks to human

health from the following end uses:

(a) in food contact materials within the scope of Council Directive 89/109/EEC14;

(b) in cosmetic products within the scope of Council Directive 76/768/EEC15.

6. Any manufacturer or importer shall identify and apply the appropriate measures to

adequately control the risks identified in the chemical safety assessment, and where

suitable, recommend them in the safety data sheets which he supplies in accordance

with Article 29.

7. Any manufacturer or importer required to conduct a chemical safety assessment shall

keep his chemical safety report available and up to date.

16 OJ L 230, 19.8.1991, p. 1.

17 OJ L 366, 15.12.1992, p. 10.

18 OJ L 98, 7.4.2001, p. 6.

19 OJ L 224, 21.8.2002, p. 23.

20 OJ L 192, 31.7.2003, p. 40.

21 OJ L 123, 24.4.1998, p. 1.

22 OJ L

CHAPTER 2A

SUBSTANCES REGARDED AS BEING REGISTERED

Article 13a

Substances in plant protection and biocidal products

1. Active substances and co-formulants manufactured or imported for use in plant

protection products only and included either in Annex I to Council

Directive 91/414/EEC16 or in Commission Regulation (EEC) No 3600/9217,

Commission Regulation (EC) No 703/200118, Commission Regulation (EC) No

1490/200219, Commission Decision 2003/565/EC20 and for any substance for which

a Commission Decision on the completeness of the dossier has been taken pursuant

to Article 6 of Directive 91/414/EEC shall be regarded as being registered and the

registration as completed for manufacture or import for the use as a plant protection

product and therefore as fulfilling the requirements of Chapters 2, 6 and of

Article 20.

2. Active substances manufactured or imported for use in biocidal products only and

included either in Annexes I, IA or IB to Directive 98/8/EC of the European

Parliament and of the Council21 or in Commission Regulation (EC)

No …/…{Second Review Regulation}22, until the date of the decision referred to in

the second subparagraph of Article 16(2) of Directive 98/8/EC, shall be regarded as

being registered and the registration as completed for manufacture or import for the

use in a biocidal product and therefore as fulfilling the requirements of Chapters 2, 6

and of Article 20.

Article 13b

Duties of the Commission, the Agency and registrants of substances regarded as being

registered

1. The Commission or the relevant Community body shall make information equivalent

to that required by Article 9 available to the Agency for substances registered

according to Article 13a. The Agency shall include this information or a reference

thereto in its databases and notify the Competent Authorities thereof by the deadline

in Article 26(2)(a).

2. Articles 19, 23, 24, 25, 26, 26a shall not apply to uses of substances registered

according to Article 13a.

CHAPTER 3

REGISTRATION OF POLYMERS

Article 14

Polymers

Polymers are exempted from registration under this Title.

CHAPTER 4

OBLIGATION TO REGISTER AND

INFORMATION REQUIREMENTS

FOR CERTAIN TYPES OF ISOLATED INTERMEDIATES

Article 15

Registration of on-site isolated intermediates

1. Any manufacturer of an on-site isolated intermediate in quantities of 1 tonne or more

per year shall submit a registration to the Agency for the on-site isolated

intermediate.

2. A registration for an on-site isolated intermediate shall include all the following

information, to the extent that the manufacturer is able to submit it without any

additional testing:

(a) the identity of the manufacturer as specified in section 1 of Annex IV;

(b) the identity of the intermediate as specified in section 2 of Annex IV ;

(c) the classification of the intermediate as specified in section 4 of Annex IV;

(d) any available existing information on physicochemical, human health or

environmental properties of the intermediate. Where a full study report is

available, a study summary shall be submitted;

(e) a brief general description of the use, as specified in section 3.5 of Annex IV;

(f) details of the risk management measures applied.

The registrant shall be in legitimate possession of or have permission to refer to the

full study report summarised under (d) for the purpose of registration.

The registration shall be accompanied by a fee in accordance with Title VIIIa.

3. Paragraph 2 shall apply only to on-site isolated intermediates if the manufacturer

confirms that the substance is only manufactured and used under strictly controlled

conditions in that it is rigorously contained by technical means during its whole

lifecycle. Control and procedural technologies shall be used to minimise emission

and any resulting exposure.

If these conditions are not fulfilled, the registration shall include the information

specified in Article 9.

Article 16

Registration of transported isolated intermediates

1. Any manufacturer or importer of a transported isolated intermediate in quantities of

1 tonne or more per year shall submit a registration to the Agency for the transported

isolated intermediate.

2. A registration for a transported isolated intermediate shall include all the following

information:

(a) the identity of the manufacturer or importer as specified in section 1 of

Annex IV;

(b) the identity of the intermediate as specified in section 2 of Annex IV;

(c) the classification of the intermediate as specified in section 4 of Annex IV;

(d) any available existing information on physicochemical, human health or

environmental properties of the intermediate. Where a full study report is

available, a study summary shall be submitted;

(e) a brief general description of the use, as specified in section 3.5 of Annex IV;

(f) information on risk management measures applied and recommended to the

user in accordance with paragraph 4.

The registrant shall be in legitimate possession of or have permission to refer to the

full study report summarised under (d) for the purpose of registration.

The registration shall be accompanied by a fee in accordance with Title VIIIa.

3. A registration for a transported isolated intermediate in quantities of more than

1 000 tonnes per year per manufacturer or importer shall include the information

specified in Annex V in addition to the information required under paragraph 2.

For the generation of this information, Article 12 shall apply.

4. Paragraphs 2 and 3 shall apply only to transported isolated intermediates if the

manufacturer or importer confirms himself or states that he has received

confirmation from the user that the synthesis of (an)other substance(s) from that

intermediate takes place on other sites under the following strictly controlled

conditions:

(a) the substance is rigorously contained by technical means during its whole

lifecycle including manufacture, , purification, cleaning and maintenance,

sampling, analysis, loading and unloading of equipment or vessels, waste

disposal or purification and storage;

(b) Procedural and control technologies shall be used that minimise emission and

any resulting exposure;

(c) only properly trained and authorised personnel handle the substance;

(d) in the case of cleaning and maintenance works, special procedures such as

purging and washing are applied before the system is opened and entered;

(e) [deleted]

(f) in cases of accident and where waste is generated, procedural and/or control

technologies are used to minimise emissions and the resulting exposure during

purification or cleaning and maintenance procedures;

(g) substance-handling procedures are well documented and strictly supervised by

the site operator.

(h) [deleted]

If the conditions listed in the first subparagraph are not fulfilled, the registration shall

include the information specified in Article 9.

Article 17

Joint submission of data on isolated intermediates by multiple registrants

1. When an on-site isolated intermediate or transported isolated intermediate is intended

to be manufactured in the Community by one or more manufacturers and/or imported

by one or more importers, the following shall apply.

The information specified in Article 15(2) (c) and (d) and 16(2) (c) and (d) shall first

be submitted by one manufacturer or importer acting with the agreement of the other

assenting manufacturer(s) or importer(s) (“the lead registrant”). If a registrant

disagrees with the lead registrant on the selection of the information specified in

Article 15(2) (c) and (d) and 16(2) (c) and (d), he shall submit an explanation as to

the nature of the disagreement, along with any accompanying information that he

considers relevant, when subsequently submitting his registration.

Each registrant shall subsequently submit separately the information specified in

Article 15(2)(a), (b), (e) and (f) and Article 16(2)(a) (b), (e) and (f).

2. A submission for registration shall be accompanied by the fee in accordance with

Title VIIIa.

CHAPTER 5

COMMON PROVISIONS FOR ALL REGISTRATIONS

Article 18

Duties of the Agency

1. The Agency shall assign a submission number to each registration, which is to be

used for all correspondence regarding the registration until the registration is deemed

to be complete, and a submission date, which shall be the date of receipt of the

registration at the Agency.

2. The Agency shall undertake a completeness check of each registration in order to

ascertain that all the elements required under Articles 9 and 11 or under Articles 15 or

16, as well as the registration fee referred to in Articles 5(4), 6(1), 6(4), 15(2) or 16(2),

have been provided. The completeness check shall not include an assessment of the

quality or the adequacy of any data or justifications submitted.

The Agency shall undertake the completeness check within three weeks of the

submission date, or within three months of the relevant deadline of Article 21, as

regards registrations of phase-in substances submitted in the course of the 2-month

period immediately preceding that deadline.

If a registration is incomplete, the Agency shall inform the registrant, before expiry of

the three week or three month period referred to in the previous subparagraph, as to

what further information is required in order for the registration to be complete, while

setting a reasonable deadline for this. The registrant shall update his registration and

submit it to the Agency within the deadline set. The Agency shall confirm the

submission date of the further information to the registrant. The Agency shall perform

a further completeness check, considering the further information submitted.

The Agency shall reject the registration if the registrant fails to complete his

registration within the deadline set. The registration fee shall not be reimbursed in

such cases.

2bis. Once the registration is complete, the Agency shall assign a registration number to the

substance concerned and a registration date, which shall be the same as the submission

date. The Agency shall without delay communicate the registration number and

registration date to the registrant concerned. The registration number should be used

for all subsequent correspondence regarding registration.

3. The Agency shall notify the competent authority of the relevant Member State within

30 days of the submission date, that the following information is available in the

Agency database: the registration dossier together with the submission or registration

number, the submission or registration date, the result of the completeness check and

any request for further information and deadline set in accordance with the third

subparagraph of paragraph 2. The relevant Member State shall be the Member State

within which the manufacture takes place or the importer is established.

If the manufacturer has production sites in more than one Member State, the relevant

Member State shall be the one in which the head office of the manufacturer is

established. The other Member States where the production sites are established shall

also be notified.

The Agency shall forthwith notify the competent authority of the relevant Member

State(s) when any further information submitted by the registrant is available on the

Agency database.

4. An appeal may be brought, in accordance with Articles 87, 88 and 89, against

Agency decisions under paragraph 2 of this Article .

5. Where additional information for a particular substance is submitted to the Agency

by a new registrant, the Agency shall notify the existing registrants that this

information is available on the database for the purposes of Article 20.

Article 19

Manufacturing and import of substances

1. A registrant may start or continue the manufacture or import of a substance, if there

is no indication to the contrary from the Agency in accordance with Article 18(2)

within the three weeks after the registration date, without prejudice to Article 25(8).

In the case of registrations of phase-in substances, a registrant may continue the

manufacture or import of the substance, if there is no indication to the contrary from

the Agency in accordance with Article 18(2) within the three weeks after the

registration date or if submitted within the 2-month period before the relevant

deadline of Article 21, if there is no indication to the contrary from the Agency in

accordance with Article 18(2) within the 3 months from that deadline, without

prejudice to Article 25(8).

In the case of an update of a registration according to Article 20 a registrant may

continue the manufacture or import of a substance, if there is no indication to the

contrary from the Agency in accordance with Article 18(2) within the three weeks

after the update date, without prejudice to Article 25(8).

2. If the Agency has informed the registrant that he is to submit further information in

accordance with the third subparagraph of Article 18(2), the registrant may start the

manufacture or import if there is no indication to the contrary from the Agency,

within the three weeks after receipt by the Agency of the further information

necessary to complete his registration, without prejudice to Article 25(8).

3. If one manufacturer or importer submits parts of the registration on behalf of other

manufacturers and/or importers, as provided for in Articles 10 or 17, those other

manufacturers and/or importers may manufacture the substance in the Community or

import it only after the expiry of the time-limit laid down in paragraph 1 or 2 of this

Article and provided that there is no indication to the contrary from the Agency in

respect of the registration of the one manufacturer or importer acting on behalf of

others.

4. Paragraphs 1, 2 and 3 shall apply to on-site isolated intermediates or transported

isolated intermediates.

Article 20

Further duties of registrants

1. Following registration, a registrant shall be responsible on his own initiative for

updating his registration without undue delay with relevant new information and

submitting it to the Agency in the following cases:

(a) any change in his status, such as being a manufacturer or an importer, or in his

identity, such as his name or address or that the company has merged or split;

(b) any change in the composition of the substance as given in section 2 of

Annex IV;

(c) changes in the annual or total quantities manufactured or imported by him if

these result in a change of tonnage band, including cessation of manufacture or

import;

(d) new identified uses and new uses advised against as in section 3.7 of Annex IV

for which the substance is manufactured or imported of which he can

reasonably be expected to have knowledge ;

(e) new knowledge of the risks of the substance to human health and/or the

environment of which he may reasonably be expected to have become aware

which leads to changes in the safety data sheet or the chemical safety report;

(f) any change in the classification and labelling of the substance;

(g) any update or amendment of the chemical safety report or section V of Annex

IV;

(h) the registrant identifies the need to perform a test listed in Annex VII or VIII,

in which cases a testing proposal shall be developed;

(i) any change in the access granted to information in the registration.

The Agency shall communicate this information to the competent authority of the

relevant Member State.

1bis. A registrant shall submit to the Agency an update of the registration containing the

information required by the decision made in accordance with Articles 39, 40 or 44

or take into account a decision made in accordance with articles 57 and 70, within

the deadline specified in that decision. The Agency shall notify the competent

authority of the relevant Member State that the information is available on its

database.

1ter. The Agency shall undertake a completeness check according to Article 18(2) first

and second subparagraphs of each updated registration. In cases where the update is

in accordance with article 11(2) and 20(1)(c) then the Agency shall check the

completeness of the information supplied by the registrant and Article 18(2)

paragraph 1 first sentence and third sentence, second paragraph and third paragraph

shall apply adapted as necessary.

2. In cases covered by Articles 10 or 17, each registrant shall submit separately the

information specified in paragraph 1(c).

CHAPTER 6

TRANSITIONAL PROVISIONS APPLICABLE TO

PHASE-IN SUBSTANCES AND NOTIFIED SUBSTANCES

Article 21

Specific provisions for phase-in substances

1. Article 19 shall not apply to the following substances for a period of 3 years after the

entry into force of this Regulation:

(a) phase-in substances classified as carcinogenic, mutagenic or toxic to

reproduction, categories 1 and 2, in accordance with Directive 67/548/EEC and

manufactured in the Community or imported, in quantities reaching 1 tonne or

more per year per manufacturer or per importer, at least once following the

entry into force of this Regulation;

(b) phase-in substances classified as very toxic to aquatic organisms and may

cause long-term adverse effects in the aquatic environment (R50-53) in

accordance with Directive 67/548/EEC, and manufactured in the Community

or imported in quantities reaching 100 tonnes or more per year per

manufacturer or per importer, at least once following the entry into force of this

Regulation;

(c) phase-in substances manufactured in the Community or imported, in quantities

reaching 1 000 tonnes or more per year per manufacturer or per importer, at

least once following the entry into force of this Regulation.

2. Article 19 shall not apply for a period of 6 years after entry into force of this

Regulation to phase-in substances manufactured in the Community or imported, in

quantities reaching 100 tonnes or more per year per manufacturer or per importer, at

least once following the entry into force of this Regulation.

3. Article 19 shall not apply for a period of 11 years after entry into force of this

Regulation to phase-in substances manufactured in the Community or imported, in

quantities reaching 1 tonne or more per year per manufacturer or per importer, at

least once following the entry into force of this Regulation.

Article 22

Notified substances

1. A notification in accordance with Directive 67/548/EEC shall be regarded as a

registration for the purposes of this Title and the Agency shall assign a registration

number within one year of entry into force of this Regulation.

2. If the quantity of a notified substance manufactured or imported per manufacturer or

importer reaches the next tonnage threshold under Article 11, the additional required

information corresponding to that tonnage threshold, as well as to all the lower

tonnage thresholds, shall be submitted in accordance with Articles 9 and 11, unless it

has already been submitted in accordance with those Articles.

TITLE III

DATA SHARING AND

AVOIDANCE OF UNNECESSARY TESTING

CHAPTER 1

OBJECTIVES AND GENERAL RULES

Article 23

Objectives and general rules

1. In order to avoid animal testing, testing on vertebrate animals for the purposes of

this Regulation shall be undertaken only as a last resort. It is also necessary to take

measures limiting duplication of other tests. The information specified in Article 9(a)

(vi) and (vii) shall therefore be made available between manufacturers or importers

for the purposes of registration.

2. The sharing and joint submission of information in accordance with this Regulation

shall concern technical data and in particular information related to the intrinsic

properties of substances. Registrants shall refrain from exchanging information

concerning their market behaviour, in particular as regards production capacities,

production or sales volumes, import volumes or market shares.

3. Any study summaries or robust study summaries of studies submitted in the

framework of a registration under this Regulation at least 10 years previously shall

be made freely available by the Agency to any other registrants or potential

registrants.

CHAPTER 2

RULES FOR NON-PHASE-IN SUBSTANCES AND REGISTRANTS OF

PHASE-IN SUBSTANCES WHO HAVE NOT PRE-REGISTERED

Article 24

Duty to inquire prior to registration

1. [deleted]

2. [deleted]

3. The potential registrant of a non-phase-in substance, or the potential registrant of a

phase-in substance who has not pre-registered in accordance with Article 26 shall

inquire from the Agency whether a registration has already been submitted for the

same substance. He shall submit all the following information to the Agency with the

inquiry:

(a) his identity as specified in section 1 of Annex IV, with the exception of the use

sites;

(b) the identity of the substance, as specified in section 2 of Annex IV;

(c) which information requirements would require new studies involving

vertebrate animals to be carried out by him;

(d) which information requirements would require other new studies to be carried

out by him.

4. If the same substance has previously not been registered, the Agency shall inform the

potential registrant accordingly.

5. If the same substance has previously been registered less than 10 years earlier, the

Agency shall inform the potential registrant without delay of the names and

addresses of the previous registrant(s) and of the relevant summaries or robust study

summaries, as the case may be, already submitted by them

These studies shall not be repeated.

The Agency shall simultaneously inform the previous registrants of the name and

address of the potential registrant. The available studies must be shared with the

potential registrant in accordance with Article 25.

6. If several potential registrants have made an inquiry in respect of the same substance,

the Agency shall inform all potential registrants without delay of the name and

address of the other potential registrants

Article 25

Sharing of existing data in the case of registered substances

1. In the case of substances previously registered less than 10 years earlier as referred to

in Article 24(5), the potential registrant shall ask the previous registrant(s) for the

information he requires with respect to Article 9(a)(vi) and (vii) in order to register.

2. The potential and the previous registrant(s) as referred to in paragraph 1 shall make

every effort to reach an agreement to make available the information required by the

potential registrant(s) with respect of Article 9(a)(vi) and (vii). Such an agreement

may be replaced by submission of the matter to an arbitration board and acceptance

of the arbitration order.

2bis. The previous registrant and potential registrant(s) shall make every effort to ensure

that the costs of sharing the information are determined in a fair, transparent and non

discriminatory way. This may be facilitated by following cost sharing guidance

based on those principles and which is adopted by the Agency in accordance with

Article 73(2)(f). Registrants are only required to share in the costs of information

that they are required to submit to satisfy their registration requirements.

3. On agreement on the sharing of the information, the new registrant shall have

permission to refer to these studies in his registration dossier.

4. If there is failure to reach such an agreement, the potential registrant(s) shall inform

the Agency and the previous registrant(s) thereof at least 1 month after receipt, from

the Agency, of the name and address of the previous registrant(s).

5. Within one month from the receipt of the information referred to in paragraph 4, the

Agency shall take the decision to make available to the potential registrant the study

summaries or robust study summaries as required by him. The previous registrant(s)

shall have a claim on the potential registrant for an equal share of the cost incurred

by him, which shall be enforceable in the national courts.

6. [deleted]

7. An appeal may be brought, in accordance with Articles 87, 88 and 89, against

Agency decisions under paragraph 5 of this Article.

8. The registration waiting period in accordance with Article 19 (1) for the new

registrant shall be extended by a period of 4 months, if the previous registrant so

requests.

CHAPTER 3

RULES FOR PHASE-IN-SUBSTANCES

Article 26

Duty to pre-register for phase-in substances

1. In order to benefit from the transitional regime provided for in Article 21 each

potential registrant of a phase-in substance, including without limitation

intermediates, shall submit all the following information to the Agency:

(a) the name of the substance as specified in section 2 of Annex IV , including its

Einecs and CAS number or, if not available, any other identity codes;

(b) his name and address and the name of the contact person and, where

appropriate, the name and address of the person representing him in accordance

with Article 27(3) as specified in section 1 of Annex IV;

(c) the envisaged deadline for the registration/tonnage band;

(cbis) the name(s) of substance(s) as specified in section 2 of Annex IV, including

their Einecs and CAS number or, if not available, any other identity codes, for

which the available information is relevant for the application of Annex IX

paragraphs 1.3 and 1.5.

(d) [deleted]

(e) [deleted]

2. The information referred to in paragraph 1 shall be submitted at the latest:

(a) six months after entry into force of this Regulation for:

(i) phase-in substances manufactured or imported in quantities of 1000

tonnes or more per year; or

(ii) phase-in substances manufactured or imported in quantities of 1 tonne

or more per year and classified as carcinogenic, mutagenic or toxic to

reproduction categories 1 or 2 in accordance with Directive 67/548/EEC; or

(iii) phase-in substances manufactured or imported in quantities of 100

tonnes or more per year and classified as very toxic to aquatic organisms and

may cause long-term adverse effects in the aquatic environment (R50-53) in

accordance with Directive 67/548/EEC.

(b) Four years and six months after entry into force of this Regulation for phase-in

substances manufactured or imported in quantities of 1 tonne or more per year.

3. Registrants who do not submit the information required under paragraph 1 shall not

be able to rely on Article 21.

4. [deleted]

5. The Agency shall, within one month of the expiry of the deadlines referred to in

paragraph 2(a) and (b) above, publish on its website a list of the substances referred to

in paragraphs 1(a) and (cbis). That list shall comprise only the names of the

substances, including their EINECs and CAS number if available.

Article 26a

Submission of information to the Agency following publication of lists

1. Any manufacturer or importer of a substance in quantities of 1 tonne or more per year

which appears on a list published by the Agency in accordance with Article 26(5), and

who holds any of the information specified in Article 9(a) (vi) and (vii), shall within

18 months before the deadline in Article 21(1) for substances which have been pre-

registered in accordance with Article 26(2)(a) above, submit to the Agency the

information referred to in Article 26(1).

2. Within one month of the deadline specified in paragraph 1 the Agency shall inform

each manufacturer, importer and downstream user who has submitted the requested

information, of the names and addresses of previous registrant(s) and of the other

potential registrants.

3. Manufacturers or importers of phase-in substances in quantities of less than 1 tonne

per year, as well as downstream users and third parties, that appear on the list

published by the Agency in accordance with Article 26(5), may submit the

information referred to in paragraph 26(1) or any other relevant information to the

Agency with the intention of being part of the substance information exchange forum

as referred to in Article 27.

Article 27

Substance Information Exchange Fora

1. All manufacturers and importers who have submitted information to the Agency in

accordance with Article 26 and Article 26a for the same phase-in substance shall be

participants in a substance information exchange forum (SIEF).

2. The aim of each SIEF shall be to:

(i) facilitate for the purposes of registration, the exchange of the information

specified in Article 9(a) (vi) and (vii) between manufacturers and importers, thereby

avoiding the duplication of studies; and

(ii) agree classification and labelling where there is a difference in the

classification and labelling of the substance.

SIEF participants shall provide other participants with existing studies, react to requests

by other participants for information, collectively identify needs for further studies and

arrange for them to be carried out. Each SIEF shall be operational until 11 years after

entry into force of this Regulation.

3. A manufacturer or importer may appoint a natural or legal person established in the

Community to participate in a SIEF as his representative at pre-registration. The

identity of a manufacturer or importer who has appointed a representative shall not

be disclosed by the Agency to other registrants.

Article28

Sharing of data involving tests

1. Before testing is carried out in order to meet the information requirements for the

purposes of registration, a SIEF participant shall inquire whether a relevant study is

availableby communicating within his SIEF. If a relevant study is available within

the SIEF, a participant of that SIEF shall request that study within two months of the

deadline set in Article 26(2).

Within two weeks of the request, the owner of the study shall provide proof of its

cost to the participant(s) requesting it. The participant(s) and the owner shall make

every effort to ensure that the costs of sharing the information are determined in a

fair, transparent and non discriminatory way. This may be facilitated by following

any cost sharing guidance which is based on those principles and is adopted by the

Agency in accordance with Article 73(2)(f). If they cannot reach such an agreement,

the cost shall be shared equally. The owner shall give permission to refer to the full

study report for the purpose of registration within two weeks of receipt of payment.

Registrants are only required to share in the costs of information that they are

required to submit to satisfy their registration requirements.

2. If a relevant study involving tests is not available within the SIEF, only one study

shall be conducted per information requirement within each SIEF by one of its

members acting on behalf of the others. They shall take all reasonable steps to reach

an agreement within a deadline set by the Agency as to who is to carry out the test on

behalf of the other participants and to submit a summary or robust study summary to

the Agency. If no agreement is reached, the Agency shall specify which registrant or

downstream user shall perform the test. All participants of the SIEF who require a

study shall contribute to the costs for the elaboration of the study with a share

corresponding to the number of participating potential registrants. Those participants

that do not carry out the study themselves shall have the right to receive the full study

report within two weeks following payment to the participant that carried out the

study.

3. If the owner of a study as referred to in paragraph 1 which involves testing on

vertebrate animals refuses to provide either proof of the cost of that study or the

study itself to another participant(s), he shall not be able to proceed with registration

until he provides the information to the other participants(s). The other participant(s)

shall proceed with registration without fulfilling the relevant information

requirement, explaining the reason for this in the registration dossier. The study shall

not be repeated unless within 12 months of the date of registration of the other

participant(s), the owner of this information has not provided it to them and the

Agency decides that the test should be repeated by them. However, if a registration

containing the summary or robust study summary, as the case may be, has already

been submitted by another registrant, the Agency shall take the decision to make

available to the other participant(s) that summary or robust study summary, as the

case may be. The other registrant shall have a claim on the participants for an equal

share of the cost, which shall be enforceable in the national courts.

3bis. If the owner of a study as referred to in paragraph 1 which does not involve testing

on vertebrate animals refuses to provide either proof of the cost of that study or the

study itself to another participant(s), the other members of the SIEF shall proceed

with registration as if no relevant study was available in the SIEF.

4. An appeal may be brought, in accordance with Articles 87, 88 and 89, against

Agency decisions under paragraph 3 of this Article.

5. The owner of the study who has refused to provide either the costs or the study itself,

as referred to in paragraph 3, shall be penalised in accordance with Article 123.

TITLE IV

INFORMATION

IN THE SUPPLY CHAIN

Article 29

Requirements for Safety Data Sheets

1. Where a substance or preparation meets the criteria for classification as dangerous in

accordance with Directives 67/548/EEC or 1999/45/EC, or it is persistent,

bioaccumulative and toxic or very persistent and very bioaccumulative in accordance

with the criteria set out in Annex XII, the supplier shall provide the recipient with a

safety data sheet compiled in accordance with Annex Ia.

2. Any actor in the supply chain who is required, under Articles 13 or 34, to carry out a

chemical safety assessment for a substance shall ensure that the information in the

safety data sheet is consistent with the information in this assessment. If the safety

data sheet is developed for a preparation, the actor in the supply chain may prepare a

chemical safety assessment for the preparation. In that case, it is sufficient if the

information in the safety data sheet is consistent with the chemical safety report for

the preparation instead of with the chemical safety report for each substance in the

preparation.

3. Where a preparation does not meet the criteria for classification as dangerous in

accordance with Articles 5, 6 and 7 of Directive 1999/45/EC, but contains in an

individual concentration of = 1 % by weight for non-gaseous preparations and

= 0.2 % by volume for gaseous preparations at least one substance posing health or

environmental hazards, or one substance for which there are Community workplace

exposure limits, the supplier shall provide, a downstream user or distributor at his

request, with a safety data sheet compiled in accordance with Annex Ia.

4. The safety data sheet need not be supplied where dangerous substances or preparations

offered or sold to the general public are provided with sufficient information to enable

users to take the necessary measures as regards the protection of health, safety and the

environment, unless requested by a downstream user or distributor.

5. The safety data sheet shall be supplied in an official language of the Member State(s)

where the substance or preparation is placed on the market, unless the Member State

provides otherwise.

6. The safety data sheet shall be dated and shall contain the following headings:

1. identification of the substance/preparation and of the company/undertaking;

2. hazards identification;

3. composition/information on ingredients;

4. first-aid measures;

5. fire-fighting measures;

6. accidental release measures;

7. handling and storage;

8. exposure controls/personal protection;

9. physical and chemical properties;

10. stability and reactivity;

11. toxicological information;

12. ecological information;

13. disposal considerations;

14. transport information;

15. regulatory information;

16. other information.

7. Any registrant or downstream user who is required to prepare a chemical safety report

according to Article 13 or 34 shall place the relevant exposure scenarios (including

use and exposure categories where appropriate) in an annex to the safety data sheet

covering identified uses.

Any downstream user shall include relevant exposure scenarios, and use other relevant

information, from the safety data sheet supplied to him when compiling his own safety

data sheet for identified uses.

Any distributor shall pass on relevant exposure scenarios, and use other relevant

information, from the safety data sheet supplied to him when compiling his own safety

data sheet for uses for which he has passed on information according to Article 34(2).

8. A safety data sheet shall be provided free of charge on paper or electronically at the

latest at the time of the first delivery of a substance or preparation following the entry

into force of this Regulation.

9. Suppliers shall update it without delay on the following occasions:

(a) as soon as new information which may affect the risk management measures

become available;

(b) [deleted]

(c) once an authorisation has been granted or refused;

(d) once a restriction has been imposed.

The new, dated version of the information, identified as 'Revision: (date)', shall be

provided free of charge on paper or electronically to all former recipients to whom

they have supplied the substance or preparation within the preceding 12 months. Any

updates following registration shall include the registration number.

10. Paragraphs 1 to 9 shall not apply to substances in the following preparations and to

the following preparations in the finished state, intended for the final user:

(a) medicinal products for human or veterinary use, within the scope of Regulation

(EEC) No 2309/93, Directive 2001/82/EC of the European Parliament and of

the Council and as defined in Directive 2001/83/EC of the European

Parliament and of the Council;

(b) cosmetic products as defined in the scope of Directive 76/768/EEC;

(c) mixtures of substances which, in the form of waste, are covered by Directives

75/442/EEC and 78/319/EEC;

(d) foodstuffs;

(e) animal feedingstuffs;

(f) containing radioactive substances as defined by Directive 80/836/Euratom;

(g) medical devices which are invasive or used in direct physical contact with the

human body in so far as Community measures lay down provisions for the

classification and labelling of dangerous substances and preparations which

ensure the same level of information provision and protection as Directive

1999/45.

Article 30

Duty to communicate information down the supply chain for substances on their own or in

preparations for which a safety data sheet is not required

1. Any supplier of a substance on its own or in a preparation who does not have to

supply a safety data sheet in accordance with Article 29 shall provide the recipient

with the following information:

(a) the registration number(s) referred to in Article 18 (2bis), if available, for any

substances for which information is communicated under Article 30(1)(b), (c)

or (d);

(b) whether the substance is subject to authorisation and details of any

authorisation granted or denied under Title VII in this supply chain;

(c) details of any restriction imposed under Title VIII;

(d) any other available and relevant information about the substance that is

necessary to enable appropriate risk management measures to be identified and

applied.

2. This information shall be communicated free of charge on paper or electronically at

the latest at the time of the first delivery of a substance on its own or in a preparation

following the entry into force of this Regulation.

3. Suppliers shall update this information without delay on the following occasions:

(a) as soon as new information which may affect the risk management measures

becomes available;

(b) [deleted]

(c) once an authorisation has been granted or refused;

(d) once a restriction has been imposed.

In addition, the updated information shall be provided free of charge on paper or

electronically to all former recipients to whom they have supplied the substance or

preparation within the preceding 12 months. Any updates following registration

shall include the registration number.

4. Any producer or importer of articles who is required to provide a notification to the

Agency under Article 6(2) shall copy this notification to the next downstream user in

the supply chain. This obligation to copy the notification to the next downstream user

of the article shall extend down the supply chain.

Article 31

Duty to communicate information on

substances and preparations up the supply chain

Any actor in the supply chain of a substance or a preparation shall communicate the following

information to the next actor or distributor up the supply chain:

(a) new information on hazardous properties, regardless of the uses concerned;

(b) any other information that might call into question the appropriateness of the risk

management measures identified in a safety data sheet supplied to him, which shall

be communicated only for identified uses.

Distributors shall pass on that information to the next actor or distributor up the supply chain.

Article 32

Access to information for workers

Workers and their representatives shall be granted access by their employer to the information

provided in accordance with Article 29 and 30 in relation to substances or preparations that

they use or may be exposed to in the course of their work.

Article 33

Obligation to keep information

1. Each manufacturer, importer, downstream user and distributor shall assemble and

keep available all the information he requires to carry out his duties under this

Regulation for a period of at least 10 years after he last manufactured, imported,

supplied or used the substance or preparation. That manufacturer, importer,

downstream user or distributor shall submit this information or make it available

without delay upon request to any competent authority, of the Member State in which

he is established or to the Agency, without prejudice to Titles II and VI.

2. In the event of a registrant ceasing activity, or transferring part or all of his operations

to a third party, the party responsible for liquidating the registrant’s undertaking or

assuming responsibility for marketing the substance or preparation concerned, shall be

bound by the obligation in Article 33(1) in place of the registrant.

TITLE V

DOWNSTREAM USERS

Article 34

Downstream user chemical safety assessments and

duty to identify, apply and recommend risk reduction measures

1. A downstream user or distributor may provide information to assist in the

preparation of a registration.

2. Any downstream user shall have the right to make a use known in writing (on paper

or electronically) to the manufacturer, importer, downstream user or distributor who

supplies him with a substance with the aim of making this an identified use. In

making a use known, he shall provide sufficient information to allow the

manufacturer, importer or downstream user who has supplied the substance, to

prepare an exposure scenario for his use in the manufacturer, importer or

downstream user’s chemical safety assessment.

Distributors shall pass on such information to the next actor or distributor up the

supply chain.

Downstream users in receipt of such information may prepare an exposure scenario

for the identified use(s), or pass the information to the next actor up the supply chain.

3. For registered substances, the manufacturer, importer or downstream user shall

comply with the obligation laid down in Article 13 before he next supplies the

substance to the downstream user making the request referred to in paragraph 2,

provided that the request was made at least one month before the supply, or within 1

month after the request, whichever is the later.

For phase-in substances, the manufacturer, importer or downstream user shall

comply with this request and with the obligations laid down in Article 13 before the

relevant deadline in Article 21, provided that the downstream user makes his request

at least 12 months before the deadline in question.

Where the manufacturer, importer or downstream user, having assessed the use in

accordance with Article 13, is unable to include it as an identified use, he shall

provide the Agency and the downstream user with the reason(s) for that decision in

writing without delay.

4. A downstream user of a substance on its own or in a preparation shall prepare a

chemical safety report in accordance with Annex XI for any use outside the

conditions described in an exposure scenario communicated to him in a safety data

sheet.

A downstream user need not prepare such a chemical safety report in any of the

following cases:

(a) a safety data sheet is not required to be communicated with the substance or

preparation in accordance with Article 29;

(b) a chemical safety report is not required to be completed by his supplier in

accordance with Article 13;

(c) the downstream user implements or recommends an exposure scenario which

includes as a minimum the conditions described in the exposure scenario

communicated to him in the safety data sheet;

(d) the substance is present in a preparation in a concentration lower than any of

the concentrations set out in Article 13(2);

(e) the downstream user is using the substance for the purposes of product and

process oriented research and development, provided that potential risks to

human health and the environment are adequately controlled.

5. Any downstream user shall identify, apply and where suitable, recommend,

appropriate measures to adequately control risks identified in any of the following:

(a) the safety data sheet(s) supplied to him;

(b) his own chemical safety assessment;

(c) any information on risk management measures supplied to him in accordance

with Article 30.

6. Downstream users shall keep their chemical safety report up to date and available.

7. A chemical safety report prepared in accordance with paragraph 4 need not include

consideration of the risks to human health from the end uses set out in Article 13(5).

Article 35

Obligation for downstream users to report information

1. Before commencing or continuing with a particular use of a substance that has been

registered by an actor up the supply chain in accordance with Articles 5 or 16, the

downstream user shall report to the Agency the information specified in paragraph 2

of this Article, in the following cases:

(a) the downstream user has to prepare a chemical safety report in accordance

with Article 34(4); or

(b) the downstream user is relying on the exemption in Article 34(4)(e).

2. The information reported by the downstream user shall include the following:

(a) his identity and contact details as specified in section 1.1 of Annex IV;

(b) the registration number(s) referred to in Article 18(1), if available;

(c) the identity of the substance(s) as specified in section 2 of Annex IV;

(d) the identity of the manufacturer(s) or the importer(s) or other supplier as

specified in section 1.1 of Annex IV;

(e) a brief general description of the use(s), as specified in section 3.5 of Annex

IV, and of the conditions of use(s);

(f) a proposal for additional testing on vertebrate animals, where this is considered

necessary by the downstream user to complete his chemical safety assessment.

3. The downstream user shall update this information without delay in the event of a

change in the information reported in accordance with paragraph 1.

4. A downstream user shall report to the Agency if his classification of a substance is

different to that of his supplier.

5. Reporting in accordance with paragraphs 1 to 4 shall not be required in respect of a

substance, on its own or in a preparation, used by the downstream user in quantities

of less than 1 tonne per year for that particular use.

Article 36

Application of downstream user obligations

1. Downstream users shall be required to comply with the requirements of Article 34 at

the latest 12 months after receiving a registration number communicated to them by

their suppliers in a safety data sheet.

2. Downstream users shall be required to comply with the requirements of Article 35 at

the latest 6 months after receiving a registration number communicated to them by

their suppliers in a safety data sheet.

TITLE VI

EVALUATION

CHAPTER 1

SCOPE

[deleted]

Article 37

Scope

[deleted]

CHAPTER 2

DOSSIER EVALUATION

Article 38

Competent authority

[deleted]

Article 39

Examination of testing proposals

1. The Agency shall examine any testing proposal set out in a registration or a

downstream user report for provision of the information specified in Annexes VII

and VIII for a substance. Priority should be given to registrations of substances

which have or may have PBT, vPvB, sensitizing and/or CMR properties, or classified

substances above 100tpa with uses resulting in widespread and diffuse exposure.

2. On the basis of the examination under paragraph 1, the Agency shall draft one of the

following decisions and that decision shall be taken in accordance with the procedure

laid down in Articles 48 and 49:

(a) a decision requiring the registrant(s) or downstream user(s) concerned to carry

out the proposed test and setting a deadline for submission of the summary of

the test result, or the robust study summary if required by Annex I;

(b) a decision in accordance with point (a), but modifying the conditions under

which the test is to be carried out;

(c) a decision rejecting the testing proposal;

(d) a decision in accordance with point (a) or (b), if several registrants of the same

substance have submitted proposals for the same test, giving them the

opportunity to reach an agreement on who will perform the test on behalf of all

of them and to inform the Agency accordingly within 90 days. If the Agency is

not informed of such agreement within such 90 days, it shall designate one of

the registrants to perform the test on behalf of all of them.

3. The registrant or downstream user shall submit the information required to the

Agency by the deadline set.

Article 40

Compliance check of registrations

1. The Agency may examine any registration in order to verify any of the following:

(a) that the information in the technical dossier(s) submitted pursuant to Article 9

complies with the requirements of Articles 9, 11 and 12 and with Annexes IV

to VIII;

(b) that the adaptations of the standard information requirements and the related

justifications submitted in the technical dossier(s) comply with the rules

governing such adaptations set out in Annexes V to VIII and with the general

rules set out in Annex IX;

(c) that any required chemical safety assessment and chemical safety report

comply with the requirements of Annex I and that the proposed risk

management measures are adequate.

2. On the basis of an examination made pursuant to paragraph 1, the Agency may

prepare a draft decision requiring the registrant(s) to submit any information needed

to bring the registration(s) into compliance with the relevant information

requirements and specifying adequate time limits for the submission of further

information. Such a decision shall be taken in accordance with the procedure laid

down in Articles 48 and 49.

3. The registrant shall submit the information required to the Agency by the deadline

set.

4. To ensure that registration dossiers comply with the Regulation, the Agency shall

select a percentage of those dossiers no lower than 5% of the total received by the

Agency in any year for compliance checking. The Agency shall give priority, but not

exclusively, to dossiers meeting at least one of the following criteria:

(a) the dossier contains a disagreement with the lead registrant view on the selection

of the information specified in Article 9(a)(iv), (vi) and/or (vii) as per Article

10(1); or

(b) the dossier is for a substance manufactured or imported in quantities of 1 tonne or

more per year and does not meet the requirements of Annex V; or

(c) the quality of data submitted under Article 9(iii), (iv), (vi) and (vii) has not been

assessed by a competent third-party.

Article 41

Check of information submitted and

follow-up to dossier evaluation

1. The Agency shall examine any information submitted in consequence of a decision

taken under Articles 39 or 40, and draft any appropriate decisions in accordance with

Article 39 or 40, if necessary.

2. Once the dossier evaluation is completed, the Agency shall notify the Commission

and the competent authorities of the Member States of the information obtained, how

to use the information and any conclusions made. The competent authorities shall use

the information obtained from this evaluation for the purposes of Articles 43a bis (6),

56(3) and 66(2).The Agency shall use the information obtained from this evaluation

for the purposes of Article 43a.

Article 42

Procedure and time periods for examination of testing proposals

1. [deleted]

2. In the case of non phase-in substances, the Agency shall prepare a draft decision in

accordance with Article 39(2) within 180 days of receiving a registration or

downstream user report containing a testing proposal.

3. In the case of phase-in substances, the Agency shall prepare the draft decisions in

accordance with Article 39(2):

(a) within 5 years of the entry into force of this Regulation for all registrations

received within the deadline referred to in Article 21 (1) containing proposals

for testing in order to fulfil the information requirements in Annexes VII and

VIII;

(b) within 9 years of the entry into force of this Regulation for all registrations

received within the deadline referred to in Article 21 (2) containing proposals

for testing in order to fulfil the information requirements in Annex VII only;

(c) within 15 years of the entry into force of this Regulation for any registrations

containing testing proposals received within the deadline referred to in

Article 21 (3).

4. The list of registration dossiers being evaluated under Article 39 shall be made

available to Member States.

Article 43

Procedure and time periods for compliance check

1. [deleted]

2. The Agency shall prepare a draft decision in accordance with Article 40 within

12 months of the start of the compliance check of the dossier.

3. [deleted]

4. The list of dossiers being evaluated under Article 40 shall be made available to

Member States.

CHAPTER 3

SUBSTANCE EVALUATION

Article 43a

Criteria for substance evaluation

1. In order to provide a harmonised approach, the Agency shall in co-operation with the

Member States develop criteria for prioritising substances with a view to further

evaluation. Prioritisation shall be on a risk-based approach. The criteria shall consider;

(a) hazard data, for instance structural similarity of the substance with known

substances of concern or with substances which are persistent and liable to bio-

accumulate, suggesting that the substance or one or more of its transformation

products has properties of concern or is persistent and liable to bio-accumulate;

(b) exposure information;

(c) tonnage, including aggregated tonnage from the registrations submitted by

several registrants.

2. The Agency shall use the criteria in paragraph 1 for the purpose of compiling a draft

Community rolling action plan which shall cover a period of three years and shall

specify substances to be evaluated each year. Substances shall be included if there are

grounds for considering (either on the basis of a dossier evaluation carried out by the

Agency or on the basis of any other appropriate source, including information in the

registration dossier) that a given substance constitutes a risk to health or the

environment. The Agency shall submit the first draft rolling action plan to the Member

States within 4 years of entry into force of this Regulation. The Agency shall submit

draft annual updates to the rolling action plan to the Member States by 28 February

each year.

The Agency shall adopt the final Community rolling action plan on the basis of an

opinion from the Member States Committee and shall publish the plan on its website,

identifying the Member State who will carry out the evaluation of the substances listed

therein as determined according to Article 43a bis.

Article 43a bis

Competent authority

1. The Agency shall rely on the competent authorities of Member States for evaluation

of substances on the Community rolling action plan. In carrying out an evaluation of

a substance, the competent authorities may appoint another body to act on their

behalf.

2. A Member State may choose a substance(s) from the Community rolling action plan,

with the aim of becoming a competent authority for the purposes of Articles 44 - 46.

Any substance from the rolling action plan that is not initially chosen by any Member

State shall be attributed to a Member State by the Agency, taking into account the

Member State in which the manufacturer(s) or importer(s) is located, the respective

proportions of total Community gross domestic product, the number of substances

already being evaluated by a Member State and the expertise available.

3. In cases where two or more Member States have expressed an interest in evaluating

the same substance, the competent authority for the purposes of Articles 44, 45 and 46

shall be determined in accordance with the following procedure.

The Agency shall refer the matter to the Member State Committee provided for in

Article 72(1)(e), hereinafter “the Member State Committee”, in order to agree which

authority shall be the competent authority, taking into account the principle that the

allocation of substances among Member States shall reflect their proportion of the

total Community gross domestic product.

If, within 60 days of the referral, the Member State Committee reaches unanimous

agreement, the Member States concerned shall adopt substances for evaluation

accordingly.

If the Member State Committee fails to reach a unanimous agreement, the Agency

shall submit the conflicting opinions to the Commission, which shall decide which

authority shall be the competent authority, in accordance with the procedure referred

to in Article 130(3), and the Member States shall adopt substances for evaluation

accordingly.

4. The competent authority identified in accordance with paragraphs 2 and 3 shall

evaluate the allocated substances in accordance with this Chapter.

5. A Member State may notify the Agency at any time of a substance not on the

Community rolling action plan, whenever it is in possession of information which

suggests that the substance is a priority for evaluation. The Agency shall add that

substance to the Community rolling action plan and the Member State shall be

responsible for evaluating that substance.

Article 44

Requests for further information and check of information submitted

1. If the competent authority considers that further information is required, including, if

appropriate, information not required in Annexes V to VIII, it shall prepare a draft

decision, stating reasons, requiring the registrant(s) to submit the further information

and setting a deadline for its submission. The decision shall be taken in accordance

with the procedure laid down in Articles 48 and 49a.

2. The registrant shall submit the information required to the Agency by the deadline

set.

3. A draft decision shall be prepared within 12 months of the publication of the

Community rolling action plan on the Agency’s website for substances to be

evaluated that year. The competent authority shall examine any information

submitted in consequence of such a decision, and shall draft any appropriate

decisions in accordance with this Article, if necessary, within 12 months of the

information being submitted.

4. The competent authority shall finish its evaluation activities within 12 months of the

start of the evaluation of the substance or within 12 months of the information being

submitted under paragraph3, and notify the Agency accordinglyIf this deadline is

exceeded, the evaluation shall be deemed to be finished.

Article 45

Coherence with other activities

1. An evaluation of a substance shall be based on all relevant information submitted on

that particular substance and on any previous evaluation under this Title. Where

information on intrinsic properties of a substance has been generated by reference to

structurally related substance(s), the evaluation may also cover these related

substances. Any draft decision requiring further information under Article 44 may be

justified only by a change of circumstances or acquired knowledge.

2. In order to ensure a harmonised approach to requests for further information, the

Agency shall monitor draft decisions under Article 44 and shall develop criteria and

priorities. Where appropriate, implementing measures shall be adopted in accordance

with the procedure referred to in Article 130(3).

Article 46

Follow-up to substance evaluation

Once the substance evaluation has been completed, the competent authority shall use the

information obtained from this evaluation for the purposes of Articles 56(3), 66(2) and

112(1). The competent authority shall inform the Agency of its conclusions as to whether or

how to use the information obtained. The Agency shall in turn inform the Commission, the

registrant and the competent authorities of the other Member States.

CHAPTER 4

EVALUATION OF INTERMEDIATES

Article 47

Further information on on-site isolated intermediates

For on-site isolated intermediates that are used in strictly controlled conditions, neither

dossier nor substance evaluation shall apply. However, where the competent authority of the

Member State in whose territory the site is located considers that a risk to human health or the

environment equivalent to the level of concern arising from the use of substances meeting the

criteria in Article 54, arises from the use of an on-site isolated intermediate and that risk is not

controlled, it may:

(a) require the registrant to submit further information directly related to the risk

identified. This request shall be accompanied by a written justification;

(b) examine any information submitted and, if necessary, take any appropriate risk

reduction measures to address the risks identified in relation to the site in question.

The procedure provided for in the first paragraph may be undertaken only by the competent

authority referred to therein. The competent authority shall inform the Agency of the results

of such an evaluation, which shall then inform the competent authorities of other Member

States and make the results available to them.

CHAPTER 5

COMMON PROVISIONS

Article 48

Registrants’ and downstream users’ rights

1. The Agencyshall notify any draft decision under Articles 39, 40 or 44 to the

registrant(s) or downstream user(s) concerned, informing them of their right to

comment within 30 days of receipt. If they wish to comment, the concerned

registrant(s) or downstream user(s) shall provide their comments to the Agency. The

Agency in turn shall inform the competent authority of the submission of the

comments without delay. The competent authority (for decisions taken under Article

44) and the Agency (for decisions taken under Articles 39 and 40) shall take any

comments received into account and may amend the draft decision accordingly.

2. If a registrant has ceased the manufacture or import of the substance, he shall inform

the Agency of this fact with the consequence that the registered volume in his

registration shall be put to zero and no further information may be requested with

respect to that substance, unless the registrant notifies the restart of the manufacture

or import of the substance. The Agency shall inform the competent authority of the

Member State in which the manufacturer or importer is located.

3. The registrant may cease the manufacture or import of the substance upon receipt of

the draft decision. In such cases, he shall inform the Agency of this fact with the

consequence that his registration shall no longer be valid, and no further information

may be requested with respect to that substance, unless he submits a new registration.

The Agency shall inform the competent authority of the Member State in which the

manufacturer or importer is located.

4. Notwithstanding paragraphs 2 and 3, further information may be required in

accordance with Article 44 in either or both of the following cases:

(a) where the competent authority prepares a dossier in accordance with

Annex XIV concluding that there is a potential long-term risk to man or the

environment justifying the need for further information;

(b) where the exposure to the substance manufactured or imported by the

registrant(s) concerned contributes significantly to that risk.

The procedure in Articles 66 to 70 shall apply mutatis mutandis.

Article 49

Adoption of decisions under dossier evaluation

1. The Agency shall notify its draft decision in accordance with Articles 39 or 40,

together with the comments of the registrant or downstream user, to the competent

authorities of the Member States.

2. Within 30 days of circulation, the Member States may propose amendments to the

draft decision to the Agency.

3. If the Agency does not receive any proposals, it shall take the decision in the version

notified under paragraph 1.

4. If the Agency receives a proposal for amendment, it may modify the draft decision.

The Agency shall refer a draft decision, together with any amendments proposed, to

the Member State Committee within 15 days of the end of the 30-day period referred

to in paragraph 2.

5. The Agency shall forthwith communicate any proposal for amendment to any

registrants or downstream users concerned and allow them to comment within

30 days. The Member State Committee shall take any comments received into

account.

6. If, within 60 days of the referral, the Member State Committee reaches a unanimous

agreement on the draft decision, the Agency shall take the decision accordingly.

If the Member State Committee fails to reach a unanimous agreement, it shall adopt

an opinion in accordance with Article 81(8) within 60 days of the referral. The

Agency shall transmit that opinion to the Commission.

7. Within 60 days of receipt of the opinion, the Commission shall prepare a draft

decision to be taken in accordance with the procedure referred to in Article 130(2).

8. An appeal may be brought, in accordance with Articles 87, 88 and 89, against

Agency decisions under paragraphs 3 and 6.

Article 49a

Adoption of decisions under substance evaluation

1. The competent authority shall circulate its draft decision in accordance with Article

44, together with any comments by the registrant or downstream user, to the Agency

and to the competent authorities of the other Member States.

2. The provisions of Article 49(2) to (8) shall apply mutatis mutandi

Article 50

Cost sharing for tests without an agreement between registrants and/or downstream users

1. Where registrants or downstream users are required to perform a test as a result of a

decision taken under this Title, those registrants or downstream users shall make

every effort to reach an agreement as to who is to carry it out on behalf of the other

registrants or downstream users and to inform the Agency accordingly within 90

days. If the Agency is not informed of such agreement within such 90 days, it shall

designate one of the registrants to perform the test on behalf of all of them.

2. If a registrant or downstream user performs a test on behalf of others, they shall all

share the cost of that study equally.

3. In the case referred to in paragraph 1, the registrant or downstream user who

performs the test shall provide each of the others concerned with a copy of the test.

4. The person performing and submitting the study shall have a claim against the others

accordingly. The others shall have a claim for a copy of the study. Any person

concerned shall be able to make a claim in order to prohibit another person from

manufacturing, importing or placing the substance on the market if that other person

either fails to pay his share of the cost or to provide security for that amount or fails

to hand over a copy of the study performed. All claims shall be enforceable in the

national courts. Any person may choose to submit their claims for remuneration to an

arbitration board and accept the arbitration order.

Article 51

Publication of information on evaluation

By 28 February of each year, the Agency shall publish on its website a report on the progress

made over the previous calendar year towards discharging the obligations incumbent upon it

in relation to evaluation. This report shall include, in particular, recommendations to potential

registrants in order to improve the quality of future registrations.

TITLE VII

AUTHORISATION

CHAPTER 1

AUTHORISATION REQUIREMENT

Article 52

Aim of authorisation

The aim of this Title is to ensure the good functioning of the internal market while assuring

that the risks from substances of very high concern are properly controlled and that these

substances are eventually replaced by suitable alternative substances or technologies where

these are economically and technically viable.

Article 53

General provisions

1. A manufacturer, importer or downstream user shall not place on the market a

substance for a use or use it himself if that substance is included in Annex XIII,

unless:

(a) the use(s) of that substance on its own, in a preparation or the incorporation of

a substance into an article for which the substance is placed on the market or

for which he uses the substance himself has been authorised in accordance with

Articles 57 to 61; or

(b) the use(s) of that substance on its own, in a preparation or the incorporation of

the substance into an article for which the substance is placed on the market or

for which he uses the substance himself has been exempted from the

authorisation requirement in Annex XIII itself in accordance with

Article 55(2); or

(c) the date referred to in Article 55(1)(c)(i) has not been reached; or

(d) the date referred to in Article 55(1)(c)(i) has been reached and he made an

application 18 months before that date but a decision on the application for

authorisation has not yet been taken. or (e) in cases where the substance is

placed on the market, authorisation for that use has been granted to his

immediate downstream user.

2. A downstream user may use a substance meeting the criteria set out in paragraph 1

provided that the use is in accordance with the conditions of an authorisation granted

to an actor up his supply chain for that use.

3. [deleted]

23 OJ L 350, 28.12.1998, p. 58.

4. Paragraphs 1 and 2 shall not apply to the use of substances in scientific research and

development or in product and process orientated research and development in

quantities not exceeding 1 tonne per year.

5. Paragraphs 1 and 2 shall not apply to the following uses of substances:

(a) uses in plant protection products within the scope of Directive 91/414/EEC;

(b) uses in biocidal products within the scope of Directive 98/8/EC;

(c) [deleted]

(d) [deleted]

(e) [deleted]

(f) [deleted]

(g) [deleted]

(h) use as motor fuels covered by Directive 98/70/EC of the European Parliament

and of the Council23;

(i) uses as fuel in mobile or fixed combustion plants of mineral oil products and

use as fuels in closed systems.

6. In the case of substances that are subject to authorisation only because they meet the

criteria in Article 54(a), (b) and (c) or because they are identified in accordance with

Article 54(f) only because of hazards to human health, paragraphs 1 and 2 of this

Article shall not apply to the following uses:

(a) uses in cosmetic products within the scope of Directive 76/768/EEC;

(b) uses in food contact materials within the scope of Directive 89/109/EEC.

7. Paragraphs 1 and 2 shall not apply to the use of substances when they are present in

preparations:

(a) for substances referred to in Article 54(d), (e) and (f), below a concentration

limit of 0.1% weight by weight (w/w);

(b) for all other substances, below the lowest of the concentration limits specified

in Directive 1999/45/EC or in Annex I to Directive 67/548/EEC which result in

the classification of the preparation as dangerous.

Article 54

Substances to be included in Annex XIII

The following substances may be included in Annex XIII in accordance with the procedure

laid down in Article 55:

(a) substances meeting the criteria for classification as carcinogenic category 1 or 2 in

accordance with Directive 67/548/EEC;

(b) substances meeting the criteria for classification as mutagenic category 1 or 2 in

accordance with Directive 67/548/EEC;

(c) substances meeting the criteria for classification as toxic for reproduction category 1

or 2 in accordance with Directive 67/548/EEC;

(d) substances which are persistent, bioaccumulative and toxic in accordance with the

criteria set out in Annex XII;

(e) substances which are very persistent and very bioaccumulative in accordance with

the criteria set out in Annex XII;

(f) substances, such as those having endocrine disrupting properties or those having

persistent, bioaccumulative and toxic properties or very persistent and very

bioaccumulative properties, which do not fulfil the criteria of points (d) and (e) and

for which there is scientific evidence of probable serious effects to humans or the

environment which give rise to an equivalent level of concern to those of other

substances listed in points (a) to (e) on a case-by-case basis in accordance with the

procedure set out in Article 56.

Article 55

Inclusion of substances in Annex XIII

1. Whenever a decision is taken to include in Annex XIII substances referred to in

Article 54, such a decision shall be taken in accordance with the procedure referred

to in Article 130(3). It shall specify for each substance:

(a) the identity of the substance as specified in section 2 of Annex IV;

(b) the intrinsic property (properties) of the substance referred to in Article 54;

(c) transitional arrangements:

(i) the date(s) from which the placing on the market and the use of the

substance shall be prohibited unless an authorisation is granted,

hereinafter “the sunset date”, which should take account, where

appropriate, the production cycle specified for that use;

(ii) a date or dates at least 18 months before the sunset date(s) by which

applications must be received if the applicant wishes to continue to use

the substance or place it on the market for certain uses after the sunset

date(s); these continued uses shall be allowed after the sunset date until a

decision on the application for authorisation is taken;

(d) review periods for certain uses, if appropriate;

(e) uses or categories of uses exempted from the authorisation requirement, if any,

and conditions for such exemptions, if any.

2. Uses or categories of uses may be exempted from the authorisation requirement

provided that, on the basis of the existing specific Community legislation imposing

minimum requirements relating to the protection of health or the environment for the

use of the substance, the risk is adequately controlled. In the establishment of such

exemptions, account shall be taken, in particular, of the proportionality of risk to

human health and the environment related to the nature of the substance, such as

where the hazard is modified by the physical form.

3. Prior to a decision to include substances in Annex XIII, the Agency shall recommend

priority substances to be included specifying for each substance the items set out in

paragraph 1. Priority shall normally be given to substances with:

(a) PBT or vPvB properties;

(b) wide dispersive use; or

(c) high volumes.

The number of substances included in Annex XIII and the dates specified under

paragraph 1 shall also take account of the Agency’s capacity to handle applications

in the time provided for. The Agency shall make its first recommendation of priority

substances to be included in Annex XIII no later than 2 years after entry into force of

the Regulation. The Agency shall make further recommendations at least every

second year with a view to including further substances in Annex XIII.

4. Before the Agency sends its recommendation to the Commission it shall make it

publicly available on its website, clearly indicating the date of publication, taking

into account confidentiality in accordance with Article 116. The Agency shall invite

all interested parties to submit comments within three months of the date of

publication, in particular on uses which should be exempt from the authorisation

requirement.

The Agency shall update its recommendation, taking into account the comments

received.

5. After inclusion of a substance in Annex XIII, this substance shall not be subjected to

new restrictions under the procedure outlined in Title VIII covering the risks to

human health or the environment from the use of the substance arising from the

intrinsic properties specified in Annex XIII.

6. Substances for which all uses have been prohibited under Title VIII or by other

Community legislation shall not be included in Annex XIII or shall be removed from

it.

7. Substances which as a result of new information no longer meet the criteria of Article

54 (a) – (f), shall be removed from Annex XIII in accordance with the procedure

referred to in Article 130(3).

Article 56

Identification of substances referred to in Article 54

1. The procedure set out in paragraphs 2 to 9 of this Article shall apply for the purpose

of identifying substances meeting the criteria referred to in Article 54 and

establishing a list of these substances not yet included in Annex XIII.

1bis. The Agency shall include on the list referred to in paragraph 1 a substance meeting the

criteria in Article 54 (a) – (c) without delay once it has been included in Annex I of

Directive 67/548/EEC. The Agency may include that substance in its

recommendations under Article 55(3).

2. The Commission may ask the Agency to prepare a dossier in accordance with

relevant sections of Annex XIV for substances which in its opinion meet the criteria

set out in Article 54 (d), (e) and (f). The Agency shall make this dossier available to

the Member States.

3. Any Member State may prepare a dossier in accordance with Annex XIV for

substances which in its opinion meet the criteria set out in Article 54(d), (e) and (f)

and forward it to the Agency. The Agency shall make this dossier available within 30

days of receipt to the other Member States.

4. Within 60 days of circulation, the other Member States or the Agency may comment

on the identification of the substance in the dossier to the Agency.

5. If the Agency does not receive any comments, it shall include this substance on the

list referred to in paragraph 1. The Agency may include this substance in its

recommendations under Article 55(3).

6. Upon receipt of comments from another Member State or on its own initiative, the

Agency shall refer the dossier to the Member State Committee within 15 days of the

end of the 60-day period referred to in paragraph 4.

7. If, within 30 days of the referral, the Member State Committee reaches a unanimous

agreement on the identification, the Agency shall include the substance in the list

referred to in paragraph 1. The Agency may include that substance in its

recommendations under Article 55(3). If the Member State Committee fails to reach

a unanimous agreement, it shall adopt an opinion within 30 days of the referral. The

Agency shall transmit that opinion to the Commission within 30 days of the referral,

including information on any minority view within the Committee.

8. The Commission shall prepare a draft proposal on the identification of the substance

within 3 months of receipt of the opinion of the Member State Committee. A final

decision on the identification of the substance shall be taken in accordance with the

procedure referred to in Article 130(3).

9. The Agency shall publish and update the list referred to in paragraph 1 on its website

without delay after a decision on inclusion of a substance has been taken.

24 OJ L 189, 20.7.1990, p. 17.

25 OJ L 169, 12.7.1993, p. 1.

26 OJ L 331, 7.12.1998, p. 1.

CHAPTER 2

THE GRANTING OF AUTHORISATIONS

Article 57

The granting of authorisations

1. The Commission shall be responsible for taking decisions on applications for

authorisations in accordance with this Title.

2. An authorisation shall be granted if the risk to human health or the environment from

the use of a substance arising from the intrinsic properties specified in Annex XIII is

adequately controlled in accordance with Annex I, section 6, and as documented in the

applicant’s chemical safety report. The Commission shall take into account all

discharges, emissions and losses known at the time of decision.

The Commission shall not consider the risks to human health arising from the use of a

substance in a medical device regulated by Council Directive 90/385/EEC24, Council

Directive 93/42/EEC25 or Directive 98/79/EC of the European Parliament and of the

Council26.

3. If an authorisation cannot be granted under paragraph 2, an authorisation may be

granted if it is shown that socio-economic benefits outweigh the risk to human health

or the environment arising from the use of the substance and if there are no suitable

alternative substances or technologies. This decision shall be taken after consideration

of all of the following elements:

(a) the risk posed by the uses of the substance;

(b) the socio-economic benefits arising from its use and the socio-economic

implications of a refusal to authorise as demonstrated by the applicant or other

interested parties;

(c) the analysis of the alternatives submitted by the applicant under Article 59(5)

and any third party contributions submitted under Article 61(2);

(d) available information on the health or environmental risks of any alternative

substances or technologies.

4. A use shall not be authorised if this would constitute a relaxation of a restriction set

out in Annex XVI.

5. An authorisation shall be granted only if the application is made in conformity with

the requirements of Article 59.

5bis. Where an application for authorisation includes the information specified in Article

59(5)(b), this information will be considered in determining the duration of the time-

limited review period in Article 57(6).

6. Authorisations shall be subject to a time-limited review (whose duration will be

determined on a case-by-case basis) without prejudice to any decision on a future

period of authorisation and shall normally be subject to conditions, including

monitoring.

7. The authorisation shall specify:

(a) the person(s) to whom the authorisation is granted;

(b) the identity of the substance(s);

(c) the use(s) for which the authorisation is granted;

(d) any conditions under which the authorisation is granted;

(e) the time-limited review period;

(f) any monitoring arrangement.

8. Notwithstanding any conditions of an authorisation, the holder shall ensure that the

level of exposure is reduced to as low as is technically possible.

Article 58

Review of authorisations

1. Authorisations granted in accordance with Article 57 shall be regarded as valid until

the Commission decides on a new application, provided that the holder of the

authorisation submits a new application at least 18 months before the expiry of the

time-limit. Rather than re-submitting all elements of the original application for the

current authorisation, the applicant may submit only the number of the current

authorisation, subject to the second, third and fourth subparagraphs.

A holder of an authorisation granted in accordance with Article 57 shall submit an

update of any substitution plan included in his application. If the holder cannot

demonstrate that the risk is adequately controlled, he shall submit an update of the

socio-economic analysis, analysis of alternatives and substitution plan contained in

the original application.

If he can now demonstrate that the risk is adequately controlled, he shall submit an

update of the chemical safety report.

If any other elements of the original application have changed, he shall also submit

updates of these element(s).

2. Authorisations may be reviewed at any time if:

(i) the circumstances of the original authorisation have changed so as to affect the

risk to human health or the environment, or the socio-economic impact; or,

(ii) third parties supply new information on possible substitutes to the Agency.

In such cases, the Commission shall set a reasonable deadline by which the holder(s)

of the authorisation may submit further information necessary for the review and

indicate by when it will take a decision in accordance with Article 61.

3. In its review decision the Commission may, taking into account proportionality,

amend the authorisation or withdraw the authorisation from the time of the decision,

if under the changed circumstances it would not have been granted.

In cases where there is a serious and immediate risk for human health or the

environment, the Commission may suspend the authorisation pending the review,

taking into account proportionality.

4. If an environmental quality standard referred to in Directive 96/61/EC is not met, the

authorisations granted for the use of the substance concerned may be reviewed.

5. If the environmental objectives as referred to in Article 4(1) of Directive 2000/60/EC

are not met, the authorisations granted for the use of the substance concerned in the

relevant river basin may be reviewed.

6. If a use of a substance is subsequently prohibited in Annex XVII, the Commission

shall withdraw the authorisation for that use.

If a use of a substance is subsequently made subject to conditions in Annex XVII, the

Commission shall amend the authorisation accordingly.

Article 59

Applications for authorisations

1. An application for an authorisation shall be made to the Agency.

2. Applications for authorisation may be made by the manufacturer(s), importer(s)

and/or downstream user(s) of the substance. Applications may be made by one or

several persons.

3. Applications may be made for one or several substances, and for one or several uses.

Applications may be made for the applicant’s own use(s) and/or for uses for which

he intends to place the substance on the market.

4. An application for authorisation shall include the following information:

(a) the identity of the substance(s), as referred to in section 2 of Annex IV;

(b) the name and contact details of the person or persons making the application;

(c) a request for authorisation, specifying for which use(s) the authorisation is

sought and covering the use of the substance in preparations and/or the

incorporation of the substance in articles, where this is relevant;

(d) unless already submitted as part of the registration, a chemical safety report in

accordance with Annex I covering the risks to human health and/or the

environment from the use of the substance(s) arising from the intrinsic

properties specified in Annex XIII.

5. The application may include:

(a) a socio-economic analysis conducted in accordance with Annex XV;

(b) an analysis of the alternatives considering their risks and the technical and

economic feasibility of substitution, where appropriate accompanied by a

substitution plan, including research and development and a timetable for

proposed actions by the applicant.

(c) a justification for not considering risks to human health and the environment

arising either from:

(i) emissions of a substance from an installation for which a permit was granted

in accordance with Council Directive 96/61/EC; or

(ii) discharges of a substance from a point source governed by the requirement

for prior regulation referred to in Article 11(3) and legislation adopted under

Article 16 of Directive 2000/60/EC of the European Parliament and the

Council.

6. The application shall not include the risks to human health arising from the use of a

substance in a medical device regulated by Directives 90/385/EEC, 93/42/EEC or

98/79/EC.

7. An application for an authorisation shall be accompanied by the fee in accordance

with Title VIIIa.

Article 60

Subsequent applications for authorisation

1. If an application has been made for a use of a substance, a subsequent applicant may

refer, by means of a letter of access granted by the previous applicant, to the parts of

the previous application submitted in accordance with Article 59(4)(d) and (5).

2. If an authorisation has been granted for a use of a substance, a subsequent applicant

may refer, by means of a letter of access granted by the holder of the authorisation, to

the parts of the holder’s application submitted in accordance with

Article 59(4)(d) and (5).

Article 61

Procedure for authorisation decisions

1. The Agency shall acknowledge the date of receipt of the application. The Agency’s

Committees for Risk Assessment and Socio-economic Analysis shall give their draft

opinions within ten months of the date of receipt of the application.

2. The Agency shall make available on its web-site broad information on uses, taking

confidentiality into account in accordance with Article 116, for which applications

have been received, with a deadline by which information on alternative substances

or technologies may be submitted by interested third parties.

3. In preparing its opinion, each Committee referred to in paragraph 1 shall first check

that the application includes all the information specified in Article 59 that is relevant

to its remit. If necessary, the Committees shall, in consultation with each other, make

a joint request to the applicant for additional information to bring the application into

conformity with the requirements of Article 59. The Socio-economic Analysis

Committee may, if it deems it necessary, require the applicant to submit, within a

specified time period, additional information on possible alternative substances or

technologies. Each Committee shall also take into account any information submitted

by third parties.

4. The draft opinions shall include the following elements:

(a) Risk Assessment Committee: an assessment of the risk to health and/or the

environment arising from the use(s) of the substance as described in the

application;

(b) Socio-economic Analysis Committee: an assessment of the socio-economic

factors and the availability, suitability and technical feasibility of alternatives

associated with the use(s) of the substance as described in the application,

when an application is made in accordance with Article 59(5).

5. The Agency shall send these draft opinions to the applicant by the end of the

deadline set out in paragraph 1. Within 1 month of receipt of the draft opinion, the

applicant may provide written notice that he wishes to comment. The draft opinion

shall be deemed to have been received 7 days after the Agency has sent it.

If the applicant does not wish to comment, the Agency shall send these opinions to

the Commission, the Member States and the applicant, within 15 days of the end of

the period within which the applicant may comment or within 15 days of receipt of

notice from the applicant that he does not intend to comment.

If the applicant wishes to comment, he shall send his written argumentation to the

Agency within 2 months of the receipt of the draft opinion. The Committees shall

consider the comments and adopt their final opinions within 2 months of receipt of

the written argumentation, taking this argumentation into account where appropriate.

Within a further 15 days the Agency shall send the opinions, with the written

argumentation attached, to the Commission, the Member States and the applicant.

6. The Agency shall make the non-confidential parts of its opinions and the non-

confidential parts of any attachments thereto publicly available on its website, in

accordance with Article 116.

7. In cases covered by Article 60(1), the Agency shall treat the applications together,

provided the deadlines for the first application can be met.

8. The Commission shall prepare a draft authorisation decision within 3 months of

receipt of the opinions from the Agency. A final decision granting or refusing the

authorisation shall be taken in accordance with the procedure referred to in

Article 130(2).

9. Summaries of the Commission decisions, including the authorisation number, shall

be published in the Official Journal of the European Union and shall be made

publicly available in a database established and kept up to date by the Agency.

10. In cases covered by Article 60(2), the deadline set out in paragraph 1 of this Article

shall be shortened to 5 months.

CHAPTER 3

AUTHORISATIONS IN THE SUPPLY CHAIN

Article 62

Obligation of holders of authorisations

Holders of an authorisation, as well as downstream users including the substances in a

preparation, shall include the authorisation number on the label before they place the

substance or a preparation containing the substance on the market for an authorised use

without prejudice to Directive 67/548/EEC and Directive 99/45/EC. This should be done

without delay once the authorisation number has been made publicly available in accordance

with Article 61(9).

Article 63

Downstream Users

1. Downstream users using a substance in accordance with Article 53(2) shall notify the

Agency within 3 months of the first supply of the substance. .

2. The Agency shall establish and keep up to date a register of downstream users who

have made a notification in accordance with paragraph 1. The Agency shall grant

access to this register to the competent authorities of the Member States.

TITLE VIII

RESTRICTIONS ON THE MANUFACTURING,

MARKETING AND USE OF CERTAIN DANGEROUS

SUBSTANCES AND PREPARATIONS

CHAPTER 1

GENERAL ISSUES

Article 64

General provisions

1. A substance on its own, in a preparation or in an article, for which Annex XVI

contains a restriction shall not be manufactured, placed on the market or used unless

it complies with the conditions of that restriction. This shall not apply to the

manufacture, placing on the market or use of a substance in scientific research and

development. Annex XVI shall specify when and how the restriction does not apply

to product and process orientated research and development, as well as the maximum

quantity exempted.

2. A substance on its own, in a preparation or in an article, for which Annex XVII

contains a restriction shall not be manufactured, placed on the market or used unless

it complies with the conditions of that restriction. This shall not apply to the

manufacture, placing on the market or use of a substance for laboratory scale

research or the use of the substance as a reference standard.

3. [deleted]

4. Article 64 shall not apply to substances used in cosmetic products, as defined by

Directive 76/768/EC, with regard to restrictions addressing the risks to human health

within the scope of that Directive.

5. Up until 6 years after entry into force of the Regulation, a Member State may

maintain any existing and more stringent restrictions on the manufacture, placing on

the market or use of a substance, provided that those restrictions have been notified

and approved by the Commission under Article 95 EC.

CHAPTER 2

THE RESTRICTIONS PROCESS

Article 65

Introducing new and amending current restrictions

1. When there is an unacceptable risk to human health or the environment, arising from

the manufacture, use or placing on the market of substances, which needs to be

addressed on a Community-wide basis, Annex XVI shall be amended in accordance

with the procedure referred to in Article 130(3) by adopting new restrictions, or

amending current restrictions in Annex XVI, for the manufacture, use or placing on

the market of substances on their own, in preparations or in articles, pursuant to the

procedure set out in Articles 66 to 70. Any such decision shall take into account the

socio-economic impact of the restriction, including the availability of alternatives.

The first subparagraph shall not apply to the use of a substance as an on-site isolated

intermediate, except in cases covered by paragraph 3.

2. For a substance on its own, in a preparation or in an article which meet the criteria

for classification as carcinogenic, mutagenic or toxic to reproduction, categories 1

and 2, and could be used as such or in a preparation by consumers and for which

restrictions to consumer use are proposed by the Commission, Annex XVI shall be

amended in accordance with the procedure referred to in Article 130(3). Articles 66

to 70 shall not apply.

3. Notwithstanding Article 55(5), at the latest upon the inclusion of a substance in the

Stockholm Convention or the UNECE Protocol on Persistent Organic Pollutants, the

Commission shall present a draft for the inclusion of that substance in Annex XVII.

The draft measures shall as minimum implement the obligations arising from these

international commitments for the Community. Annex XVII shall be amended in

accordance with the procedure referred to in Article 130 (3). Articles 66 to 70 shall

not apply.

4. [deleted]

Article 66

Preparation of a proposal

1. If the Commission considers that the manufacture, placing on the market or use of a

substance on its own, in a preparation or in an article poses a risk to human health or

the environment that is not adequately controlled and needs to be addressed, it shall

ask the Agency to prepare a dossier which conforms to the requirements of

Annex XIV. If this dossier demonstrates that the risk is unacceptable, and therefore

action on a Community-wide basis is necessary, beyond any measures already in

place, the Agency shall suggest restrictions, in order to initiate the restrictions

process.

2. If a Member State considers that the manufacture, placing on the market or use of a

substance on its own, in a preparation or in an article poses a risk to human health or

the environment that is not adequately controlled and needs to be addressed it shall

notify the Agency that it proposes to prepare a dossier which conforms to the

requirements of the relevant sections of Annex XIV. If the substance is not on the

list maintained by the Agency referred to in paragraph 66(2) bis below, the Member

State shall prepare a dossier which conforms to the requirements of Annex XIV. If

this dossier demonstrates that action on a Community-wide basis is necessary,

beyond any measures already in place, the Member State shall submit it to the

Agency in the format outlined in Annex XIV, in order to initiate the restrictions

process.

The Agency or Member States shall refer to any dossier, chemical safety report or

risk assessment submitted to the Agency or Member State under this Regulation. The

Agency or Member States shall also refer to any relevant risk assessment submitted

for the purposes of other Community Regulations or Directives. To this end other

bodies, such as agencies, established under Community law and carrying out a

similar task shall provide information to the Agency or Member State concerned on

request.

The Committee for Risk Assessment and the Committee for Socio-economic

Analysis shall check whether the dossier submitted conforms to the requirements of

Annex XIV. Within 30 days of receipt, the respective Committee shall inform the

Agency or the Member State suggesting restrictions, as to whether the dossier

conforms. If the dossier does not conform, the reasons shall be given to the Agency

or the Member State in writing within 45 days of receipt. The Agency or the Member

State shall bring the dossier into conformity within 60 days of the date of receipt of

the reasons from the Agency, otherwise the procedure under this Chapter shall be

terminated. The Agency shall publish without delay the intention of the Commission

or of a Member State to instigate a restriction procedure for a substance and shall

inform those who submitted a registration for that substance.

2(bis). The Agency shall maintain a list of substances for which a dossier conforming to the

requirements of Annex XIV is planned or underway by either the Agency or a

Member State for the purposes of a proposed restriction. If a substance is on the list,

no other such dossier shall be prepared. If it is proposed by either a Member State or

the Agency that an existing restriction listed in Annex XVI should be re-examined a

decision on whether to do so shall be taken in accordance with the procedure referred

to in Article 130(2) based on evidence presented by the Member State or the Agency.

3. Without prejudice to Article 116, the Agency shall make publicly available on its

website all dossiers conforming with Annex XIV including the restrictions suggested

pursuant to paragraphs 1 and 2 without delay, clearly indicating the date of

publication. The Agency shall invite all interested parties to submit individually or

jointly within 6 months of the date of publication:

(a) comments on dossiers and the suggested restrictions;

(b) a socio-economic analysis, or information which can contribute to one, of the

suggested restrictions, examining the advantages and drawbacks of the

proposed restrictions. It shall conform to the requirements in Annex XV.

Article 67

Agency opinion: Committee for risk assessment

Within 9 months of the date of publication referred to in Article 66(3), the Committee for

Risk Assessment shall formulate an opinion as to whether the suggested restrictions are

appropriate in reducing the risk to human health and/or the environment, based on its

consideration of the relevant parts of the dossier. This opinion shall take account of the

Member State dossier or of the dossier prepared by the Agency at the request of the

Commission, and the views of interested parties referred to in point (a) of Article 66(3).

Article 68

Agency opinion: Committee for socio-economic analysis

1. Within 12 months of the date of publication referred to in Article 66(3), the

Committee for Socio-economic Analysis shall formulate an opinion on the suggested

restrictions, based on its consideration of the relevant parts of the dossier and the

socio-economic impact. It shall prepare a draft opinion on the suggested restrictions

and on the related socio-economic impact, taking account of the analyses or

information according to point (b) of Article 66(3), if there are any. The Agency

shall publish the draft opinion on its website without delay. The Agency shall invite

interested parties to give their comments on the draft opinion no later than 60 days

from the publication of that draft opinion.

2. The Committee for Socio-economic Analysis shall without delay adopt its opinion,

taking into account where appropriate further comments received by the deadline set.

This opinion shall take account of the comments and socio-economic analyses of

interested parties submitted under point (b) of Article 66 (3) and under Article 68 (1).

3. Where the opinion of the Committee for Risk Assessment diverges significantly from

the restrictions suggested by a Member State or the Commission, the Agency may

postpone the deadline for the opinion of the Committee for Socio-economic Analysis

by a maximum of 90 days.

Article 69

Submission of an opinion to the Commission

1. The Agency shall submit to the Commission without delay the opinions of the

Committees for Risk Assessment and Socio-economic Analysis on restrictions

suggested for substances on their own, in preparations or in articles. If one or both of

the Committees do not formulate an opinion by the deadline set in Articles 67(1) and

68(1) the Agency shall inform the Commission accordingly, stating the reasons.

2. Without prejudice to article 116 the Agency shall publish the opinions of the two

Committees on its website without delay.

3. The Agency shall provide the Commission and/or Member State on request with all

documents and evidence submitted to or considered by it.

Article 70

Commission decision

1. If the conditions laid down in Article 65 are fulfilled, the Commission shall prepare a

draft amendment to Annex XVI, within 3 months of receipt of the opinion of the

Committee for Socio Economic analysis or the end of the deadline established under

Article 68 if that Committee does not form an opinion, whichever is the earlier.

Where the draft amendment diverges from the original proposal or if it does not take

the opinions from the Agency into account, the Commission shall annex a detailed

explanation of the reasons for the differences.

2. A final decision shall be taken in accordance with the procedure referred to in

Article 130(3). The Commission shall send the draft amendment to the Member

States at least 45 days before voting.

TITLE VIIIA

FEES

Article 70a

Fees

1. The fees that are required according to Articles 5, 6(1), 6(4), 7(2), 10, 15, 16, 17, 59(7)

and 88(3) shall be specified in a Commission Regulation adopted in accordance with

the procedure referred to in Article 130(3).

2. The structure and amount of the fees referred to in paragraph 1 shall take account of

the work required by this Regulation to be carried out by the Agency and the

competent authority and shall be fixed at such a level as to ensure that the revenue

derived from them when combined with other sources of the Agency’s revenue

pursuant to Article 93(1) is sufficient to cover the full cost of the services delivered.

In the case of Articles 5, 6(1), 6(4), 7(2), 10, 15, 16, the structure and amount of fees

shall take account of the tonnage range of the substance being registered.

In the case of Article 59(7), a reduced fee shall be set for SMEs.

Furthermore, the amount of the fees required according to Articles 15 and 16 should

take into account proportionality.

3. In certain specified cases a proportion of the fees will be transferred to the relevant

Member State Competent Authority in accordance with this Regulation.

TITLE IX

AGENCY

Article 71

Establishment

A European Chemicals Agency (the Agency) is established for the purposes of managing the

technical, scientific and administrative aspects of REACH and to ensure consistency of

decisions at Community level.

Article 72

Composition

1. The Agency shall comprise:

(a) a Management Board, which shall exercise the responsibilities set out in

Article 74;

(b) an Executive Director, who shall exercise the responsibilities set out in

Article 79;

(c) a Committee for Risk Assessment, which shall be responsible for preparing the

opinion of the Agency on evaluations, applications for authorisation, proposals

for restrictions, and any other questions that arise from the operation of this

Regulation relating to risks to human health or the environment;

(d) a Committee for Socio-economic Analysis, which shall be responsible for

preparing the opinion of the Agency on applications for authorisation,

proposals for restrictions, and any other questions that arise from the operation

of this Regulation relating to the socio-economic impact of possible legislative

action on substances;

(e) a Member State Committee, which shall be responsible for resolving potential

divergences of opinions on draft decisions proposed by the Agency or the

Member States under Title VI, preparing the opinion of the Agency on

proposals for classification and labelling under Title X and proposals for

identification of substances of very high concern to be subjected to the

authorisation procedure under Title VII and, where invited by the Executive

Director, for preparing an opinion in cases where the Committee for Risk

Assessment or the Committee for Socio-economic Analysis cannot reach

unanimous agreement;

(f) a Forum for Exchange of Information on Enforcement, hereinafter “the

Forum”, which shall co-ordinate a network of Member States authorities

responsible for enforcement of this Regulation;

(g) a Secretariat, which shall work under the leadership of the Executive Director

and provide technical, scientific and administrative support for the Committees

and the Forum and ensure appropriate co-ordination between them. It shall also

undertake the work required of the Agency under the procedures for pre-

registration, registration and evaluation as well as preparation of guidance,

database maintenance and information provision;

(h) a Board of Appeal, which shall decide on appeals against decisions taken by

the Agency.

2. The Committees referred to in points (c), (d) and (e) of paragraph 1, hereinafter “the

Committees”, and the Forum may each establish working groups. For this purpose

they shall adopt, in accordance with their rules of procedure, precise arrangements

for delegating certain tasks to these working groups.

3. The Committees and the Forum may, if they consider it appropriate, seek advice on

important questions of a general scientific or ethical nature from appropriate sources

of expertise.

Article 73

Tasks

1. The Agency shall provide the Member States and the institutions of the Community

with the best possible scientific and technical advice on questions relating to

chemicals which fall within its remit and which are referred to it in accordance with

the provisions of the present Regulation.

2. The Secretariat shall undertake the following tasks:

(a) performing the tasks allotted to it under Title II; including facilitating the

efficient registration of imported substances, in a way consistent with the

Community’s international trading obligations towards third countries;.

(b) performing the tasks allotted to it under Title III;

(c) performing the tasks allotted to it under Title VI;

(d) establishing and maintaining database(s) with information on all registered

substances, the classification and labelling inventory and the harmonised

classification and labelling list. It shall make the information identified in

Article 116(1) in the data base(s) publicly available, free of charge, over the

Internet, and making other non-confidential information in the databases

available on request;

(e) making publicly available information as to which substances are being, and

have been evaluated within 90 days of receipt of the information at the Agency,

in accordance with Article 116(1);

(f) providing technical and scientific guidance and tools where appropriate for the

operation of this Regulation in particular to assist the development of chemical

safety reports by industry and especially by Small and Medium sized

Enterprises (SMEs);

(g) providing technical and scientific guidance on the operation of this Regulation

for Member State competent authorities and providing support to the help

desks established by Member States under Title XII;

(h) preparing explanatory information on this Regulation for other stakeholders;

(i) at the Commission’s request, providing technical and scientific support for

steps to improve co-operation between the Community, its Member States,

international organisations and third countries on scientific and technical issues

relating to the safety of substances, as well as active participation in technical

assistance and capacity building activities on sound management of chemicals

in developing countries;

(j) keep a Manual of Decisions and Opinions based on conclusions from the

Member States Committee regarding interpretation and implementation of this

Regulation;

(k) notification of decisions taken by the Agency;

(l) provision of formats for submission of information to the Agency.

3. The Committees shall undertake the following:

(a) performing the tasks allotted to them under Title VI;

(b) performing the tasks allotted to them under Title VII;

(c) performing the tasks allotted to them under Title VIII;

(d) performing the tasks allotted to them under Title X;

(e) at the Commission’s request, providing technical and scientific support for

steps to improve co-operation between the Community, its Member States,

international organisations and third countries on scientific and technical issues

relating to the safety of substances, as well as active participation in technical

assistance and capacity building activities on sound management of chemicals

in developing countries;

(f) at the Commission's request, drawing up an opinion on any other aspects

concerning the safety of substances on their own, in preparations or articles.

4. The Forum shall undertake the following tasks:

(a) spreading good practice and highlighting problems at Community level;

(b) proposing, co-ordinating and evaluating harmonised enforcement projects and

joint inspections;

(c) co-ordinating exchange of inspectors;

(d) identifying enforcement strategies, as well as best practice in enforcement;

(e) developing working methods and tools of use to local inspectors;

(f) developing an electronic information exchange procedure;

(g) liaising with industry and other stakeholders, including relevant international

organisations, as necessary;

(h) examine proposals for restrictions with a view to advising on enforceability.

Article 74

Powers of the Management Board

The Management Board shall appoint the Executive Director pursuant to Article 80 and an

accounting officer in accordance with Article 43 of Regulation (EC, Euratom) No 2343/2002.

It shall adopt:

(a) by 30 April each year, the general report of the Agency for the previous year and

forward it by 15 June at the latest to the Member States, the European Parliament, the

Council, the Commission, the European Economic and Social Committee and the

Court of Auditors;

(b) by 31 October each year the work programme of the Agency for the coming year and

forward it to the Member States, the European Parliament, the Council and the

Commission;

(c) the final budget of the Agency pursuant to Article 93 of this Regulation before the

beginning of the financial year, adjusting it, where necessary, according to the

Community contribution and any other revenue of the Agency;

(d) a multi-annual work programme, which shall be regularly revised.

It shall adopt the internal rules and procedures of the Agency. The rules shall be made public.

It shall perform its duties in relation to the Agency's budget pursuant to Articles 93, 94

and 101.

It shall exercise disciplinary authority over the Executive Director.

It shall adopt its rules of procedure.

It shall appoint the Chairman, the members and alternates of the Board of Appeal.

It shall appoint the members of the Agency committees as set out in Article 81.

It shall forward annually to the budgetary authority any information relevant to the outcome

of the evaluation procedures.

Article 75

Composition of the Management Board

1. The Management Board shall be composed of one representative from each Member

State and a maximum of six representatives nominated by the Commission, including

three individuals from interested parties without voting rights.

2. Members shall be appointed on the basis of their relevant experience and expertise in

the field of chemicals safety or the regulation of chemicals whilst ensuring there is

relevant expertise amongst the board members in the fields of general, financial and

legal matters.

3. The duration of the term of office shall be four years. The term of office may be

renewed once. However, for the first mandate, the Council and the Commission shall

each identify half of their nominees for whom this period shall be six years.

Article 76

Chairmanship of the Management Board

1. The Management Board shall elect a Chairman and a Deputy-Chairman from among

the members with voting rights. The Deputy-Chairman shall automatically take the

place of the Chairman if he is prevented from attending to his duties.

2. The terms of the office of the Chairman and the Deputy-Chairman shall be two years

and shall expire when they cease to be members of the Management Board. The term

of office shall be renewable once.

Article 77

Meetings

1. The meetings of the Management Board shall be convened by the invitation of its

Chairman or at the request of a simple majority of the Board members.

2. The Executive Director shall take part in the meetings of the Management Board,

without voting rights.

3. The Management Board may invite the Chairmen of the Committees or the

Chairman of the Forum, as referred to in Article 72(1)(c) to (f), to attend its meetings

without voting rights.

Article 78

Voting

The Management Board shall adopt rules of procedure for voting, including the conditions for

a member to vote on behalf of another member. The Management Board shall act by a two-

thirds majority of all members with the right to vote.

Article 79

Duties and powers of the Executive Director

1. The Agency shall be managed by its Executive Director, who shall perform his

duties in the interests of the Community, and independently of any specific interests.

2. The Executive Director shall be the legal representative of the Agency. He shall be

responsible for:

(a) the day-to-day administration of the Agency;

(b) managing all the Agency resources necessary for carrying out its tasks;

(c) ensuring that the time-limits laid down in Community legislation for the

adoption of opinions by the Agency are complied with;

(d) ensuring appropriate and timely co-ordination between the Committees and the

Forum;

(e) concluding and managing necessary contracts with service providers;

(f) the preparation of the statement of revenue and expenditure and the execution

of the budget of the Agency pursuant to Article 93;

(g) all staff matters;

(h) providing the secretariat for the Management Board;

(i) preparing draft opinions of the Management Board concerning the proposed

rules of procedure of the Committees and of the Forum;

(j) making arrangements, upon request from the Management Board, for the

execution of any further function(s) (within the remit of Article 73) allotted to

the Agency by delegation from the Commission.

(k) determining the terms and conditions for use of software packages.

3. Each year, the Executive Director shall submit the following to the Management

Board for approval:

(a) a draft report covering the activities of the Agency in the previous year,

including information about the number of registration dossiers received, the

number of substances evaluated, the number of applications for authorisation

received, the number of proposals for restriction received by the Agency and

opined upon, the time taken for completion of the associated procedures, and

the substances authorised, dossiers rejected, substances restricted; complaints

received and the action taken; an overview of the activities of the Forum;

(b) a draft programme of work for the coming year;

(c) the draft annual accounts;

(d) the draft forecast budget for the coming year;

(e) a draft multi-annual work programme.

Article 80

Appointment of the Executive Director

1. The Commission shall propose candidates for the post of the Executive Director

based on a list following publication of the post in the Official Journal of the

European Union and other press or internet sites as appropriate.

2. The Executive Director of the Agency shall be appointed by the Management Board

on the grounds of merit and documented administrative and management skills, as

well as his relevant experience in the fields of chemical safety or regulation. The

Management Board shall take its decision by a two-thirds majority of all members

with a right to vote.

Power to dismiss the Executive Director shall lie with the Management Board, in

accordance with the same procedure.

3. The term of the office of the Executive Director shall be 5 years. It may be prolonged

by the Management Board once for another period of up to 5 years.

Article 81

Establishment of the Committees

1. Each Member State may nominate candidates to membership of the Risk Assessment

Committee. The Executive Director shall establish a list of the nominees, which shall

be published on the Agency’s website. The Management Board shall appoint the

members of the Committee from this list, including at least one member but not more

than two from the nominees of each Member State that has nominated candidates.

Members shall be appointed for their role and experience in performing the tasks

specified in Article 73(3).

2. Each Member State may nominate candidates to membership of the Socio-economic

Analysis Committee. The Executive Director shall establish a list of the nominees,

which shall be published on the Agency’s website. The Management Board shall

appoint the members of the Committee from this list, including at least one member

but not more than two from the nominees of each Member State that has nominated

candidates. Members shall be appointed for their role and experience in performing

the tasks specified in Article 73(3) .

3. Each Member State shall appoint one member to the Member State Committee.

4. The Committees should aim to have a broad range of relevant expertise among their

members. To this end the Committees may co-opt a maximum of five additional

members chosen on the basis of their specific competence.

Members of the Committees shall be appointed for a term of three years which shall

be renewable.

The members of the Management Board may not be members of the Committees.

The members of each Committee may be accompanied by advisers on scientific,

technical or regulatory matters.

The Executive Director or his representative and representatives of the Commission

shall be entitled to attend all the meetings of the Committees and working groups

convened by the Agency or its committees as observers. Stakeholders may also be

invited to attend meetings as observers, as appropriate, at the request of the

Committee members, or the Management Board.

5. The members of each Committee appointed following nomination by a Member

State shall ensure that there is appropriate co-ordination between the tasks of the

Agency and the work of their Member State competent authority.

6. The members of the Committees shall be supported by the scientific and technical

resources available to the Member States. To this end, Member States shall provide

adequate scientific and technical resources to the members of the Committees that

they have nominated. Each Member State competent authority shall facilitate the

activities of the Committees and their working groups.

7. The Member States shall refrain from giving the members of the Risk Assessment

Committee or of the Socio-Economic Analysis Committee, or their scientific and

technical advisers and experts, any instruction which is incompatible with the

individual tasks of those persons or with the tasks, responsibilities and independence

of the Agency.

8. When preparing an opinion, each Committee shall use its best endeavours to reach a

consensus. If such a consensus cannot be reached, the opinion shall consist of the

position of the majority of members and the minority position(s), with their grounds.

9. Each Committee drafts a proposal for its own rules of procedure, to be approved by

the Management Board, within 6 months of the Committees first being appointed.

These rules shall in particular lay down the procedures for replacing members, the

procedures for delegating certain tasks to working groups, the creation of working

groups and the establishment of a procedure for the urgent adoption of opinions. The

Chairman of each Committee shall be an employee of the Agency.

Article 82

Establishment of the Forum

1. Each Member State shall appoint, for a three-year term, which shall be renewable,

one member to the Forum. Members shall be chosen for their role and experience in

enforcement of chemicals legislation and shall maintain relevant contacts with the

Member State competent authorities.

The Forum should aim to have a broad range of relevant expertise among its

members. To this end the Forum may co-opt a maximum of five additional members

chosen on the basis of their specific competence. These members shall be appointed

for a term of three years, which shall be renewable.

The members of the Forum may be accompanied by scientific and technical advisers.

The Executive Director of the Agency or his representative and representatives of the

Commission shall be entitled to attend all the meetings of the Forum and its working

groups. Stakeholders may also be invited to attend meetings as observers, as

appropriate, at the request of Forum members, or the Management Board.

2. The members of the Forum appointed by a Member State shall ensure that there is

appropriate co-ordination between the tasks of the Forum and the work of their

Member State competent authority.

3. The members of the Forum shall be supported by the scientific and technical

resources available to the competent authorities of the Member States. Each Member

State competent authority shall facilitate the activities of the Forum and its working

groups. The Member States shall refrain from giving the Forum members, or their

scientific and technical advisers and experts any instruction which is incompatible

with the individual tasks of those persons or with the tasks and responsibilities of the

Forum.

4. The Forum shall draft a proposal for its own rules of procedure, to be adopted by the

Management Board, within 6 months of the Committees first being appointed.

These rules shall in particular lay down the procedures for appointing and replacing

the Chairman, replacing members and the procedures for delegating certain tasks to

working groups.

Article 83

Rapporteurs of Committees and use of experts

1. Where, in accordance with Article 73, a Committee is required to take a decision,

provide an opinion or consider whether a Member State dossier conforms with the

requirements of Annex XIV, it shall appoint one of its members as a rapporteur. The

Committee concerned may appoint a second member to act as co-rapporteur. For

each case, rapporteurs and co-rapporteurs shall undertake to act in the interests of the

Community and shall make a declaration of commitment to fulfil their duties and a

declaration of interests in writing. A member of a Committee shall not be appointed

rapporteur for a particular case if he indicates any interest that might be prejudicial to

the independent consideration of that case. The Committee concerned may replace

the rapporteur or co-rapporteur by another one of its members at any time, if, for

example, they are unable to fulfil their duties within the prescribed time limits, or if a

potentially prejudicial interest comes to light.

2. Member States shall transmit to the Agency the names of experts with proven

experience in the tasks required by Article 73, who would be available to serve on

working groups of the Committees, together with an indication of their qualifications

and specific areas of expertise.

The Agency shall keep an up-to-date list of experts. The list shall include the experts

referred to in the first subparagraph and other experts identified directly by the

Secretariat.

3. The provision of services by Committee members or any expert serving on a working

group of the Committees or Forum, or performing any other task for the Agency

shall be governed by a written contract between the Agency and the person

concerned, or where appropriate between the Agency and the employer of the person

concerned.

The person concerned, or his employer, shall be remunerated by the Agency in

accordance with a scale of fees to be included in the financial arrangements

established by the Management Board. Where the person concerned fails to fulfil his

duties, the Executive Director has the right to terminate or suspend the contract or

withhold remuneration.

4. The provision of services for which there are several potential providers may require

a call for an expression of interest;

(i) if the scientific and technical context allows, and

(ii) if it is compatible with the duties of the Agency, in particular the need to provide

a high level of protection of human health and the environment.

The Management Board shall adopt the appropriate procedures on a proposal from

the Executive Director.

5. The Agency may use the services of experts for the discharge of other specific tasks

for which it is responsible.

Article 84

Qualification and interests of members of committees and boards

1. The membership of the Committees and of the Forum shall be made public.

Individual members may request that their names not be made public if they believe

that such publication could place them at risk. The Executive Director shall decide

whether to agree to such requests. When each appointment is published, the

professional qualifications of each member shall be specified.

2. Members of the Management Board, the Executive Director and members of the

Committees and of the Forum shall make a declaration of commitment to fulfil their

duties and a declaration of interests which could be considered to be prejudicial to

their independence. These declarations shall be made annually in writing.

3. At each of their meetings, members of the Management Board, the Executive

Director, members of the Committees and of the Forum and any experts participating

in the meeting shall declare any interests which could be considered to be prejudicial

to their independence with respect to any points on the agenda. Anyone declaring

such interests shall not participate in any voting on the relevant agenda point.

Article 85

Establishment of the Board of Appeal

1. The Board of Appeal shall consist of a Chairman and two other members.

2. The Chairman and the two members shall have alternates who shall represent them in

their absence.

3. The Chairman, the other members and the alternates shall be appointed by the

Management Board on the basis of their relevant experience and expertise in the field

of chemical safety, natural sciences or regulatory and judicial procedures from a list

of qualified candidates adopted by the Commission.

The Management Board may appoint additional members and their alternates, on

recommendation by the Executive Director, following the same procedure, if this is

necessary to ensure that the appeals can be processed at a satisfactory rate.

4. The qualifications required for the members of the Board of Appeal shall be

determined by the Commission in accordance with the procedure referred to in

Article 130(3).

5. The Chairman and the members shall have equal voting rights.

Article 86

Members of the Board of Appeal

1. The term of office of the members of the Board of Appeal, including the Chairman

and the alternates shall be 5 years. It may be prolonged once.

2. The Members of the Board of Appeal shall be independent. In making their decisions

they shall not be bound by any instructions.

3. The members of the Board of Appeal may not perform any other duties in the

Agency. The function of the Members may be a part-time function.

4. The members of the Board of Appeal may not be removed either from office or from

the list during their respective terms, unless there are serious grounds for such

removal and the Commission, after obtaining the opinion of the Management Board,

takes a decision to this effect.

5. Members of the Board of Appeal may not take part in any appeal proceedings if they

have any personal interest therein, or if they have previously been involved as

representatives of one of the parties to the proceedings, or if they participated in the

decision under appeal.

6. If a member of the Board of Appeal considers for reasons mentioned in paragraph 5

that he must not take part in any appeal proceedings, he shall inform the Board of

Appeal accordingly. Members of the Board may be objected to by any party to the

appeal proceedings on any of the grounds mentioned in paragraph 5, or if suspected

of partiality. No objection may be based on the nationality of members.

7. The Board of Appeal shall decide as to the action to be taken in the cases specified in

paragraphs 5 and 6 without the participation of the member concerned. For the

purposes of taking this decision, the member concerned shall be replaced on the

Board of Appeal by an alternate.

Article 87

Decisions subject to appeal

1. An appeal may be brought against decisions of the Agency taken pursuant to

Article 7, Article 18, Article 25(5),Article 28(2) and (3), Article 49, Article 115(2) or

Article 116.

2. An appeal lodged pursuant to paragraph 1 shall have suspensive effect.

Article 88

Persons entitled to appeal, time-limits, fees and form

1. Any natural or legal person may appeal against a decision addressed to that person,

or against a decision which, although addressed to another person, is of direct and

individual concern to the former.

2. The appeal, together with the statements of the grounds thereof, shall be filed in

writing to the Agency within 3 months of the notification of the decision to the

person concerned, or in the absence thereof, of the day on which it became known to

the latter, unless otherwise provided in this Regulation.

3. A fee may be payable by persons bringing an appeal against an Agency decision, in

accordance with Title VIIIa.

Article 89

Examination and decisions on appeal

1. If the Executive Director considers the appeal to be admissible and well founded he

may rectify the decision within one month after receipt of the appeal and the

statements of grounds thereof.

2. The President of the Board of Appeal shall examine whether the appeal is admissible

within 30 days of the appeal being filed in accordance with Article 88(2). In the

affirmative, the appeal shall be remitted to the Board of Appeal for examination of

the grounds. Parties to the appeal proceedings shall be entitled to make an oral

presentation during the procedure.

3. The Board of Appeal may exercise any power which lies within the competence of

the Agency or remit the case to the competent body of the Agency for further action.

4. The procedures for the Board of Appeal shall be determined by the Commission in

accordance with the procedure referred to in Article 130(2).

Article 90

Actions before the European Court of First Instance and the Court of Justice of the

European Communities

1. An action may be brought before the European Court of First Instance or the Court of

Justice of the European Communities, in accordance with Articles 225 or 230 of the

Treaty, contesting a decision taken by the Board of Appeal or, in cases where no

right of appeal lies before the Board, by the Agency.

2. Should the Agency fail to take a decision, proceedings for failure to act may be

brought before the European Court of First Instance or the Court Justice of the

European Communities in accordance with Articles 225 or 232 of the Treaty.

3. The Agency shall be required to take the necessary measures to comply with the

judgment of the European Court of First Instance or the Court of Justice of the

European Communities.

Article 91

Complaints to the Ombudsman

[deleted]

Article 92

Conflicts of opinion with other bodies

1. The Agency shall take care to ensure early identification of potential sources of

conflict between its opinions and those of other bodies established under Community

law, including Community Agencies,carrying out a similar task in relation to issues

of common concern.

2. Where the Agency identifies a potential source of conflict, it shall contact the body

concerned in order to ensure that any relevant scientific or technical information is

shared and to identify the scientific or technical points which are potentially

contentious.

3. Where there is a fundamental conflict over scientific or technical points and the body

concerned is a Community Agency or a scientific committee, the Agency and the

body concerned shall work together either to solve the conflict or to submit a joint

document to the Commission clarifying the scientific and/or technical points of

conflict.

Article 93

The budget of the Agency

1. The revenues of the Agency shall consist of:

(a) a subsidy from the Community, entered in the general budget of the European

Communities (Commission Section);

(b) the fees paid by undertakings;

(c) any voluntary contribution from the Member States.

2. The expenditure of the Agency shall include the staff, administrative, infrastructure

and operational expenses.

3. By 15 February of each year at the latest, the Executive Director shall draw up a

preliminary draft budget covering the operational expenditure and the programme of

work anticipated for the following financial year, and shall forward this preliminary

draft to the Management Board together with an establishment plan accompanied by

a provisional list of posts.

4. Revenue and expenditure shall be in balance.

5. Each year the Management Board, on the basis of a draft drawn up by the Executive

Director, shall produce an estimate of revenue and expenditure for the Agency for

the following financial year. This estimate, which shall include a draft establishment

plan, shall be forwarded by the Management Board to the Commission by 31 March

at the latest.

6. The estimate shall be forwarded by the Commission to the European Parliament and

the Council, hereinafter “the budgetary authority”, together with the preliminary

draft budget of the European Communities.

7. On the basis of the estimate, the Commission shall enter in the preliminary draft

budget of the European Communities the estimates it considers necessary for the

establishment plan and the amount of the subsidy to be charged to the general

budget, which it shall place before the budgetary authority in accordance with

Article 272 of the Treaty.

8. The budgetary authority shall authorise the appropriations for the subsidy to the

Agency.

The budgetary authority shall adopt the establishment plan for the Agency.

9. The budget of the Agency shall be adopted by the Management Board. It shall

become final following final adoption of the general budget of the European

Communities. Where appropriate, it shall be adjusted accordingly.

10. Any modification to the budget, including the establishment plan, shall follow the

procedure referred to above.

11. The Management Board shall, without delay, notify the budgetary authority of its

intention to implement any project which may have significant financial implications

for the funding of its budget, in particular any projects relating to property such as

the rental or purchase of buildings. It shall inform the Commission thereof.

Where a branch of the budgetary authority has notified its intention to deliver an

opinion, it shall forward its opinion to the Management Board within a period of 6

weeks from the date of notification of the project.

Article 94

Implementation of the Agency’s budget

1. The Executive Director shall perform the duties of the authorising officer and shall

implement the Agency's budget.

2. Monitoring of the commitment and payment of all the Agency's expenditure and of

the establishment and recovery of all the Agency's revenue shall be carried out by the

Accounting Officer of the Agency.

3. By 1 March at the latest following each financial year, the Agency's accounting

officer shall communicate the provisional accounts to the Commission's accounting

officer together with a report on the budgetary and financial management for that

financial year. The Commission's accounting officer shall consolidate the provisional

27 OJ L 248, 16.9.2002, p. 1.

28 OJ L 136, 31.5.1999, p. 1

29 OJ L 136, 31.5.1999, p. 15.

accounts of the institutions and decentralised bodies in accordance with Article 128

of Council Regulation (EC,Euratom) No 1605/200227.

4. By 31 March at the latest following each financial year, the Commission's

accounting officer shall forward the Agency's provisional accounts to the Court of

Auditors, together with a report on the budgetary and financial management for that

financial year. The report on the budgetary and financial management for that

financial year shall also be forwarded to the European Parliament and the Council.

5. On receipt of the Court of Auditors' observations on the Agency's provisional

accounts, pursuant to Article 129 of Regulation (EC,Euratom) No 1605/2002, the

Director shall draw up the Agency's final accounts under his own responsibility and

forward them to the Management Board for an opinion.

6. The Management Board shall deliver an opinion on the Agency's final accounts.

7. By 1 July of the following year at the latest, the Executive Director shall send the

final accounts, together with the opinion of the Management Board, to the European

Parliament, the Council, the Commission and the Court of Auditors.

8. The final accounts shall be published.

9. The Director shall send the Court of Auditors a reply to its observations by 30

September at the latest. He shall also send this reply to the Management Board.

10. The European Parliament, upon a recommendation from the Council, shall, before

30 April of year N + 2, give a discharge to the Executive Director in respect of the

implementation of the budget for year N.

Article 95

Fees

[deleted]

Article 96

Combating fraud

1. In order to combat fraud, corruption and other unlawful activities, the provisions of

Regulation (EC) No 1073/1999 of the European Parliament and of the Council28

shall apply without restrictions to the Agency.

2. The Agency shall be bound by Interinstitutional Agreement 1999/1074/Euratom29

concerning internal investigations by the European Anti-Fraud Office (OLAF) and

shall issue, without delay, the appropriate provisions applicable to all of its staff.

3. The decisions concerning funding and the implementing agreements and instruments

resulting from them shall explicitly stipulate that the Court of Auditors and OLAF

may carry out, if necessary, on-the-spot checks of the recipients of the Agency's

funding and the agents responsible for allocating it.

Article 97

Financial regulation

The financial rules applicable to the Agency shall be adopted by the Management Board after

the Commission has been consulted. They may not depart from Regulation (EC, Euratom)

No 2343/2002 unless specifically necessary for the Agency's operation and with the

Commission's prior consent.

Article 98

Legal personality of the Agency

1. The Agency shall be a body of the Community and shall have legal personality. In

each Member State it shall enjoy the most extensive legal capacity accorded to legal

persons under their laws. In particular it may acquire and dispose of movable and

immovable property and may be a party to legal proceedings.

2. The Agency shall be represented by its Executive Director.

Article 99

Liability of the Agency

1. The contractual liability of the Agency shall be governed by the law applicable to the

contract in question. The Court of Justice of the European Communities shall have

jurisdiction pursuant to any arbitration clause contained in a contract concluded by

the Agency.

2. In the case of non-contractual liability, the Agency shall, in accordance with the

general principles common to the laws of the Member States, make good any

damage caused by it or by its servants in the performance of their duties.

The Court of Justice of the European Communities shall have jurisdiction in any

dispute relating to compensation for such damages.

3. The personal financial and disciplinary liability of its servants towards the Agency

shall be governed by the relevant rules applying to the staff of the Agency.

Article 100

Privileges and immunities of the Agency

The Protocol on the Privileges and Immunities of the European Communities shall apply to

the Agency.

Article 101

Staff rules and regulations

1. The staff of the Agency shall be subject to the Regulations and Rules applicable to

officials and other servants of the European Communities. In respect of its staff, the

Agency shall exercise the powers which have been devolved to the appointing

authority.

2. The Management Board shall, in agreement with the Commission, adopt the

necessary implementing provisions.

3. The Agency’s staff shall consist of officials assigned or seconded by the Commission

or Member States on a temporary basis and of other servants recruited by the Agency

as necessary to carry out its tasks. The Agency shall recruit its personnel on the basis

of a staffing plan to be included in the multi-annual work programme referred to in

Article 74(e).

Article 101a

Languages

1. The provisions laid down in Regulation No 1 of 15 April 1958 determining the

languages to be used in the European Economic Community shall apply to the

Agency.

2. The translation services required for the functioning of the Agency shall be provided

by the Translation Centre of the bodies of the European Union.

Article 102

Duty of confidentiality

Members of the Management Board, members of the Committees and of the Forum, experts

and officials and other servants of the Agency, shall be required, even after their duties have

ceased, not to disclose information of the kind covered by the duty of professional secrecy.

Article 103

Participation of third countries

The Management Board may, in agreement with the relevant Committee or the Forum, invite

representatives of third countries to participate in the work of the Agency.

Article 104

Participation of international organisations

The Management Board may, in agreement with the relevant Committee or the Forum, invite

representatives of international organisations with interests in the field of chemicals

regulation to participate as observers in the work of the Agency.

Article 105

Contacts with stakeholder organisations

The Management Board shall, in agreement with the Commission, develop appropriate

contacts between the Agency and relevant stakeholder organisations.

Article 106

Rules on transparency

To ensure transparency, the Management Board shall, on the basis of a proposal by the

Executive Director and in agreement with the Commission, adopt rules to ensure the

availability to the public of regulatory, scientific or technical information concerning the

safety of substances on their own, in preparations or in articles which is not of a confidential

nature.

Article 107

Relations with relevant Community Bodies

1. The Agency shall co-operate with other Community bodies to ensure mutual support

in the accomplishment of their respective tasks in particular to avoid duplication of

work.

2. The Executive Director, having consulted the Committee on Risk Assessment and

the European Food Safety Authority, shall establish rules of procedure concerning

substances for which an opinion has been sought in a food safety context. These rules

of procedure shall be adopted by the Management Board, in agreement with the

Commission.

This Title shall not otherwise affect the competences vested in the European Food

Safety Authority.

3. This Title shall not affect the competences vested in the European Agency for the

Evaluation of Medicinal Products.

4. The Executive Director, having consulted the Committee on Risk Assessment, the

Committee on Socio-economic Analysis and the Advisory Committee on Safety,

Hygiene and Health Protection at Work, shall establish rules of procedure concerning

worker protection issues. These rules of procedure shall be adopted by the

Management Board, in agreement with the Commission.

This Title shall not affect the competences vested in the Advisory Committee on

Safety, Hygiene and Health Protection at Work and the European Agency for Health

and Safety at Work.

Article 108

Formats and software for submission of information to the Agency

The Agency shall specify formats and make them available free of charge, and software

packages and make them available on its website for any submissions to the Agency. Member

States, manufactures, importers, distributors or downstream users shall use these formats and

packages in their submissions to the Agency pursuant to this Regulation.

For the purposes of registration, the format of the technical dossier referred to in Article 9(a)

shall be IUCLID. The Agency shall co-ordinate the further development of this format with

the OECD to ensure maximum harmonisation.

TITLE XII

COMPETENT AUTHORITIES

Article 118

Appointment

Member States shall appoint the competent authority or competent authorities responsible for

performing the tasks allotted to competent authorities under this Regulation and for

co-operating with the European Commission and the Agency in the implementation of this

Regulation. Adequate resources shall be placed at the disposal of the competent authorities to

enable them to fulfil their tasks under this Regulation in a timely and effective manner.

Article 119

Co-operation between competent authorities

The competent authorities shall co-operate with each other in the performance of their tasks

under this Regulation and shall give the competent authorities of other Member States all the

necessary and useful support to this end.

Article 120

Communication to the public of information on risks of substances

The competent authorities of the Member States shall inform the general public about the

risks arising from substances where this is considered necessary for the protection of human

health or the environment. The Commission shall draw up guidelines in accordance with

Article 130(3) with a view to co-ordinating Member States in these activities.

Article 121

Other responsibilities of the competent authorities

Member States shall establish national helpdesks to provide advice to manufacturers,

importers, downstream users and any other interested parties on their respective

responsibilities and obligations under this Regulation, in addition to the operational guidance

documents provided by the Agency under Article 73(2)(f).

TITLE XIII

ENFORCEMENT

Article 122

Tasks of the Member States

[deleted]

Article 123

Sanctions for non-compliance

1. The Member States shall lay down the provisions on penalties applicable for

infringement of the provisions of the present Regulation and shall take all measures

necessary to ensure that they are implemented. The penalties provided for must be

effective, proportionate and dissuasive. The Member States shall notify those provisions

to the Commission no later than eighteen months after entry into force of this

Regulation and shall notify it without delay of any subsequent amendment affecting

them.

2. [deleted]

Article 124

Report

Member States shall submit a report to the Agency by 1 July every third year on the

results of the official inspections, the monitoring carried out, the penalties provided for

and the other measures taken pursuant to Articles 122 and 123 during the previous three

years. The common issues to be covered in the reports shall be agreed by the Forum.

The Agency shall make these reports available to the Commission and the Forum.

ANNEX I

GENERAL PROVISIONS FOR ASSESSING SUBSTANCES

AND PREPARING CHEMICAL SAFETY REPORTS

0. INTRODUCTION

0.1. The purpose of this Annex is to set out how manufacturers and importers are to

assess and document that the risks arising from the substance they manufacture or

import are adequately controlled during manufacture and their own use(s) and that

others further down the supply chain can adequately control the risks.

0.1a The chemical safety assessment shall be prepared by one or more competent

person(s) who have appropriate experience and received appropriate training,

including refresher training.

0.2. The chemical safety assessment shall address the manufacture of a substance and all

the identified uses. It shall consider the use of the substance on its own (including

any major impurities and additives), in a preparation and in an article. The

assessment shall consider all stages of the life-cycle of the substance resulting from

the manufacture and identified uses. The chemical safety assessment shall be based

on a comparison of the potential adverse effects of a substance with the known or

reasonably foreseeable exposure of man and/or the environment to that substance.

0.3. If the manufacturer or importer considers that the chemical safety assessment carried

out for one substance is sufficient to assess and document that the risks arising from

another substance or from a group of substances are adequately controlled then he

can use that chemical safety assessment for the other substance or group of

substances in accordance with Annex IX section 1.5. The manufacturer or importer

shall provide a justification for this.

0.4. The chemical safety assessment shall be based on the information on the substance

contained in the technical dossier and on other available and relevant information.

Manufacturers or importers submitting a proposal for testing in accordance with

Annexes V to VIII should record this under the relevant heading of the chemical

safety report. Available information from assessments carried out under other

international and national programmes shall be included. Where available and

appropriate, an assessment carried out under Community legislation (e.g. risk

assessments completed under Regulation 793/93) shall be taken into account in the

development of, and reflected in, the chemical safety report. Deviations from such

assessments shall be justified.

Thus the information to be considered includes information related to the hazards of

the substance, the exposure arising from the manufacture or import, the identified

uses of the substance, operational conditions and risk management measures applied

or recommended to downstream users to be taken into account.

In accordance with Annex IX, Section 3, in some cases, it may not be necessary to

generate missing information, because risk management measures which are

necessary to control a well-characterised risk may also be sufficient to control other

potential risks, which will not therefore need to be characterised precisely.

If the manufacturer or importer considers that further information is necessary for

producing his chemical safety report and that this information can only be obtained

by performing tests in accordance with Annex VII or VIII, he shall submit a proposal

for a testing strategy, explaining why he considers that additional information is

necessary and record this in the chemical safety report under the appropriate heading.

While waiting for results of further testing, he shall record in his chemical safety

report the interim risk management measures that he has put in place and those he

recommends to downstream users intended to manage the risks being explored.

0.5. A chemical safety assessment performed by a manufacturer or an importer for a

substance shall include the following steps in accordance with the respective sections

of this Annex:

1. Human health hazard assessment

2. Human health hazard assessment of physicochemical properties

3. Environmental hazard assessment

4. PBT and vPvB assessment

If as a result of steps 1 to 4 the manufacturer or importer concludes that the substance

or the preparation meets the criteria for classification as dangerous according to

Directive 67/548/EEC or Directive 1999/45/EC or is assessed to be a PBT or vPvB,

the chemical safety assessment shall also consider the following steps:

5. The generation of exposure scenario(s) (or the identification of relevant use

and exposure categories if appropriate)

6. Exposure assessment

7. Risk characterisation

A summary of all the relevant information used in addressing the points above, shall

be presented under the relevant heading of the chemical safety report (Section 7).

0.6. The main element of the exposure part of the chemical safety report is the description

of the exposure scenario(s) implemented for the manufacturer’s production, the

manufacturer or importer’s own use, and those recommended by the manufacturer or

importer to be implemented for the identified use(s).

The exposure scenarios contain a description of the risk management measures

which the manufacturer or importer has implemented and recommends to be

implemented by downstream users.

If the substance is placed on the market, the relevant exposure scenario(s), including

the risk management measures shall be included in an annex to the safety data sheet

in accordance with Annex Ia.

0.7. The level of detail required in describing an exposure scenario will vary substantially

from case to case, depending on the use of a substance, its hazardous properties and

the amount of information available to the manufacturer or importer. Exposure

scenarios may describe the appropriate risk management measures for several

individual processes or uses of a substance. An exposure scenario may thereby cover

a large range of processes or uses.

0.8. The process which the manufacturer or importer goes through, in carrying out their

chemical safety assessment and developing their chemical safety reports, may be

iterative. When generating an exposure scenario it will be necessary to make initial

assumptions about the operating conditions and risk managements measures. If the

initial assumptions lead to a risk characterisation indicating inadequate protection of

human health and the environment, then it shall be necessary to carry out an iterative

process with amendment of one or a number of factors until adequate control can be

demonstrated. This may require the generation of additional hazard or exposure

information or appropriate alteration of the process, operating conditions or risk

management measures. Therefore iterations may consider on the one hand

developing and revising the (initial) exposure scenario(s), which may include

developing and implementing or recommending risk management measures, and on

the other hand the need to generate further information to produce the definitive

exposure scenario. The purpose of generating further information is to establish a

more precise risk characterisation, based on a refined hazard assessment or exposure

assessment. This will allow appropriate information to be communicated down the

supply chain in the safety data sheet. The definitive exposure scenario should be

included in the chemical safety report and attached to the safety data sheet.

0.9. Where information is not necessary in accordance with Annex IX , this fact shall be

stated under the appropriate heading of the chemical safety report and a reference

shall be made to the justification in the technical dossier. This fact that no

information is required shall also be stated in the safety data sheet.

0.10. In relation to particular effects, such as ozone depletion, photochemical ozone

creation potential, strong odour and tainting, for which the procedures set out in

Sections 1 to 6 are impracticable, the risks associated with such effects shall be

assessed on a case-by-case basis and the manufacturer or importer shall include a full

description and justification of such assessments in the chemical safety report and

summarised in the safety data sheet.

0.11. Where the methodology described in this Annex is not appropriate, details of

alternative methodology used shall be explained and justified in the chemical safety

report.

0.12. Part A of the chemical safety report shall include a declaration that the risk

management measures outlined in the relevant exposure scenarios for the

manufacturer’s or importer’s own use(s) are implemented by the manufacturer or

importer and that those exposure scenarios for the identified uses are communicated

to all known distributors and users further down the supply chain in the safety data

sheet.

1. HUMAN HEALTH HAZARD ASSESSMENT

1.0. Introduction

1.0.1. The objective of the human health hazard assessment shall be:

– to determine the classification and labelling of a substance in accordance with

Directive 67/548/EEC; and

– to derive levels of exposure to the substance above which humans should not be

exposed. This level of exposure is known as the Derived No-Effect Level (DNEL).

1.0.2. The human health hazard assessment shall consider the toxicokinetic profile (ie;

absorption, metabolism, distribution and elimination) of the substance and the

following groups, (1) acute effects (acute toxicity, irritation and corrosivity),

(2) sensitisation, (3) repeated dose toxicity and (4) CMR effects (carcinogenity,

mutagenicity and toxicity for reproduction). Based on all the available information,

other effects shall be considered when necessary.

1.0.3. The hazard assessment shall comprise the following four steps:

Step 1. Evaluation of non-human information

Step 2. Evaluation of human information

Step 3. Classification and Labelling

Step 4. Derivation of Derived No-Effect Levels (DNELs)

1.0.4. The first three steps shall be undertaken for every effect for which information is

available and shall be recorded under the relevant section of the Chemical Safety

Report and where required and in accordance with Article 29, summarised in the

Safety Data Sheet under headings 2 and 11.

1.0.5. For any effect for which no relevant information is available, the relevant section

shall contain the sentence “This information is not available”.

1.0.6. Step 4 of the human health hazard assessment shall be undertaken by integrating the

results from the first three steps and shall be included under the relevant heading of

the Chemical Safety Report and summarised in the Safety Data Sheet under

heading 8.1.

1.1. Step 1: Evaluation of non-human information

1.1.1. The evaluation of non-human information shall comprise:

– the hazard identification for the effect based on all available non-human information;

– the establishment of the quantitative dose (concentration) – response (effect)

relationship.

1.1.2. When it is not possible to establish the quantitative dose (concentration) – response

(effect) relationship, then this should be justified and a semi-quantitative or

qualitative analysis shall be included. For instance, for acute effects it is usually not

possible to establish the quantitative dose (concentration) – response (effect)

relationship on the basis of the results of a test conducted in accordance with Annex

X. In such cases it suffices to determine whether and to which degree the substance

has an inherent capacity to cause the effect.

1.1.3. All non-human information used to assess a particular effect on humans and to

establish the dose (concentration) – response (effect) relationship, shall be briefly

presented, if possible in the form of a table or tables, distinguishing between in vitro,

in vivo and other information. The relevant test results (e.g., LD50, NO(A)EL or

LO(A)EL) and test conditions (e.g., test duration, route of administration) and other

relevant information shall be presented, in internationally recognised units of

measurement for that effect.

1.1.4 If one study is available then a robust study summary should be prepared for that

study. If there are several studies addressing the same effect, then, having taken into

account possible variables (eg. conduct, adequacy, relevance of test species, quality

of results etc.), normally the study or studies giving rise to the highest concern shall

be used to establish the Derived No-Effect Levels and a robust study summary shall

be prepared for that study or studies and included as part of the technical dossier. If

the study or studies giving rise to the highest concern are not used, then this shall be

fully justified and robust study summaries shall be prepared and included as part of

the technical dossier, not only for the study being used but also for all studies

demonstrating a higher concern than the study being used. It is important irrespective

of whether hazards have been identified or not that the validity of the study be

considered.

1.2. Step 2: Evaluation of human information

If no human information is available, this part shall contain the statement “No human

information is available”. However, if human information is available, it shall be presented, if

possible in the form of a table.

1.3. Step 3: Classification and Labelling

1.3.1. The appropriate classification and labelling developed in accordance with the criteria

in Directive 67/548/EEC shall be presented and justified. The assessment should

always include a statement as to whether the substance fulfils or does not fulfil the

criteria given in Directive 67/548/EEC for CMR, categories 1 and 2..

1.3.2. If the information are inadequate to decide whether a substance should be classified

for a particular end-point, the registrant shall indicate and justify the action or

decision he has taken as a result.

1.4. Step 4: Identification of Derived No-Effect Level(s) (DNEL(s))

1.4.1. Based on the outcomes of steps 1 to 2, a Derived No-Effect Level(s) shall be

established for the substance, reflecting the likely route(s), duration and frequency of

exposure. For some endpoints, especially mutagenicity and carcinogenicity, the

available information may not enable a threshold, and therefore a DNEL, to be

established. If justified by the exposure scenario(s), a single DNEL may be sufficient.

However, taking into account the available information and the exposure scenario(s)

in Section 5 of the Chemical Safety Report it may be necessary to identify different

DNELs for each relevant human population (e.g., workers, consumers and humans

liable to exposure indirectly via the environment) and possibly for certain vulnerable

sub-populations (e.g. children, pregnant women) and for different routes of exposure.

A full justification shall be given specifying, inter alia, the choice of the information

used, the route of exposure (oral, dermal, inhalation) and the duration and frequency

of exposure to the substance for which the DNEL is valid. If more than one route of

exposure is likely to occur, then a DNEL shall be established for each route of

exposure and for the exposure from all routes combined. When establishing the

DNEL, the following factors shall, inter alia, be taken into account:

(i) the uncertainty arising, among other factors, from the variability in the

experimental information and from intra- and inter-species variation;

(ii) the nature and severity of the effect;

(iii) the sensitivity of the human (sub-)population to which the quantitative and/or

qualitative information on exposure applies.

1.4.2. If it is not possible to identify a DNEL, then this shall be clearly stated and fully

justified.

2. PHYSICOCHEMICAL HAZARD ASSESSMENT

2.1. The objective of the hazard assessment for physicochemical properties shall be to

determine the classification and labelling of a substance in accordance with

Directive 67/548/EEC.

2.2. As a minimum, the potential effects to human health shall be assessed for the

following physicochemical properties:

– explosivity,

– flammability,

– oxidizing potential.

If the information are inadequate to decide whether a substance should be classified for a

particular end-point, the registrant shall indicate and justify the action or decision he has taken

as a result.

2.3. The assessment of each effect shall be presented under the relevant heading of the

Chemical Safety Report (Section 7) and where required and in accordance with

Article 29, summarised in the Safety Data Sheet under headings 2 and 9.

2.4. For every physicochemical property, the assessment shall entail an evaluation of the

inherent capacity of the substance to cause the effect resulting from the manufacture

and identified uses.

2.5. The appropriate classification and labelling developed in accordance with the criteria

in Directive 67/548/EEC shall be presented and justified.

3. ENVIRONMENTAL HAZARD ASSESSMENT

3.0. Introduction

3.0.1. The objective of the environmental hazard assessment shall be to determine the

classification and labelling of a substance in accordance with Directive 67/548/EEC

and to identify the concentration of the substance below which adverse effects in the

environmental sphere of concern are not expected to occur. This concentration is

known as the Predicted No-Effect Concentration (PNEC).

3.0.2. The environmental hazard assessment shall consider the potential effects on the

environment, comprising the (1) aquatic (including sediment), (2) terrestrial and

(3) atmospheric compartments, including the potential effects that may occur (4) via

food-chain accumulation. In addition, the potential effects on the (5) microbiological

activity of sewage treatment systems shall be considered. The assessment of the

effects on each of these five environmental spheres shall be presented under the

relevant heading of the Chemical Safety Report (Section 7) and where required and

in accordance with Article 29, summarised in the Safety Data Sheet under headings 2

and 12.

3.0.3. For any environmental sphere, for which no effect information is available, the

relevant section of the chemical safety report shall contain the sentence “This

information is not available ”. For any environmental sphere for which information

is available, but the manufacturer or importer believes that it is not necessary to

conduct the hazard assessment, the manufacturer or importer shall present the

information briefly and a justification under the relevant heading of the Chemical

Safety Report (Section 7) and where required and in accordance with Article 29,

summarised in the Safety Data Sheet under heading 12.

3.0.4. The hazard assessment shall comprise the following three steps, which shall be

clearly identified as such in the Chemical Safety Report:

Step 1. Evaluation of information

Step 2. Classification and Labelling

Step 3. Derivation of the Predicted No-Effect Concentration (PNEC)

3.1. Step 1: Evaluation of information

3.1.1. The evaluation of all available information shall comprise:

– the hazard identification based on all available information;

– the establishment of the quantitative dose (concentration) – response (effect)

relationship.

3.1.2. When it is not possible to establish the quantitative dose (concentration) – response

(effect) relationship, then this should be justified and a semi-quantitative or

qualitative analysis shall be included.

3.1.3. All information used to assess the effects on a specific environmental sphere shall be

briefly presented, if possible in the form of a table or tables. The relevant test results

(e.g. LC50 or NOEC) and test conditions (e.g. test duration, route of administration)

and other relevant information shall be presented, in internationally recognised units

of measurement for that effect.

3.1.4. All information used to assess the environmental fate of the substance shall be briefly

presented, if possible in the form of a table or tables. The relevant test results and test

1 In general, the more extensive the data and the longer the duration of the tests, the smaller is the degree

of uncertainty and the size of the assessment factor. An assessment factor of 1000 is typically applied to

the lowest of three short term L(E)C50 values derived from species representing different trophic levels

and a factor of 10 to the lowest of three long-term NOEC values derived from species representing

different trophic levels.

conditions and other relevant information shall be presented, in internationally

recognised units of measurement for that effect.

3.1.5. If one study is available then a robust study summary should be prepared for that

study. Robust summaries will be required of all key data used in the hazard

assessment. Where there are more than one studies addressing the same effect, then

the study or studies giving rise to the highest concern shall be used to draw a

conclusion and a robust study summary shall be prepared for that study or studies

and included as part of the technical dossier. If the study or studies giving rise to the

highest concern are not used, then this shall be fully justified and robust study

summaries shall be prepared and included as part of the technical dossier, not only

for the study being used but also for all studies reaching a higher concern than the

study being used. For substances where all available studies indicate no hazards an

overall assessment of the validity of all studies should be performed.

3.2. Step 2: Classification and Labelling

3.2.1. The appropriate classification and labelling developed in accordance with the criteria

in Directive 67/548/EEC shall be presented and justified.

3.2.2. If the information are inadequate to decide whether a substance should be classified

for a particular end-point, the registrant shall indicate and justify the action or

decision he has taken as a result.

3.3. Step 3: Identification of the Predicted No-Effect Concentration

3.3.1. Based on the available information, the PNEC for each environmental sphere shall be

established. The PNEC may be calculated by applying an appropriate assessment

factor to the effect values (e.g. LC50 or NOEC) . An assessment factor expresses the

difference between effects values derived for a limited number of species from

laboratory tests and the PNEC for the environmental sphere1.

3.3.2. If it is not possible to derive the PNEC, then this shall be clearly stated and fully

justified.

4. PBT AND VPVB ASSESSMENT

4.0. Introduction

4.0.1. The objective of the PBT and vPvB assessment shall be to determine if the substance

fulfils the criteria given in Annex XII and if so, to characterise the potential

emissions of the substance. The information required in Annexes V and VI enables

the registrant to be able to determine if the substance could be a PBT or vPvB. A

hazard assessment in accordance with Sections 1 and 3 of this Annex and an

exposure assessment (in accordance with Section 5 , step 2)are not appropriate for

the identification of substances satisfying the PBT and vPvB criteria in Annex XII.

Therefore, a separate PBT and vPvB assessment is required.

4.0.2. The PBT and vPvB assessment shall be based on all the information submitted as

part of the technical dossier. If the technical dossier contains for one or more

endpoints only information as required in Annexes V and VI, the registrant shall

either consider whether further information needs to be generated to fulfil the

objective of the PBT and vPvB assessment and submit a testing proposal accordingly

or implement and recommend sufficient risk management measures so as to enable

derogation from such testing according to Annex IX, section 3.

4.0.3. The PBT and vPvB assessment shall comprise the following two steps, which shall

be clearly identified as such in Part C, Section 7 of the Chemical Safety Report:

Step 1. Comparison with the Criteria

Step 2. Emission Characterisation

The assessment shall also be summarised in the Safety Data Sheet under heading 12.

4.1. Step 1: Comparison with the Criteria

This part of the PBT and vPvB assessment shall entail the comparison of the available

information with the criteria given in Annex XII and a statement of whether the substance

fulfils or does not fulfil the criteria. If the available information is not sufficient to decide

whether the substance fulfils the criteria in Annex XII, then one of the following three actions

shall be undertaken for each of the criteria for which sufficient information is not available:

(1) submit a testing proposal with the objective of generating the necessary information to

meet the objective of the PBT and vPvB assessment

(2) implement and recommend sufficient risk management measures so as to enable

derogation from such testing according to Annex IX, section 3

(3) consider if there is other evidence giving rise to an equivalent level of concern,

considered on a case-by-case basis.

4.2. Step 2: Emission Characterisation

If the substance fulfils the criteria an emission characterisation shall be conducted comprising

the relevant parts of the exposure assessment as described in Section 5. In particular it shall

contain an estimation of the amounts of the substance released to the different environmental

compartments during all activities carried out by the manufacturer or importer and all

identified uses, and an identification of the likely routes by which humans and the

environment are exposed to the substance.

5. EXPOSURE ASSESSMENT

5.0. Introduction

The objective of the exposure assessment shall be to make a quantitative or qualitative

estimate of the dose/concentration of the substance to which humans and the environment are

or may be exposed. The exposure assessment shall entail the following two steps, which shall

be clearly identified as such in the chemical safety report:

Step 1. Generation of exposure scenario(s) or the identification of relevant use and exposure

categories

Step 2. Exposure Estimation

Where required and in accordance with Article 29, the exposure scenario shall also be

included in an annex to the safety data sheet.

5.1. Step 1: Development of exposure scenarios

5.1.1. Exposure scenarios shall be developed for manufacture in the Community,

manufacturer’s and importer’s own use, and all identified uses. The exposure scenario shall

be presented under the relevant heading of the chemical safety report, and included in an

annex to the safety data sheet, using an appropriate short title giving a brief general

description of the use, consistent with those given in Annex IV section 3.8. In particular, an

exposure scenario includes, where relevant, a description of:

– the processes involved in the production by the manufacturer and, if relevant, the

further processing and use by the manufacturer or importer, including the physical form in

which the substance is manufactured, processed and/or used;

– the processes involved in the identified use of the substance foreseen by the

manufacturer or importer, including the physical form in which the substance is processed

and/or used;

– key stages in each process where release and/or exposure could occur;

– the risk management measures implemented by the manufacturer or importer to

reduce or avoid exposure of humans (including workers and consumers) and the environment

to the substance;

– the risk management measures which the manufacturer or importer recommends to

be implemented by the downstream users to reduce or avoid exposure of humans (including

workers and consumers) and the environment to the substance;

– the waste management measures implemented by the manufacturer or importer and

those recommended to be implemented by the downstream user or consumer to reduce or

avoid exposure of humans and the environment to the substance during waste, disposal and/or

recycling;

– the activities of workers related to the processes and the duration and frequency of

their exposure to the substance;

– the activities of consumers and the duration and frequency of their exposure to the

substance;

– the duration and frequency of emissions of the substance to the different

environmental compartments and sewage treatment systems and the dilution in the receiving

environmental compartment.

5.1.2. Where a manufacturer, importer or downstream user applies for an application for an

authorisation for a specific use, exposure scenarios need only be developed for that

use and the subsequent life-cycle steps.

5.2. Step 2: Exposure Estimation

5.2.1. The exposure shall be estimated for each exposure scenario developed and shall be

presented under the relevant heading of the chemical safety report and where

required and in accordance with Article 29, summarised in an annex to the safety

data sheet. The exposure estimation entails three elements: (1) emission estimation;

(2) chemical fate and pathways; and (3) estimation of exposure levels.

5.2.2. The emission estimation shall consider the emissions during all relevant parts of the

life-cycle of the substance under the assumption that the risk management measures

described in the exposure scenario have been implemented.

5.2.3. A characterisation of possible degradation, transformation, or reaction processes and

an estimation of environmental distribution and fate shall be performed.

5.2.4 An estimation of the exposure levels shall be performed for all human populations

(workers, consumers and humans liable to exposure indirectly via the environment)

and environmental spheres for which exposure to the substance is known or

reasonably foreseeable. Each relevant route of human exposure (inhalation, oral,

dermal and combined through all relevant routes and sources of exposure) shall be

addressed. Such estimations shall take account of spatial and temporal variations in

the exposure pattern. In particular, the exposure estimation shall take account of:

– adequately measured, representative exposure data,

– any major impurities and additives in the substance,

– the quantity in which the substance is produced and/or imported,

– the quantity for each identified use,

– the degree of containment,

– the activities of workers related to the processes and the duration and frequency of

their exposure to the substance,

– the activities of consumers and the duration and frequency of their exposure to the

substance,

– the duration and frequency of emissions of the substance to the different

environmental compartments and sewage treatment systems and the dilution in the receiving

environmental compartment,

– location of exposure,

– the physicochemical properties of the substance,

– transformation and/or degradation products,

– the likely routes of exposure of and potential for absorption in humans,

– the likely pathways to the environment and environmental distribution and

degradation and/or transformation (see also Section 3 Step 1).

– frequency and duration of exposure;

– matrix dependent release/migration of the substance

5.2.5 Where adequately measured representative exposure data are available, special

consideration shall be given to them when conducting the exposure assessment.

Appropriate models can be used for the estimation of exposure levels. Relevant

monitoring data from substances with analogous use and exposure patterns or

analogous properties can also be considered.

6. RISK CHARACTERISATION

6.1 The risk characterisation shall be carried out for each exposure scenario and shall be

presented under the relevant heading of the Chemical Safety Report.

6.2 The risk characterisation shall consider the human populations (exposed as workers,

consumers or indirectly via the environment and if relevant a combination thereof)

and the environmental spheres for which exposure to the substance is known or

reasonably foreseeable, under the assumption that the risk management measures

described in the exposure scenarios in the previous Section have been implemented.

In addition, the overall environmental risk caused by the substance shall be reviewed

by integrating the results for the overall releases, emissions and losses from all

sources to all environmental compartments.

6.3 The risk characterisation consists of:

– a comparison of the exposure of each human population known to be or likely to be

exposed with the appropriate Derived No-Effect Levels;

– a comparison of the predicted environmental concentrations in each environmental

sphere with the PNECs; and

– an assessment of the likelihood and severity of an event occurring due to the

physicochemical properties of the substance.

6.4 For any exposure scenario, the risk of humans and the environment can be

considered to be adequately controlled, if:

– the exposure levels estimated in Section 6.2 do not exceed the appropriate DNEL or

the PNEC, as determined in Sections 1 and 3, respectively, and;

– the likelihood and severity of an event occurring due to the physicochemical

properties of the substance as determined in Section 2 is negligible.

6.5 For those human effects and those environmental spheres for which it was not

possible to determine a DNEL or a PNEC, a qualitative assessment of the likelihood

that effects are avoided when implementing the exposure scenario shall be carried

out.

For substances satisfying the PBT and vPvB criteria, the manufacturer or importer

shall use the information as obtained in Section 5, Step 2 when implementing on its

site, and recommending for downstream users, risk management measures which

minimise exposures and emissions to humans and the environment.

7. CHEMICAL SAFETY REPORT FORMAT

The Chemical Safety Report shall include the following headings:

CHEMICAL SAFETY REPORT FORMAT

PART A

1. SUMMARY OF RISK MANAGEMENT MEASURES

2. DECLARATION THAT RISK MANAGEMENT MEASURES ARE IMPLEMENTED

3. DECLARATION THAT RISK MANAGEMENT MEASURES ARE COMMUNICATED

PART B

1. IDENTIFICATION OF THE SUBSTANCE AND PHYSICAL AND CHEMICAL PROPERTIES

2. CLASSIFICATION AND LABELLING

3. ENVIRONMENTAL FATE PROPERTIES

3.1. Degradation

3.2. Environmental distribution

3.3. Bioaccumulation

3.4 Secondary Poisoning

4. HUMAN HEALTH HAZARD ASSESSMENT

4.1. Toxicokinetics (absorption, metabolism, distribution and elimination)

4.2. Acute toxicity

4.3. Irritation

4.3.1. Skin

4.3.2. Eye

4.3.3. Respiratory Tract

4.4. Corrosivity

4.5. Sensitisation

4.5.1. Skin

4.5.2. Respiratory system

4.6. Repeated dose toxicity

4.7. Mutagenicity

4.8. Carcinogenicity

4.9. Toxicity for reproduction

4.9.1. Effects on fertility

4.9.2. Developmental Toxicity

4.10 Other effects

5. HUMAN HEALTH ASSESSMENT OF PHYSICOCHEMICAL PROPERTIES

5.1. Explosivity

5.2. Flammability

5.3. Oxidising potential

6. ENVIRONMENTAL HAZARD ASSESSMENT

6.1. Aquatic Compartment (including sediment)

6.2. Terrestrial Compartment

6.3. Atmospheric Compartment

6.4. Microbiological Activity in Sewage Treatment Systems

7. PBT AND VPVB ASSESSMENT

8. EXPOSURE ASSESSMENT

8.1. [Title of Exposure Scenario 1]

8.2.1. Exposure Scenario

8.2.2. Exposure Assessment

8.2. [Title of Exposure Scenario 2]

8.3.1. Exposure Scenario

8.3.2. Exposure Assessment

[etc.]

9. RISK CHARACTERISATION

9.1. [Title of Exposure Scenario 1]

9.1.1. Human Health

9.1.1.1. Workers

9.1.1.2. Consumers

9.1.1.3. Humans liable to indirect exposure via the environment

9.1.2. Environment

9.1.2.1. Aquatic Compartment (incl. Sediment)

9.1.2.2. Terrestrial Compartment

9.1.2.3. Atmospheric Compartment

9.1.2.4. Microbiological Activity in Sewage Treatment Systems

9.2. [Title of Exposure Scenario 2]

9.2.1. Human Health

9.2.1.1. Workers

9.2.1.2. Consumers

9.2.1.3. Humans liable to indirect exposure via the environment

9.2.2. Environment

9.2.2.1. Aquatic Compartment (incl. Sediment)

9.2.2.2. Terrestrial Compartment

9.2.2.3. Atmospheric Compartment

9.2.2.4. Microbiological Activity in Sewage Treatment Systems

[etc.]

9.x. Overall exposure (combined for all relevant emission/release sources)

9.x.1 Human health (combined for all exposure routes)

9.x.1.1

9.x.2 Environment (combined for all emission sources)

9.x.2.1

2 OJ L131, 5.5.1998, p.11

ANNEX Ia

GUIDE TO THE COMPILATION OF SAFETY DATA SHEETS

This Annex sets out the requirements for a Safety Data Sheet that is provided for a substance

or a preparation in accordance with Article 29. The Safety Data Sheet provides a mechanism

for transmitting appropriate information from the relevant Chemical Safety Report(s) down

the supply chain to the immediate downstream user(s). The information provided in the Safety

Data Sheet shall be consistent with the information in the chemical safety report, where one is

required. Where a chemical safety report has been performed, the relevant exposure

scenario(s) shall be placed into an annex of the safety data sheet, to make reference to them

under the relevant headings of the safety data sheet easier.

The purpose of this Annex is to ensure consistency and accuracy in the content of each of the

mandatory headings listed in Article 29, so that the resulting safety data sheets will enable

users to take the necessary measures relating to protection of health and safety at the

workplace, and protection of the environment.

The information provided by safety data sheets shall also meet the requirements set out in

Council Directive 98/24/EC(2) on the protection of the health and safety of workers from the

risks related to chemical agents at work. In particular, the safety data sheet shall enable the

employer to determine whether any hazardous chemical agents are present in the workplace,

and to assess any risk to the health and safety of workers arising from their use.

The information in the Safety Data Sheet shall be written in a clear and concise manner. The

safety data sheet shall be prepared by a competent person who shall take into account the

specific needs of the user audience, as far as it is known. Persons placing substances and

preparations on the market shall ensure that competent persons have received appropriate

training, including refresher training.

For preparations not classified as dangerous, but for which a safety data sheet is required

according to Article 29, proportionate information shall be provided under each heading.

Additional information may be necessary in some cases in view of the wide range of

properties of the substances and preparations. If in other cases it emerges that information on

certain properties is of no significance or that it is technically impossible to provide, the

reasons for this shall be clearly stated under each heading. Information shall be provided for

each hazardous property. If it is stated that a particular hazard does not apply, clearly

differentiate between cases where no information is available to the classifier, and cases

where negative test results are available.

Give the date of issue of the safety data sheet on the first page. When a safety data sheet has

been revised, the changes shall be brought to the attention of the recipient and identify it as

'Revision: (date).

Note

Safety data sheets are also required for certain special substances and preparations (e.g. metals

in massive form, alloys, compressed gases etc.) listed in chapters 8 and 9 of Annex VI to

Directive 67/548/EEC, for which there are labelling derogations.

1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE

COMPANY/UNDERTAKING

1.1. Identification of the substance or preparation

The term used for identification shall be identical to that provided on the label as set out in

Annex VI to Directive 67/548/EEC.

For substances subject to registration, the term shall be consistent with that provided under

registration and the registration number assigned under Article 18(1) of this Regulation shall

also be indicated.

Other means of identification available may also be indicated.

1.2. Use of the substance/preparation

Indicate the uses of the substance or preparation as far as they are known. Where there are

many possible uses, only the most important or common uses need to be listed. This shall

include a brief description of what it actually does, e.g. flame retardant, anti-oxidant, etc.

Where a chemical safety report is required, the safety data sheet shall contain information on

all the identified uses relevant to the recipient of the safety data sheet. This information shall

be consistent with the identified uses and exposure scenarios set out in the annex to the safety

data sheet.

1.3. Company/undertaking identification

Identify the person responsible for placing the substance or preparation on the market within

the Community, whether it is the manufacturer, importer or distributor. Give the full address

and telephone number of this person as well as the e-mail address of the competent person

responsible for the safety data sheet.

In addition, where this person is not located in the Member State where the substance or

preparation is placed on the market, give a full address and telephone number for the person

responsible in that Member State, if possible.

For registrants, the person identified shall be consistent with the information on the identity of

the manufacturer or importer provided in the registration.

1.4. Emergency telephone

In addition to the above mentioned information, supply the emergency telephone number of

the company and/or relevant official advisory body (this may be the body responsible for

receiving information relating to health, which is referred to in Article 17 of

Directive 1999/45/EC). Specify if this phone number is available only during office hours.

2. HAZARDS IDENTIFICATION

Give here the classification of the substance or preparation which arises from application of

the classification rules in Directives 67/548/EEC or 1999/45/EC. Indicate clearly and briefly

the hazards the substance or preparation presents to man and the environment.

Distinguish clearly between preparations which are classified as dangerous and preparations

which are not classified as dangerous according to Directive 1999/45/EC.

Describe the most important adverse physicochemical, human health and environmental

effects and symptoms relating to the uses and possible misuses of the substance or preparation

that can reasonably be foreseen.

It may be necessary to mention other hazards, such as dustiness, cross-sensitisation,

suffocation, freezing, high potency for odour or taste or environmental effects such as hazards

to soil-dwelling organisms, ozone depletion, photochemical ozone creation potential etc.,

which do not result in classification but which may contribute to the overall hazards of the

material.

The information shown on the label shall be given under heading 15.

The classification of the substance shall be consistent with the classification provided to the

classification and labelling inventory according to Title X.

3. COMPOSITION/INFORMATION ON INGREDIENTS

The information given shall enable the recipient to identify readily the hazards of the

components of the preparation. The hazards of the preparation itself shall be given under

heading 2.

3.1. It is not necessary to give the full composition (nature of the ingredients and their

concentration), although a general description of the components and their

concentrations can be helpful.

3.2. For a preparation classified as dangerous according to Directive 1999/45/EC, the

following substances shall be indicated, together with their concentration or

concentration range in the preparation:

(i) substances presenting a health or environmental hazard within the meaning of

Directive 67/548/EEC, if they are present in concentrations equal to or greater

than the lowest of:

- the applicable concentrations defined in the table of Article 3 (3) of

European Parliament and Council Directive 1999/45/EC, or

- the concentration limits given in Annex I to Council

Directive 67/548/EEC, or

- the concentration limits given in Part B of Annex II to European

Parliament and Council Directive 1999/45/EC, or

3 Where the person responsible for placing the preparation on the market can demonstrate that the

disclosure in the safety data sheet of the chemical identity of a substance which is exclusively classified

as: - irritant with the exception of those assigned R41 or irritant in combination with one or more of the

properties mentioned in point 2.3.4 of Article 10 of Directive 1999/ 45/EC; or - harmful in combination

with one or more of the properties mentioned in point 2.3.4 of Article 10 of Directive 1999/45/EC

presenting acute lethal effects alone; will put at risk the confidential nature of his intellectual property,

he may, in accordance with the provisions of Part B of Annex VI to Directive 1999/45/EC, refer to that

substance either by means of a name that identifies the most important functional chemical groups, or

by means of an alternative name.

- the concentration limits given in Part B of Annex III to European

Parliament and Council Directive 1999/45/EC, or

- the concentration limits given in Annex V to European Parliament and

Council Directive 1999/45/EC, or

- the concentration limits given in an agreed entry in the classification and

labelling inventory established under Title X,

(ii) and substances for which there are Community workplace exposure limits,

which are not already included under (i).

(iii) substances that are persistent, bioaccumulative and toxic or very persistent and

very bioaccumulative in accordance with the criteria set out in Annex XII, if

the concentration of an individual substance is equal to or greater than 0.1 %.

3.3. For a preparation not classified as dangerous according to Directive 1999/45/EC, the

following substances shall be indicated, together with their concentration or

concentration range, if they are present in an individual concentration of = 1% by

weight for non-gaseous preparations and = 0,2% by volume for gaseous preparations:

– substances presenting a health or environmental hazard within the meaning of

Directive 67/548/EEC(3);

– and substances for which there are Community workplace exposure limits.

3.4. The classification (derived either from Articles 4 and 6 of Directive 67/548/EEC,

from Annex I to Directive 67/548/EEC or from an agreed entry in the classification

and labelling inventory established under Title X) of the above substances shall be

given, including the symbol letters and R phrases which are assigned in accordance

with their physicochemical, health and environmental hazards. The R phrases do not

need to be written out in full here: reference shall be made to heading 16, where the

full text of each relevant R phrase shall be listed. If the substance does not meet the

classification criteria, the reason for indicating the substance in section 3 shall be

described, such as ‘PBT-substance’ or ‘substance with a Community workplace

exposure limit’.3.5. The name and the Registration number, assigned under

Article 18(1) of this Regulation, Einecs or Elincs number, if available, of the above

substances shall be given in accordance with Directive 67/548/EEC. The CAS

number and IUPAC name (if available) may also be helpful. For substances listed by a

generic name, according to Article 15 of Directive 1999/45/EC or the footnote to

point 3.3 of this Annex, a precise chemical identifier is not necessary..

3.6. If, in accordance with the provisions of Article 15 of Directive 1999/45/EC or the

footnote to point 3.3 of this Annex, the identity of certain substances is to be kept

confidential, their chemical nature shall be described in order to ensure safe

handling. The name used shall be the same as that which derives from the above

procedures.

4. FIRST AID MEASURES

Describe the first-aid measures.

Specify first whether immediate medical attention is required.

The information on first aid shall be brief and easy to understand by the victim, bystanders

and first-aiders. The symptoms and effects shall be briefly summarised. The instructions shall

indicate what is to be done on the spot in the case of an accident and whether delayed effects

can be expected after exposure.

Subdivide the information according to the different routes of exposure, i.e. inhalation, skin

and eye contact and ingestion, under different subheadings.

Indicate whether professional assistance by a doctor is needed or advisable.

For some substances or preparations it may be important to emphasise that special means to

provide specific and immediate treatment shall be available at the workplace.

5. FIRE-FIGHTING MEASURES

Refer to requirements for fighting a fire caused by the substance or preparation, or arising in

its vicinity by indicating:

– suitable extinguishing media,

– extinguishing media which shall not be used for safety reasons,

– special exposure hazards arising from the substance or preparation itself, combustion

products, resulting gases,

– special protective equipment for fire-fighters.

6. ACCIDENTAL RELEASE MEASURES

Depending on the substance or preparation involved, information may be needed on:

– personal precautions such as:

removal of ignition sources, provision for sufficient ventilation/respiratory

protection, control of dust, prevention of skin and eye contact,

– environmental precautions such as:

keeping away from drains, surface- and ground-water and soil, possible need to alert

the neighbourhood,

– methods for cleaning up such as:

use of absorbent material (e.g. sand, diatomaceous earth, acid binder, universal

binder, sawdust, etc.), reduction of gases/fumes with water, dilution.

Also consider the need for indications such as: "never use, neutralise with ...".

Note

If appropriate refer to headings 8 and 13.

7. HANDLING AND STORAGE

Note

Information in this section shall relate to the protection of health, safety and the environment.

It shall assist the employer in devising suitable working procedures and organisational

measures according to Article 5 of Directive 98/24/EC.

Where a chemical safety report or a registration is required, the information in this section

shall be consistent with the information given, for the identified uses and exposure scenarios

set out in the annex to the safety data sheet.

7.1. Handling

Specify precautions for safe handling including advice on technical measures such as:

containment, local and general ventilation, measures to prevent aerosol and dust generation

and fire, measures required to protect the environment (e.g. use of filters or scrubbers on

exhaust ventilation, use in a bunded area, measures for collection and disposal of

spillages, etc.) and any specific requirements or rules relating to the substance or preparation

(e.g. procedures or equipment which are prohibited or recommended) and if possible give a

brief description.

7.2. Storage

Specify the conditions for safe storage such as: specific design for storage rooms or vessels

(including retention walls and ventilation), incompatible materials, conditions of storage

(temperature and humidity limit/range, light, inert gas, etc.) special electrical equipment and

prevention of static electricity.

Give advice if relevant on quantity limits under storage conditions. In particular indicate any

special requirements such as the type of material used in the packaging/containers of the

substance or preparation.

7.3. Specific use(s)

For end products designed for specific use(s), recommendations shall refer to the identified

use(s) and be detailed and operational. If possible, reference shall be made to industry - or

sector - specific approved guidance.

4 OJ L399 30.12.1989 p 18

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1. Exposure limit values

Specify currently applicable specific control parameters including occupational exposure limit

values and/or biological limit values. Values shall be given for the Member State where the

substance or preparation is placed on the market. Give information on currently recommended

monitoring procedures.

Where a chemical safety report is required, the relevant DNELs and PNECs for the substance

shall be given for the exposure scenarios set out in the annex to the safety data sheet.

For preparations, it is useful to provide values for those constituent substances which are

required to be listed in the safety data sheet according to heading 3.

8.2. Exposure controls

For the purposes of this document exposure control means the full range of specific risk

management measures to be taken during use in order to minimise worker and environmental

exposure. Where a chemical safety report is required, a summary of the risk management

measures shall be given in section 8 of the safety data sheet for the identified uses set out in

the safety data sheet.

8.2.1. Occupational exposure controls

This information will be taken into account by the employer in carrying out an assessment of

risk to the health and safety of workers for the substance or preparation under Article 4 of

Directive 98/24/EC, which requires, in the order of priority:

- design of appropriate work processes and engineering controls, the use of adequate

equipment and materials;

- the application of collective protection measures at source, such as adequate ventilation

and appropriate organisational measures, and

- where exposure cannot be prevented by other means the use of individual protection

measures, such as personal protection equipment.

Therefore provide suitable and adequate information on these measures to enable a proper risk

assessment to be carried out under Article 4 of Directive 98/24/EC. This information shall

complement that already given under heading 7.1.

Where individual protection measures are needed, specify in detail which equipment will

provide adequate and suitable protection. Take into account Council Directive 89/686/EEC(4)

and make reference to the appropriate CEN standards:

(a) Respiratory protection

For dangerous gases, vapours or dust, specify the type of protective equipment to be used,

such as self contained breathing apparatus, adequate masks and filters.

(b) Hand protection

Specify clearly the type of gloves to be worn when handling the substance or preparation,

including:

– the type of material,

– the breakthrough time of the glove material, with regard to the amount and duration

of dermal exposure.

If necessary indicate any additional hand protection measures.

(c) Eye protection

Specify the type of eye protection equipment required such as: safety glasses, safety goggles,

face shield.

(d) Skin protection

If it is necessary to protect a part of the body other than the hands, specify the type and quality

of protection equipment required, such as: apron, boots and full protective suit. If necessary,

indicate any additional skin protection measures and specific hygiene measures.

8.2.2. Environmental exposure controls

Specify the information required by the employer to fulfil his commitments under Community

environmental protection legislation.

Where a chemical safety report is required, a summary of the risk management measures that

adequately control exposure of the environment to the substance shall be given for the

exposure scenarios set out in the annex to the safety data sheet.

9. PHYSICAL AND CHEMICAL PROPERTIES

To enable proper control measures to be taken, provide all relevant information on the

substance or preparation, particularly the information listed under heading 9.2. The

information in this section shall be consistent with the information provided in a registration

where one is required.

9.1. General information

Appearance

Indicate the physical state (solid, liquid, gas) and the colour of the substance or preparation as

supplied.

Odour

If odour is perceptible, give a brief description of it.

5 OJ L100 19.4.1994 p. 1

9.2. Important health, safety and environmental information

pH

Indicate the pH of the substance or preparation as supplied or of an aqueous solution; in the

latter case, indicate the concentration.

Boiling point/boiling range:

Flash point:

Flammability (solid, gas):

Explosive properties:

Oxidising properties:

Vapour pressure:

Relative density:

Solubility:

Water solubility:

Partition coefficient: n-octanol/water:

Viscosity:

Vapour density:

Evaporation rate:

9.3. Other information

Indicate other important safety parameters, such as, miscibility, fat solubility (solvent – oil to

be specified), conductivity, melting point/melting range, gas group (useful for European

Parliament and Council Directive 94/9/EC)(5), auto-ignition temperature etc.

Note 1

The above properties shall be determined in accordance with the specifications of Part A of

Annex X or any other comparable method.

Note 2

For preparations, information shall normally be given on the properties of the preparation

itself. However, if it is stated that a particular hazard does not apply, clearly differentiate

between cases where no information is available to the classifier, and cases where negative

test results are available. If it is considered necessary to give information about the properties

of individual components, please indicate clearly what the data refers to.

10. STABILITY AND REACTIVITY

State the stability of the substance or preparation and the possibility of hazardous reactions

occurring under certain conditions of use and also if released into the environment.

10.1. Conditions to avoid

List those conditions such as temperature, pressure, light, shock, etc., which may cause a

dangerous reaction and if possible give a brief description.

10.2. Materials to avoid

List materials such as water, air, acids, bases, oxidising agents or any other specific substance

which may cause a dangerous reaction and if possible give a brief description.

10.3. Hazardous decomposition products

List hazardous materials produced in dangerous amounts upon decomposition.

Note

Address specifically:

– the need for and the presence of stabilisers,

– the possibility of a hazardous exothermic reaction,

– safety significance, if any, of a change in physical appearance of the substance or

preparation,

– hazardous decomposition products, if any, formed upon contact with water,

– possibility of degradation to unstable products.

11. TOXICOLOGICAL INFORMATION

This section deals with the need for a concise but complete and comprehensible description of

the various toxicological (health) effects, which can arise if the user comes into contact with

the substance or preparation.

The information shall include dangerous-to-health effects from exposure to the substance or

preparation, based on the conclusion from, for example, test data and experience. The

information shall also include, where appropriate, delayed, immediate and chronic effects

from short- and long-term exposure: for example sensitisation, narcosis, carcinogenicity,

mutagenicity and reproductive toxicity (developmental toxicity and fertility). It shall also

include information on the different routes of exposure (inhalation, ingestion, skin and eye

contact), and describe the symptoms related to the physical, chemical and toxicological

characteristics.

Taking account of the information already provided under heading 3,

composition/information on ingredients, it may be necessary to make reference to specific

health effects of certain substances in the preparation.

The information in this section shall be consistent with the information provided for in a

registration where required and/or in a chemical safety report where required and shall give

information on the following groups of potential effects:

– toxicokinetics, metabolism and distribution,

– acute effects (acute toxicity, irritation and corrosivity),

– sensitisation,

– repeated dose toxicity, and

– CMR effects (carcinogenity, mutagenicity and toxicity for reproduction).

For substances subject to registration, summaries of the information derived from the

application of Annexes V to IX of this Regulation shall be given. The information shall also

include the result of the comparison of the available data with the criteria given in

Directive 67/548/EEC for CMR, categories 1 and 2, following Paragraph 1.3.1 of Annex I.

12. ECOLOGICAL INFORMATION

Describe the possible effects, behaviour and environmental fate of the substance or

preparation in air, water and/or soil. Where available, give relevant test data (e.g. LC50 fish

= 1 mg/l).

The information in this section shall be consistent with the information provided for in a

registration where required and/or in a chemical safety report where required.

Describe the most important characteristics likely to have an effect on the environment owing

to the nature of the substance or preparation and likely methods of use. Information of the

same kind shall be supplied for dangerous products arising from the degradation of substances

and preparations. This may include the following:

12.1. Ecotoxicity

This shall include relevant available data on aquatic toxicity, both acute and chronic for fish,

crustaceans, algae and other aquatic plants. In addition, toxicity data on soil micro- and

macro-organisms and other environmentally relevant organisms, such as birds, bees and

plants, shall be included when available. Where the substance or preparation has inhibitory

effects on the activity of micro-organisms, the possible impact on sewage treatment plants

shall be mentioned.

For substances subject to registration, summaries of the information derived from the

application of Annexes V to IX of this Regulation shall be included.

6 This information cannot be given for the preparation because it is substance specific. It should therefore be given, where available

and appropriate, for each constituent substance in the preparation which is required to be listed in the safety data sheet according

to the rules under heading 3 of this Annex.

12.2. Mobility

The potential of the substance or the appropriate constituents of a preparation (30), if released

to the environment, to transport to groundwater or far from the site of release.

Relevant data might include:

– known or predicted distribution to environmental compartments,

– surface tension,

– absorption/desorption.

For other physicochemical properties see heading 9.

12.3. Persistence and degradability

The potential of the substance or the appropriate constituents of a preparation (6) to degrade in

relevant environmental media, either through biodegradation or other processes such as

oxidation or hydrolysis. Degradation half lives shall be quoted where available. The potential

of the substance or appropriate constituents of a preparation(6) to degrade in sewage treatment

plants shall also be mentioned.

12.4. Bioaccumulative potential

The potential of the substance or the appropriate constituents of a preparation (6) to

accumulate in biota and, eventually, to pass through the food chain, with reference to the

octanol-water partition coefficient (Kow) and bioconcentration factor (BCF), if available.

12.5. Results of PBT assessment

Where a chemical safety report is required, the results of the PBT assessment as set in the

Chemical Safety Report shall be given.

12.6 Other adverse effects

If available, include information on any other adverse effects on the environment, e.g. ozone

depletion potential, photochemical ozone creation potential, endocrine disrupting potential

and/or global warming potential.

Remarks

Ensure that information relevant to the environment is provided under other headings of the

safety data sheet, especially advice for controlled release, accidental release measures,

transport and disposal considerations under headings 6, 7, 13, 14 and 15.

13. DISPOSAL CONSIDERATIONS

If the disposal of the substance or preparation (surplus or waste resulting from the foreseeable

use) presents a danger, a description of these residues and information on their safe handling

shall be given.

Specify the appropriate methods of disposal of both the substance or preparation and any

contaminated packaging (incineration, recycling, landfilling, etc.)

Where a chemical safety report is required, the information on the waste management

measures that adequately control exposure of humans and the environment to the substance

shall be consistent with the exposure scenarios set out in the annex to the safety data sheet.

Note

Refer to any relevant Community provisions relating to waste. In their absence, it is useful to

remind the user that national or regional provisions may be in force.

14. TRANSPORT INFORMATION

Indicate any special precautions which a user needs to be aware of or needs to comply with in

connection with transport or conveyance either within or outside his premises. Where

relevant, provide information on the transport classification for each of the modal regulations:

IMDG (sea), ADR (road, Council Directive 94/55/EC(9)), RID (rail, Council

Directive 96/49/EC(10)), ICAO/IATA (air). This might include inter alia:

– UN number,

– class,

– proper shipping name,

– packing group,

– marine pollutant,

– other applicable information.

15. REGULATORY INFORMATION

Give the health, safety and environmental information shown on the label according to

Directives 67/548/EEC and 1999/45/EC.

If the substance or preparation covered by this safety data sheet is the subject of specific

provisions in relation to protection of man or the environment at Community level

(e.g. authorisations given under Title VII or restrictions under Title VIII) these provisions

shall, as far as is possible, be stated.

Also mention, where possible, the national laws which implement these provisions and any

other national measures that may be relevant.

16. OTHER INFORMATION

Indicate any other information which the supplier assesses as being of importance for the

health and safety of the user and for the protection of the environment, for example:

– list of relevant R phrases. Write out the full text of any R phrases referred to under

headings 2 and 3 of the safety data sheet,

– training advice,

– recommended restrictions on use (i.e. non-statutory recommendations by supplier),

– further information (written references and/or technical contact point),

– sources of key data used to compile the data sheet,

For a revised safety data sheet, indicate clearly the information, which has been added,

deleted or revised (unless this has been indicated elsewhere).

ANNEX Ib

CHEMICAL SAFETY ASSESSMENTS FOR PREPARATIONS

[deleted]

ANNEX Ic

CRITERIA FOR SUBSTANCES REGISTERED IN QUANTITIES OVER 1 TONNE FOR WHICH THE

INFORMATION IN ANNEX V IS REQUIRED

For substances manufactured or imported in quantities of 1 tonne or more per year per

manufacturer or importer meeting one or more of the criteria (a) – (c) in this Annex, the

technical dossier referred to in Article 9 (a) shall include under points (vi), (vii) and (viii) of

that provision all physicochemical, toxicological and ecotoxicological information that is

relevant and available and, as a minimum, the information specified in Annex V.

(a) substances for which no information is available on human health or environmental

endpoints as described in sections 6 and 7 of Annexes V – VIII.

(b) substances for which there is an indication (eg; based on results from valid qualitative or

quantitative structure-activity relationship models ((Q)SARs)) that they are likely to meet:

(i) the criteria for classification as carcinogenic, mutagenic or toxic for reproduction,

category 1 or 2; or

(ii) the criteria in Annex XII.

(c) substances with dispersive or diffuse use(s) particularly where such substances are

incorporated into consumer articles.

ANNEX II

EXEMPTIONS FROM OBLIGATION TO REGISTER

IN ACCORDANCE WITH ARTICLE 4 (2) (a)

EINECS no

Name/Group

CAS no

200-061-5

D-glucitol C6H14O6

50-70-4

200-066-2

Ascorbic acid C6H8O6

50-81-7

200-075-1

Glucose C6H12O6

50-99-7

200-294-2

L-lysine C6H14N2O2

56-87-1

200-312-9

Palmitic acid, pure C16H32O2

57-10-3

200-313-4

Stearic acid, pure C18H36O2

57-11-4

200-334-9

Sucrose, pure C12H22O11

57-50-1

200-405-4

a-tocopheryl acetate C31H52O3

58-95-7

200-432-1

DL-methionine C5H11NO2S

59-51-8

200-711-8

D-mannitol C6H14O6

69-65-8

201-771-8

1-sorbose C6H12O6

87-79-6

204-007-1

Oleic acid, pure C18H34O2

112-80-1

204-664-4

Glycerol stearate, pure C21H42O4

123-94-4

204-696-9

Carbon dioxide CO2

124-38-9

205-278-9

Calcium pantothenate, D-form C9H17NO5.1/2Ca

137-08-6

205-582-1

Lauric acid, pure C12H24O2

143-07-7

205-590-5

Potassium oleate C18H34O2K

143-18-0

205-756-7

DL-phenylalanine C9H11NO2

150-30-1

208-407-7

Sodium gluconate C6H12O7.Na

527-07-1

212-490-5

Sodium stearate, pure C18H36O2.Na

822-16-2

215-279-6

Limestone

A noncombustible solid characteristic of

sedimentary rock. It consists primarily of calcium

carbonate

1317-65-3

215-665-4

Sorbitan oleate C24H44O6

1338-43-8

216-472-8

Calcium distearate, pure C18H36O2.1/2Ca

1592-23-0

231-147-0

Argon Ar

7440-37-1

231-153-3

Carbon C

7440-44-0

231-783-9

Nitrogen N2

7727-37-9

231-791-2

Water, distilled, conductivity or of similar purity

H2O

7732-18-5

231-955-3

Graphite C

7782-42-5

232-273-9

Sunflower oil

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acids linoleic, and oleic. (Helianthus

annuus, Compositae).

8001-21-6

232-274-4

Soybean oil

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acids linoleic, oleic, palmitic and stearic

(Soja hispida, Leguminosae).

8001-22-7

232-276-5

Safflower oil

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acid linoleic (Carthamus tinctorius,

Compositae).

8001-23-8

232-278-6

Linseed oil

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acids linoleic, linolenic and oleic (Linum

usitatissimum, Linaceae).

8001-26-1

232-281-2

Corn oil

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acids linoleic, oleic, palmitic and stearic.

(Zea mays, Gramineae).

8001-30-7

232-293-8

Castor Oil

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acid ricinoleic (Ricinus communis,

Euphorbiaceae).

8001-79-4

232-299-0

Rape oil

8002-13-9

Extractives and their physically modified

derivatives. It consists primarily of the glycerides of

the fatty acids erucic, linoleic and oleic (Brassica

napus, Cruciferae).

232-307-2

Lecithins

The complex combination of diglycerides of fatty

acids linked to the choline ester of phosphoric acid.

8002-43-5

232-436-4

Syrups, hydrolyzed starch

A complex combination obtained by the hydrolysis

of cornstarch by the action of acids or enzymes. It

consists primarily of d-glucose, maltose and

maltodextrins.

8029-43-4

232-442-7

Tallow, hydrogenated

8030-12-4

232-675-4

Dextrin

9004-53-9

232-679-6

Starch

High-polymeric carbohydrate material usually

derived form cereal grains such as corn, wheat and

sorghum, and from roots and tubers such as

potatoes and tapioca. Includes starch which has

been pregelatinised by heating in the presence of

water.

9005-25-8

232-940-4

Maltodextrin

9050-36-6

234-328-2

Vitamin A

11103-57-4

238-976-7

Sodium D-gluconate C6H12O7.xNa

14906-97-9

248-027-9

D-glucitol monostearate C24H48O7

26836-47-5

262-988-1

Fatty acids, coco, Me esters

61788-59-8

262-989-7

Fatty acids, tallow, Me esters

61788-61-2

263-060-9

Fatty acids, castor-oil

61789-44-4

263-129-3

Fatty acids, tallow

61790-37-2

266-925-9

Fatty acids, C12-18

This substance is identified by SDA Substance

Name: C12-C18 alkyl carboxylic acid and SDA

Reporting Number: 16-005-00.

67701-01-3

266-928-5

Fatty acids C16-18

This substance is identified by SDASubstance

67701-03-5

Name: C16-C18 alkyl carboxylic acid and SDA

Reporting Number: 19-005-00.

266-929-0

Fatty acids, C8-18 and C18-unsatd.

This substance is identified by SDA Substance

Name: C8-C18 and C18 unsaturated alkyl carboxylic

acid and SDA Reporting Number: 01-005-00.

67701-05-7

266-930-6

Fatty acids, C14-18 and C16-18-unsatd.

This substance is identified by SDA Substance

Name: C14-C18 and C16-C18 unsaturated alkyl

carboxylic acid and SDA Reporting Number: 04-

005-00

67701-06-8

266-932-7

Fatty acids, C16-C18 and C18-unsatd.

This substance is identified by SDA Substance

Name: C16-C18 and C18 unsaturated alkyl carboxylic

acid and SDA Reporting Number: 11-005-00

67701-08-0

266-948-4

Glycerides, C16-18 and C18-unsatd.

This substance is identified by SDA Substance

Name: C16-C18 and C18 unsaturated trialkyl

glyceride and SDA Reporting Number: 11-001-00.

67701-30-8

267-007-0

Fatty acids, C14-18 and C16-18-unsatd., Me esters

This substance is identified by SDA Substance

Name: C14-C18 and C16-C18 unsaturated alkyl

carboxylic acid methyl ester and SDA Reporting

Number: 04-010-00.

67762-26-9

267-013-3

Fatty acids, C6-12

This substance is identified by SDA Substance

Name: C6-C12 alkyl carboxylic acid and SDA

Reporting Number: 13-005-00.

67762-36-1

268-099-5

Fatty acids, C14-22 and C16-22 unsatd.

This substance is identified by SDA Substance

Name: C14-C22 and C16-C22 unsaturated alkyl

carboxylic acid and SDA Reporting Number: 07-

005-00

68002-85-7

268-616-4

Syrups, corn, dehydrated

68131-37-3

269-657-0

Fatty acids, soya

68308-53-2

269-658-6

Glycerides, tallow mono- , di- and tri-,

hydrogenated

68308-54-3

270-298-7

Fatty acids, C14-22

68424-37-3

270-304-8

Fatty acids, linseed-oil

68424-45-3

270-312-1

Glycerides, C16-18 and C18-unsatd. mono- and di-

This substance is identified by SDA Substance

Name: C16-C18 and C18 unsaturated alkyl and C16-

C18 and C18 unsaturated dialkyl glyceride and SDA

Reporting Number: 11-002-00.

68424-61-3

288-123-8

Glycerides, C10-18

85665-33-4

292-771-7

Fatty acids, C12-14

90990-10-6

292-776-4

Fatty acids, C12-18 and C18-unsatd.

90990-15-1

296-916-5

Fatty acids, rape-oil, erucic acid-low

93165-31-2

ANNEX III

EXEMPTIONS FROM THE OBLIGATION TO REGISTER

IN ACCORDANCE WITH ARTICLE 4 (2) (b)

2. Substances which result from a chemical reaction that occurs incidental to exposure

of another substance or article to environmental factors such as air, moisture,

microbial organisms or sunlight;

3. Substances which result from a chemical reaction that occurs incidental to storage of

another substance, preparation or article;

4. Substances which result from a chemical reaction occurring upon end use of other

substances, preparations or articles and which are not themselves manufactured,

imported or placed on the market;

5. Substances which are not themselves manufactured, imported or placed on the

market and which result from a chemical reaction that occurs when:

(i) a stabiliser, colorant, flavouring agent, antioxidant, filler, solvent, carrier,

surfactant, plasticiser, corrosion inhibitor, antifoamer or defoamer, dispersant,

precipitation inhibitor, desiccant, binder, emulsifier, de-emulsifier, dewatering

agent, agglomerating agent, adhesion promoter, flow modifier, pH neutraliser,

sequesterant, coagulant, flocculant, fire retardant, lubricant, chelating agent, or

quality control reagent functions as intended, or

(ii) a substance solely intended to provide a specific physicochemical characteristic

functions as intended;

6. By-products, unless they are imported or placed on the market themselves;

7. Hydrates of a substance or hydrated ions, formed by association of a substance with

water, provided that the substance has been registered by the manufacturer or

importer using this exemption;

8. The following substances which occur in nature, if they are not chemically modified:

Ores, ore concentrates, natural gas, liquefied petroleum gas, natural gas condensate,

process gases and components thereof, crude oil, coal, coke;

8a. Minerals and other substances occurring in nature not listed under paragraph 8, if they

are not chemically modified, unless they meet the criteria for classification as

dangerous according to Directive 67/548/EEC;

9. Basic elemental substances for which hazards and risks are already well known:

hydrogen, oxygen, noble gases (argon, helium, neon, xenon), nitrogen.

ANNEX IV

INFORMATION REQUIREMENTS REFERRED TO IN ARTICLE 9

GUIDANCE NOTE

ON FULFILLING THE REQUIREMENTS OF ANNEXES IV TO IX

Annexes IV to IX specify the information that shall be submitted for registration and

evaluation purposes according to Articles 9, 11 and 12, 39, 40 and 44. For the lowest tonnage

level, the standard requirements are in Annex V, and every time a new tonnage level is

reached, the requirements of the corresponding Annex have to be added. For each registration

the precise information requirements will differ, according to tonnage, use and exposure. The

Annexes shall thus be considered as a whole, and in conjunction with the overall requirements

of registration, evaluation and the duty of care.

STEP 1 – GATHER AND SHARE EXISTING INFORMATION

The registrant should gather all existing available test data on the substance to be registered,

this would include a literature search for relevant information on the substance. Wherever

practicable, registrations should be submitted by consortia, in accordance with Article 10 or

17. This will enable test data to be shared, thereby avoiding unnecessary testing and reducing

costs. The registrant should also collect all other available and relevant information on the

substance regardless whether testing for a given endpoint is required or not at the specific

tonnage level. This should include alternative data (e.g. from (Q)SARs, read-across from

other substances, in-vitro testing, epidemiological data) which may assist in identifying the

presence or absence of hazardous properties of the substance and which can in certain cases

replace the results of animal tests. In addition, information on exposure, use and risk

management measures in accordance with Article 9 and this Annex should be collected.

Considering all this information together, the registrant will be able to determine the need to

generate further information.

STEP 2 – CONSIDER INFORMATION NEEDS

The registrant shall identify what information is required for the registration. First, the

relevant Annex or Annexes to be followed shall be identified, according to tonnage. These

Annexes set out the standard information requirements, but shall be considered in conjunction

with Annex IX, which allows variation from the standard approach, where it can be justified.

In particular, information on exposure, use and risk management measures shall be considered

at this stage in order to determine the information needs for the substance.

STEP 3 – IDENTIFY INFORMATION GAPS

The registrant shall then compare the information needs for the substance with the

information already available and identify where there are gaps. It is important at this stage to

ensure that the available data is relevant and has sufficient quality to fulfil the requirements.

STEP 4 – GENERATE NEW DATA/PROPOSE TESTING STRATEGY

In some cases it will not be necessary to generate new data. However, where there is an

information gap that needs to be filled, new data shall be generated (Annexes V and VI), or a

testing strategy shall be proposed (Annexes VII and VIII), depending on the tonnage. New

tests on vertebrates shall only be conducted or proposed as a last resort when all other data

sources have been exhausted.

In some cases, the rules set out in Annex V to IX may require certain tests to be undertaken

earlier than or in addition to the standard requirements.

NOTES

Note 1: If it is not technically possible, or if it does not appear scientifically necessary to give

information, the reasons shall be clearly stated, in accordance with the relevant provisions.

Note 2: The registrant may wish to declare that certain information submitted in the

registration dossier is confidential. If this is the case, he shall list the items and provide a

justification in accordance with Article 115.

INFORMATION REFERRED TO IN ARTICLE 9 (1) (A) (I) TO (V)

1. GENERAL REGISTRANT INFORMATION

1.1. Registrant

1.1.1. Name, address, telephone number, fax number and e-mail address

1.1.2. Contact person

1.1.3. Location of the registrant’s production and own use site(s), as appropriate

1.2. Joint submission of data by consortia: other consortia members

Articles 10 or 17 foresee that parts of the registration may be submitted by one manufacturer

or importer on behalf of other members of the consortium.

In this case, that manufacturer or importer shall identify the other members of the consortium

specifying:

– their name, address, telephone number, fax number and e-mail address,

– parts of the present registration which apply to other members of the consortium.

Mention the number(s) given in Annex IV, V, VI, VII or VIII, as appropriate.

Any other consortium members shall identify the manufacturer/importer submitting on his

behalf specifying:

– his name, address, telephone number, fax number and e-mail address,

– parts of the registration which are submitted by those manufacturer(s) or importer(s).

Mention the number(s) given in Annex IV, V, VI, VII or VIII, as appropriate.

2. IDENTIFICATION OF THE SUBSTANCE

For each substance, the information given in this section shall be sufficient to enable each

substance to be identified. If it is not technically possible or if it does not appear scientifically

necessary to give information on one or more of the items below, the reasons shall be clearly

stated.

2.1. Name or other identifier of each substance

2.1.1. Name(s) in the IUPAC nomenclature or other international chemical name(s)

2.1.2. Other names (usual name, trade name, abbreviation)

2.1.3. EINECS or ELINCS number (if available and appropriate)

2.1.4. CAS name and CAS number (if available)

2.1.5. Other identity code (if available)

2.2. Information related to molecular and structural formula of each substance

2.2.1. Molecular and structural formula (including SMILES notation, if available)

2.2.2 Information on optical activity and typical ratio of (stereo) isomers (if applicable

and appropriate)

2.2.3. Molecular weight or molecular weight range

2.3. Composition of each substance

2.3.1. Degree of purity (%)

2.3.2. Nature of impurities, including isomers and by-products

2.3.3. Percentage of (significant) main impurities

2.3.4. Nature and order of magnitude (......ppm, ......%) of any additives (e.g. stabilising

agents or inhibitors)

2.3.5. Spectral data (ultra-violet, infra-red, nuclear magnetic resonance or mass spectrum)

2.3.6. High-pressure liquid chromatogram, gas chromatogram

2.3.7. Description of the analytical methods or the appropriate bibliographical references

for the identification of the substance and, where appropriate, for the identification

of impurities and additives. This information shall be sufficient to allow the methods

to be reproduced.

3. INFORMATION ON MANUFACTURE AND USE(S) OF THE SUBSTANCE(S)

3.1. Overall manufacture and/or imports in tonnes per manufacturer or importer

per year in:

3.1.1. The calendar year of the registration (estimated quantity)

3.2. In case of a manufacturer: Brief description of the technological process used in

manufacture

Precise details of the process, particularly those of a commercially sensitive nature,

are not required.

3.3. An indication of the tonnage used for his own use(s)

3.4. Form (substance, preparation or article) and/or physical state under which the

substance is made available to downstream users. Concentration or

concentration range of the substance in preparations made available to

downstream users and quantities of the substance in articles made available to

downstream users.

3.5. Brief general description of the identified use(s)

3.6. Information on waste quantities and composition of waste resulting from

production and identified uses

3.7. Uses advised against (see safety data sheet heading 16)

Where applicable, an indication of the uses, which the registrant advises against and

why (i.e. non-statutory recommendations by supplier). This need not be an

exhaustive list.

4. CLASSIFICATION AND LABELLING

4.1. The hazard classification of the substance(s), resulting from the application of

Articles 4 and 6 of Directive 67/548/EEC;

In addition, for each entry, the reasons why no classification is given for an endpoint

should be provided (i.e. if data are lacking, inconclusive, or conclusive but not

sufficient for classification);

4.2. The resulting hazard label for the substance(s), resulting from the application of

Articles 23 to 25 of Directive 67/548/EEC;

4.3. Specific concentration limits, where applicable, resulting from the application of

Article 4 (4) of Directive 67/548/EEC and Articles 4 to 7 of Directive

1999/45/EC.

5. GUIDANCE ON SAFE USE CONCERNING:

This information shall be consistent with that in the Safety Data Sheet, where such a Safety

Data Sheet is required according to Article 29 of this Regulation.

5.1. First-aid measures (safety data sheet heading 4)

5.2. Fire-fighting measures (safety data sheet heading 5)

5.3. Accidental release measures (safety data sheet heading 6)

5.4. Handling and Storage (safety data sheet heading 7)

5.5. Transport information (safety data sheet heading 14)

Where a chemical safety report is not required, the following additional information is

required:

5.6. Exposure Controls/Personal Protection (safety data sheet heading 8)

5.7. Stability and Reactivity (safety data sheet heading 10)

5.8. Disposal considerations

5.8.1. Disposal considerations (safety data sheet heading 13)

5.8.2. Information on recycling and methods of disposal for industry

5.8.3 Information on recycling and methods of disposal for the public

6. INFORMATION ON EXPOSURE FOR SUBSTANCES REGISTERED IN QUANTITIES

BETWEEN 1 AND 10 TONNES PER YEAR PER MANUFATCURER OR IMPORTER

6.1. Main use category:

6.1.1. (a) industrial use

(b) professional use

(c) consumer use

6.1.2 Specification for industrial and professional use:

(a) used in closed system

(b) use resulting in inclusion into or onto matrix

(c) non-dispersive use

(d) dispersive use

6.2. Significant route(s) of exposure:

6.2.1 Human exposure:

(a) oral

(b) dermal

(c) inhalatory

6.2.2 Environmental exposure

(a) water

(b) air

(c) solid waste

6.3. Pattern of exposure:

(a) accidental/infrequent

(b) occasional

(c) continuous /frequent

7 Note: conditions for not requiring a specific test that are set out in the appropriate test methods in Annex X itself that are not

repeated in column 2, also apply

ANNEX V

STANDARD INFORMATION REQUIREMENTS FOR SUBSTANCES

MANUFACTURED OR IMPORTED IN QUANTITIES OF 1 TONNE OR MORE

Column 1 of this Annex establishes the standard information required for substances

manufactured or imported in quantities of 1 tonne or more and meeting the criteria in Annex

Ic in accordance with Article 11 (1) (a). Any other relevant physicochemical, toxicological

and ecotoxicological information that is available shall be provided.

Column 2 of this Annex lists specific rules according to which the required standard

information may be omitted, replaced by other information, provided at a different stage or

adapted in another way. If the conditions are met under which column 2 of this Annex allows

adaptations, the registrant shall clearly state this fact and the reasons for each adaptation under

the appropriate headings in the registration dossier.

In addition to these specific rules, a registrant may adapt the required standard information set

out in column 1 of this Annex according to the general rules contained in Annex IX with the

exception of section 3 on substance-tailored exposure waiving. In this case as well, he shall

clearly state the reasons for any decision to adapt the standard information under the

appropriate headings in the registration dossier referring to the appropriate specific rule(s) in

column 2 or in Annexes IX or X7.

Before new tests are carried out to determine the properties listed in this Annex, all available

in vitro data, in vivo data, historical human data, data from valid (Q)SARs and data from

structurally related substances (read-across approach) shall be assessed first. In vivo testing

with corrosive substances at concentration/dose levels causing corrosivity shall be avoided.

Prior to testing, further guidance on testing strategies should be consulted in addition to this

Annex.

When, for certain endpoints, information is not provided for other reasons than those

mentioned in column 2 of this Annex or in Annex IX, this fact and the reasons shall also be

clearly stated.

5. INFORMATION ON THE PHYSICOCHEMICAL PROPERTIES OF THE SUBSTANCE

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

5.1. State of the substance at 20o C and 101,3 kPa

5.2. Melting/freezing point

5.2. The study does not need to be conducted below a lower limit of -20 °C.

5.3. Boiling point

5.3. The study does not need to be conducted:

– for gases; or

– for solids which either melt above 300 °C or decompose before boiling. In such cases the

boiling point under reduced pressure may be estimated or measured; or

– for substances which decompose before boiling (e.g. auto-oxidation, rearrangement,

degradation, decomposition, etc.).

5.4. Relative density

5.4. The study does not need to be conducted if:

– the substance is only stable in solution in a particular solvent and the solution density is

similar to that of the solvent. In such cases, an indication of whether the solution density is

higher or lower than the solvent density is sufficient; or

– the substance is a gas. In this case, an estimation based on calculation shall be made from its

molecular weight and the Ideal Gas Laws.

5.5. Vapour pressure

5.5. The study does not need to be conducted if

the melting point is above 300 °C.

If the melting point is between 200 °C and 300 °C, a limit value based on measurement or a

recognised calculation method is sufficient.

5.6. Surface tension

5.6. The study need only be conducted if:

- based on structure, surface activity is expected or can be predicted; or

- surface activity is a desired property of the material.

If the water solubility is below 1 mg/l at 20 °C the test does not need to be conducted.

5.7. Water solubility

5.7. The study does not need to be conducted if:

– the substance is hydrolytically unstable at pH 4, 7 and 9 (half-life less than 12 hours); or

– the substance is readily oxidisable in water.

If the substance appears "insoluble" in water, a limit test up to the detection limit of the analytical

method shall be performed.

5.8. Partition coefficient n-octanol/water

5.8. The study does not need to be conducted if the substance is inorganic. If the test cannot be

performed (e.g. the substance decomposes, has a high surface activity, reacts violently during the

performance of the test or does not dissolve in water or in octanol, or it is not possible to obtain a

sufficiently pure substance), a calculated value for log P as well as details of the calculation

method shall be provided.

5.9. Flash-point

5.9. The study does not need to be conducted if:

– the substance is inorganic; or

– the substance only contains volatile organic components with flash-points above 100 °C for

aqueous solutions; or

– the estimated flash-point is above 200 °C; or

–the flash-point can be accurately predicted by interpolation from existing characterised

materials.

5.10. Flammability

5.10. The study does not need to be conducted:

– if the substance is a solid which possesses explosive or pyrophoric properties. These properties

should always be considered before considering flammability; or

– for gases, if the concentration of the flammable gas in a mixture with inert gases is so low

that, when mixed with air, the concentration is all time below the lower limit; or

– for substances which spontaneously ignite when in contact with air.

5.11. Explosive properties

5.11. The study does not need to be conducted if:

– there are no chemical groups associated with explosive properties present in the molecule; or

– the substance contains chemical groups associated with explosive properties which include

oxygen and the calculated oxygen balance is less than –200; or

– the organic substance or a homogenous mixture of organic substances contains chemical

groups associated with explosive properties, but the exothermic decomposition energy is less

than 500 J/g and the onset of exothermic decomposition is below 500 ºC; or

– for mixtures of inorganic oxidising substances (UN Division 5.1) with organic materials, the

concentration of the inorganic oxidising substance is:

– less than 15%, by mass, if assigned to UN Packaging Group I (high hazard) or II

(medium hazard)

– less than 30%, by mass, if assigned to UN Packaging Group III (low hazard).

Note: Neither a test for propagation of detonation nor a test for sensitivity to detonative shock is

required if the exothermic decomposition energy of organic materials is less than 800 J/g.

5.12. Self-ignition temperature

5.12. The study does not need to be conducted:

– if the substance is explosive or ignites spontaneously with air at room temperature; or

– for liquids non flammable in air, e.g. no flash point up to 200 °C; or

– for gases having no flammable range; or

– for solids, if the substance has a melting point < 160 °C, or if preliminary results exclude

self-heating of the substance up to 400 °C.

5.13. Oxidising properties

5.13. The study does not need to be conducted if:

– the substance is explosive; or

– the substance is highly flammable; or

– the substance is an organic peroxide; or

– the substance is incapable of reacting exothermically with combustible materials, for example

on the basis of the chemical structure (e.g. organic substances not containing oxygen or

halogen atoms and these elements are not chemically bonded to nitrogen or oxygen, or

inorganic substances not containing oxygen or halogen atoms).

The full test does not need to be conducted for solids if the preliminary test clearly indicates that

the test substance has oxidising properties.

Note that as there is no test method to determine the oxidising properties of gaseous mixtures, the

evaluation of these properties must be realised by an estimation method based on the comparison

of the oxidising potential of gases in a mixture with that of the oxidising potential of oxygen in

air.

5.14. Granulometry

5.14. The study does not need to be conducted if the substance is marketed or used in a non solid

or granular form.

6. TOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

6.1. Skin irritation or skin corrosion

6.1. Steps 3 and 4 do not need to be conducted if:

The assessment of this endpoint shall comprise the following

consecutive steps:

(1) an assessment of the available human and animal data,

(2) an assessment of the acid or alkaline reserve,

(3) in vitro study for skin corrosion,

(4) in vitro study for skin irritation.

– the available information indicates that the criteria are met for classification as corrosive to the

skin or irritating to eyes; or

– the substance is flammable in air at room temperature; or

– the substance is classified as very toxic in contact with skin; or

– an acute toxicity study by the dermal route does not indicate skin irritation up to the limit dose

level (2000 mg/kg body weight).

6.2. Eye irritation

6.2. Step 3 does not need to be conducted if:

The assessment of this endpoint shall comprise the following

consecutive steps:

(1) an assessment of the available human and animal data,

(2) an assessment of the acid or alkaline reserve,

(3) in vitro study for eye irritation.

– the available information indicates that the criteria are met for classification as corrosive to the

skin or irritating to eyes; or

– the substance is flammable in air at room temperature; or

6.3. Skin sensitisation

6.3. Step 2 does not need to be conducted if:

The assessment of this endpoint shall comprise the following

– the available information indicates that the substance should be classified for skin sensitisation

consecutive steps:

(1) an assessment of the available human, animal and alternative

data,

(2) In vivo testing .

or corrosivity; or

– the substance is a strong acid (pH < 2.0) or base (pH > 11.5); or

– the substance is flammable in air at room temperature.

The Murine Local Lymph Node Assay (LLNA) is the first-choice method for in vivo testing. Only

in exceptional circumstances should another test be used. Justification for the use of another test

shall be provided.

6.4. Mutagenicity

6.4. Further mutagenicity studies shall be considered in case of a positive result.

6.4.1. In vitro gene mutation study in bacteria

6.5. Acute toxicity

6.5. The study/ies do(es) not generally need to be conducted if:

– the substance is classified as corrosive to the skin.

6.5.1. By oral route

The study need not be conducted if a study on acute toxicity by the inhalation route (6.5.2) is

available.

7. ECOTOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

7.1. Aquatic toxicity

7.1.1. Short-term toxicity testing on invertebrates

(preferred species Daphnia)

The registrant may consider long-term toxicity testing instead of

short-term.

7.1.1. The study does not need to be conducted if:

there are mitigating factors indicating that aquatic toxicity is unlikely to occur, for instance if the

substance is highly insoluble in water or the substance is unlikely to cross biological membranes;

or

– a long-term aquatic toxicity study on invertebrates is available; or

– adequate information for environmental classification and labelling is available.

The long-term aquatic toxicity study on Daphnia (Annex VII, 7.1.5) shall be considered if the

substance is poorly water soluble.

7.1.2. Growth inhibition study aquatic plants (algae

preferred)

7.1.2. The study does not need to be conducted if there are mitigating factors indicating that

aquatic toxicity is unlikely to occur for instance if the substance is highly insoluble in water or the

substance is unlikely to cross biological membranes.

7.2. Degradation

7.2.1. Biotic

7.2.1.1. Ready biodegradability

7.2.1.1The study does not need to be conducted if the substance is inorganic.

Any other relevant physicochemical, toxicological and ecotoxicological information that is available shall be provided.

8 Note: conditions for not requiring a specific test that are set out in the appropriate test methods in Annex X itself that are not

repeated in column 2, also apply

ANNEX VI

ADDITIONAL STANDARD INFORMATION REQUIREMENTS FOR

SUBSTANCES MANUFACTURED OR IMPORTED IN QUANTITIES OF 10 TONNES OR MORE

Column 1 of this Annex establishes the standard information required for all substances

manufactured or imported in quantities of 10 tonnes or more in accordance with

Article 11 (1) (b). Accordingly, the information required in column 1 of this Annex is

additional to that required in column 1 of Annex V. Any other relevant physicochemical,

toxicological and ecotoxicological information that is available shall be provided. Column 2

of this Annex lists specific rules according to which the required standard information may be

omitted, replaced by other information, provided at a different stage or adapted in another

way. If the conditions are met under which column 2 of this Annex allows adaptations, the

registrant shall clearly state this fact and the reasons for each adaptation under the appropriate

headings in the registration dossier.

In addition to these specific rules, a registrant may adapt the required standard information set

out in column 1 of this Annex according to the general rules contained in Annex IX. In this

case as well, he shall clearly state the reasons for any decision to adapt the standard

information under the appropriate headings in the registration dossier referring to the

appropriate specific rule(s) in column 2 or in Annexes IX or X8.

Before new tests are carried out to determine the properties listed in this Annex, all available

in vitro data, in vivo data, historical human data, data from valid (Q)SARs and data from

structurally related substances (read-across approach) shall be assessed first. In vivo testing

with corrosive substances at concentration/dose levels causing corrosivity shall be avoided.

Prior to testing, further guidance on testing strategies should be consulted in addition to this

Annex.

When, for certain endpoints, information is not provided for other reasons than those

mentioned in column 2 of this Annex or in Annex IX, this fact and the reasons shall also be

clearly stated.

6. TOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

6.1. Skin irritation

6.1.1. In vivo skin irritation

6.1.1. The study does not need to be conducted if:

– the substance is classified as corrosive to the skin or as a skin irritant; or

– the substance is a strong acid (pH < 2.0) or base (pH > 11.5); or

– the substance is flammable in air at room temperature; or

– the substance is classified as very toxic in contact with skin; or

– an acute toxicity study by the dermal route does not indicate skin irritation up to the limit dose

level (2000 mg/kg body weight); or

6.2. Eye irritation

6.2.1. In vivo eye irritation

6.2.1. The study does not need to be conducted if:

– the substance is classified as irritating to eyes with risk of serious damage to eyes; or

– the substance is classified as corrosive to the skin and provided that the registrant classified the

substance as eye irritant; or

– the substance is a strong acid (pH < 2.0) or base (pH > 11.5); or

– the substance is flammable in air at room temperature; or

6.4. Mutagenicity

6.4.2. In vitro cytogenicity study in mammalian cells

6.4.3. In vitro gene mutation study in mammalian cells, if a

negative result in Annex V, 6.4.1. and Annex VI, 6.4.2.

6.4.2. The study does not usually need to be conducted

- if adequate data from an in vivo cytogenicity test are available or

- the substance is known to be carcinogenic category 1 or 2 or mutagenic category 1, 2 or 3.

6.4.3. The study does not usually need to be conducted if adequate data from a reliable in vivo

mammalian gene mutation test are available.

6.4. Appropriate in vivo mutagenicity studies shall be considered in case of a positive result in

any of the genotoxicity studies in Annex V or VI.

6.5. Acute toxicity

6.5. The study/ies do(es) not generally need to be conducted if:

– the substance is classified as corrosive to the skin.

In addition to the oral route (6.5.1), for substances other than gases, the information mentioned

under 6.5.2. to 6.5.3. shall be provided for at least one other route. The choice for the second

route will depend on the nature of the substance and the likely route of human exposure. If there

is only one route of exposure, information for only that route need be provided.

6.5.2. By inhalation

6.5.2. Testing by the inhalation route is appropriate if exposure of humans via inhalation is likely

taking into account the vapour pressure of the substance and/or the possibility of exposure to

aerosols, particles or droplets of an inhalable size.

6.5.3. By dermal route

6.5.3. Testing by the dermal route is appropriate if:

(1) inhalation of the substance is unlikely; and

(2) skin contact in production and/or use is likely; and

(3) the physicochemical and toxicological properties suggest potential for a significant rate of

absorption through the skin.

6.6. Repeated dose toxicity

6.6.1. Short-term repeated dose toxicity study (28 days),

one species, male and female, most appropriate route of

administration, having regard to the likely route of human

exposure.

6.6.1. The short-term toxicity study (28 days) does not need to be conducted if:

– a reliable sub-chronic (90 days) or chronic toxicity study is available, provided that an

appropriate species, dosage, solvent and route of administration were used; or

– where a substance undergoes immediate disintegration and there are sufficient data on the

cleavage products; or

– relevant human exposure can be excluded.

The appropriate route shall be chosen on the following basis:

Testing by the dermal route is appropriate if:

(1) inhalation of the substance is unlikely; and

(2) skin contact in production and/or use is likely; and

(3) the physicochemical and toxicological properties suggest potential for a significant rate of

absorption through the skin.

Testing by the inhalation route is appropriate if exposure of humans via inhalation is likely taking

into account the vapour pressure of the substance and/or the possibility of exposure to aerosols,

particles or droplets of an inhalable size.

The sub-chronic toxicity study (90 days) (Annex VII, 6.6.2) shall be proposed by the registrant

if:

– the frequency and duration of human exposure indicates that a longer term study is

appropriate; and one of the following conditions is met:

– other available data indicate that the substance may have a dangerous property that cannot be

detected in a short-term toxicity study; or

– appropriately designed toxicokinetic studies reveal accumulation of the substance or its

metabolites in certain tissues or organs which would possibly remain undetected in a short-

term toxicity study but which are liable to result in adverse effects after prolonged exposure.

Further studies shall be proposed by the registrant or may be required by the Agency in

accordance with Article 39 or 40 in case of:

– failure to identify a NOAEL in the 28 or the 90 days study, unless the reason for the failure to

identify a NOAEL is absence of adverse toxic effects; or

– toxicity of particular concern (e.g., serious/severe effects); or

– indications of an effect for which the available evidence is inadequate for toxicological and/or

risk characterisation; In such cases it may also be more appropriate to perform specific

toxicological studies that are designed to investigate these effects (e.g., immunotoxicity,

neurotoxicity); or

– the route of exposure used in the initial repeated dose study was inappropriate in relation to the

expected route of human exposure and route-to-route extrapolation cannot be made; or

– particular concern regarding exposure (e.g. use in consumer products leading to exposure

levels which are close to the dose levels at which toxicity to humans may be expected ); or

– effects shown in substances with a clear relationship in molecular structure with the substance

being studied, were not detected in the 28 or the 90 days study.

6.7. Reproductive toxicity

6.7.1 This study does not need to be conducted if:

– the substance is known to be a genotoxic carcinogen and appropriate risk management

measures are implemented; or

– the substance is known to be a germ cell mutagen and appropriate risk management measures

are implemented; or

– relevant human exposure can be excluded.

6.7.1. Screening for reproductive/developmental toxicity,

one species (OECD 421), if there is no evidence from available

information on structurally related substances, from (Q)SAR

estimates or from in vitro methods that the substance may be a

developmental toxicant.

6.8 Toxicokinetics

6.8.1. Assessment of the toxicokinetic behaviour of the

substance to the extent that can be derived from the relevant

available information

7. ECOTOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

7.1.3. Short-term toxicity testing on fish: The registrant may

consider long-term toxicity testing instead of short-term.

7.1.3. The study does not need to be conducted if:

– there are mitigating factors indicating that aquatic toxicity is unlikely to occur, for instance if

the substance is highly insoluble in water or

the substance is unlikely to cross biological membranes; or

– a long-term aquatic toxicity study on fish is available.

Long-term aquatic toxicity testing as described in Annex VII shall be considered if the chemical

safety assessment according to Annex I indicates the need to investigate further effects on aquatic

organisms. The choice of the appropriate test(s) will depend on the results of the chemical safety

assessment.

The long-term aquatic toxicity study on fish (Annex VII, 7.1.6) shall be considered if the

substance is poorly water soluble.

7.1.4. Activated sludge respiration inhibition testing

7.1.4. The study does not need to be conducted if:

– there is no emission to a sewage treatment plant; or

– there are mitigating factors indicating that microbial toxicity is unlikely to occur, for instance

the substance is highly insoluble in water; or

– the substance is found to be readily biodegradable and the applied test concentrations are in

the range of concentrations that can be expected in the influent of a sewage treatment plant.

The study may be replaced by a nitrification inhibition test if available data show that the

substance is likely to be an inhibitor of microbial growth or function, in particular nitrifying

bacteria.

7.2. Degradation

7.2. Further degradation testing shall be considered if the chemical safety assessment according to

Annex I indicates the need to investigate further the degradation of the substance. The choice of

the appropriate test(s) will depend on the results of the chemical safety assessment.

7.2.2. Abiotic

7.2.2.1. Hydrolysis as a function of pH.

7.2.2.1. The study does not need to be conducted if:

– the substance is readily biodegradable; or

– the substance is highly insoluble in water.

7.3. Fate and behaviour in the environment

7.3.1. Adsorption/desorption screening

7.3.1. The study does not need to be conducted if:

– based on the physicochemical properties the substance can be expected to have a low potential

for adsorption (e.g. the substance has a low octanol water partition coefficient); or

– the substance and its relevant degradation products decompose rapidly.

9 Note: conditions for not requiring a specific test that are set out in the appropriate test methods in Annex X itself that are not

repeated in column 2, also apply

ANNEX VII

ADDITIONAL STANDARD INFORMATION REQUIREMENTS FOR

SUBSTANCES MANUFACTURED OR IMPORTED IN QUANTITIES OF 100 TONNES OR MORE

At the level of this Annex, the registrant must submit a proposal and a time schedule for

fulfilling the information requirements of this Annex in accordance with Article 11 (1) (c).

Column 1 of this Annex establishes the standard information required for all substances

manufactured or imported in quantities of 100 tonnes or more in accordance with

Article 11 (1) (c). Accordingly, the information required in column 1 of this Annex is

additional to that required in column 1 of Annexes V and VI. Any other relevant

physicochemical, toxicological and ecotoxicological information that is available shall be

provided. Column 2 of this Annex lists specific rules according to which the registrant may

propose to omit the required standard information, replace it by other information, provide it

at a later stage or adapt it in another way. If the conditions are met under which column 2 of

this Annex allows an adaptation to be proposed, the registrant shall clearly state this fact and

the reasons for proposing each adaptation under the appropriate headings in the registration

dossier.

In addition to these specific rules, a registrant may propose to adapt the required standard

information set out in column 1 of this Annex according to the general rules contained in

Annex IX. In this case as well, he shall clearly state the reasons for any decision to propose

adaptations to the standard information under the appropriate headings in the registration

dossier referring to the appropriate specific rule(s) in column 2 or in Annexes IX or X9.

Before new tests are carried out to determine the properties listed in this Annex, all available

in vitro data, in vivo data, historical human data, data from valid (Q)SARs and data from

structurally related substances (read-across approach) shall be assessed first. In vivo testing

with corrosive substances at concentration/dose levels causing corrosivity shall be avoided.

Prior to testing, further guidance on testing strategies should be consulted in addition to this

Annex.

When, for certain endpoints, it is proposed not to provide information for other reasons than

those mentioned in column 2 of this Annex or in Annex IX, this fact and the reasons shall also

be clearly stated.

5. INFORMATION ON THE PHYSICOCHEMICAL PROPERTIES OF THE SUBSTANCE

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

5.15. Stability in organic solvents and identity of relevant

degradation products

Only required if stability of the substance is considered to be

critical.

5.15. The study does not need to be conducted if the substance is inorganic.

5.16. Dissociation constant

5.16. The study does not need to be conducted if:

– the substance is hydrolytically unstable (half-life less than 12 hours) or is readily

oxidisable in water; or

– It is scientifically not possible to perform the test for instance if the analytical method is

not sensitive enough..

5.17. Viscosity

6. TOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

6.4. If there is a positive result in any of the in vitro genotoxicity studies in Annex V or VI

and there are no results available from an in vivostudy already, an appropriate in vivo

somatic cell genotoxicity study shall be proposed by the registrant.

If there is a positive result from in an in vivo somatic cell study available, the potential for

germ cell mutagenicity should be considered on the basis of all available data, including

toxicokinetic evidence. If no clear conclusions about germ cell mutagenicity can be made,

additional investigations shall be considered..

6.6. Repeated dose toxicity

6.6.1. Short-term repeated dose toxicity study (28 days),

one species, male and female, most appropriate route of

administration, having regard to the likely route of human

exposure, unless already provided as part of Annex VI

requirements or if tests according to 6.6.2 is proposed. In this

case, Section 3 of Annex IX shall not apply.

6.6.2. Sub-chronic toxicity study (90-day), one species,

rodent, male and female, most appropriate route of

administration, having regard to the likely route of human

exposure.

6.6.2. The sub-chronic toxicity study (90 days) does not need to be conducted if:

– a reliable short-term toxicity study (28 days) is available showing severe toxicity effects

according to the criteria for classifying the substance as R48, for which the observed

NOAEL-28 days, with the application of an appropriate uncertainty factor, allows the

extrapolation towards the NOAEL-90 days for the same route of exposure; or

– a reliable chronic toxicity study is available, provided that an appropriate species and

route of administration were used; or

– a substance undergoes immediate disintegration and there are sufficient data on the

cleavage products (both for systemic effects and effects at the site of uptake); or

– the substance is unreactive, insoluble and not inhalable and there is no evidence of

absorption and no evidence of toxicity in a 28-day “limit test”, particularly if such a

pattern is coupled with limited human exposure.

The appropriate route shall be chosen on the following basis:

Testing by the dermal route is appropriate if:

(1) skin contact in production and/or use is likely; and

(2) the physicochemical properties suggest a significant rate of absorption through the skin;

and

(3) one of the following conditions is met:

– toxicity is observed in the acute dermal toxicity test at lower doses than in the oral

toxicity test; or

– systemic effects or other evidence of absorption is observed in skin and/or eye irritation

studies; or

– in vitro tests indicate significant dermal absorption; or

– significant dermal toxicity or dermal penetration is recognised for structurally-related

substances.

Testing by the dermal route is inappropriate if:

- inhalation of the substance is unlikely; and

- skin contact in production and/or use is likely; and

- the physicochemical and toxicological properties suggest potential for significant

absorption through the skin.

Testing by the inhalation route is appropriate if:

- exposure of humans via inhalation is likely taking into account the vapour pressure of the

substance and/or the possibility of exposure to aerosols, particles or droplets of an inhalable

size.

Further studies shall be proposed by the registrant or may be required by the Agency in

accordance with Articles 39 or 40:

– failure to identify a NOAEL in the 90 days study unless the reason for the failure to

identify a NOAEL is absence of adverse toxic effects; or

– toxicity of particular concern (e.g. serious/severe effects); or

– indications of an effect for which the available evidence is inadequate for toxicological

and/or risk characterisation; In such cases it may also be more appropriate to perform

specific toxicological studies that are designed to investigate these effects (e.g.

immunotoxicity, neurotoxicity); or

– particular concern regarding exposure (e.g. use in consumer products leading to

exposure levels which are high relative to the dose levels at which toxicity to humans

may be expected).

6.7. Reproductive toxicity

6.7. The studies do not need to be conducted if:

– the substance is known to be a genotoxic carcinogen and appropriate risk management

measures are implemented; or

– the substance is known to be a germ cell mutagen and appropriate risk management

measures are implemented; or

– the substance is of low toxicological activity (no evidence of toxicity seen in any of the

tests available), it can be proven from toxicokinetic data that no systemic absorption

occurs via relevant routes of exposure (e.g. plasma/blood concentrations below

detection limit using a sensitive method and absence of the substance and of metabolites

of the substance in urine, bile or exhaled air) and there is no or no significant human

exposure.

6.7.2. Developmental toxicity study, one species, most

appropriate route of administration, having regard to the likely

route of human exposure (Annex X B.31 or OECD 414).

6.7.2 The study shall be initially performed on one species. A decision on the need to

perform a study at this tonnage level or the next on a second species should be based on the

outcome of the first test and all other relevant available data.

6.7.3. Two-generation reproductive toxicity study, one

species, male and female, most appropriate route of

administration, having regard to the likely route of human

exposure, if the 28-day or 90-day study indicates adverse effects

on reproductive organs or tissues.

6.7.3 The study shall be initially performed on one species. A decision on the need to

perform a study at this tonnage level or the next on a second species should be based on the

outcome of the first test and all other relevant available date.

7. ECOTOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

7.1. Aquatic toxicity

7.1. Long-term toxicity testing shall be proposed by the registrant if the chemical safety

assessment according to Annex I indicates the need to investigate further the effects on

aquatic organisms. The choice of the appropriate test(s) depends on the results of the

chemical safety assessment.

7.1.5. Long-term toxicity testing on invertebrates (preferred

species Daphnia), (unless already provided as part of Annex V

requirements)

–

7.1.6. Long-term toxicity testing on fish, (unless already

provided as part of Annex VI requirements)

The information shall be provided for one of the following 7.1.6.1,

7.1.6.2 or 7.1.6.3.

7.1.6.1 Fish early-life stage (FELS) toxicity test

7.1.6.2 Fish short-term toxicity test on embryo and sac-fry

stages

7.1.6.3 Fish, juvenile growth test

7.2. Degradation

7.2. Further biotic degradation testing shall be proposed by the registrant if the chemical

safety assessment according to Annex I indicates the need to investigate further the

degradation of the substance and its degradation products. The choice of the appropriate

test(s) depends on the results of the chemical safety assessment but may include simulation

testing in appropriate media (eg. water, sediment or soil).

7.2.1. Biotic

7.2.1.2. Simulation testing on ultimate degradation in surface water

7.2.1.2. The study need not be conducted if:

– the substances is highly insoluble in water ;

– the substance is readily biodegradable.

7.2.1.3. Soil simulation testing (for substances with a high

potential for adsorption to soil)

7.2.1.3. The study need not be conducted:

– if the substance is readily biodegradable; or

– if direct and indirect exposure of soil is unlikely.

7.2.1.4. Sediment simulation testing (for substances with a high

potential for adsorption to sediment)

7.2.1.4. The study need not be conducted:

– if the substance is readily biodegradable; or

– if direct or indirect exposure of soil is unlikely.

7.2.3. Identification of degradation products

7.2.3. Unless the substance is readily biodegradable

7.3. Fate and behaviour in the environment

7.3.2. Bioaccumulatio in aquatic species, preferably fish

7.3.2. The study need not be conducted if:

– the substance has a low potential for bioaccumulation (for instance a log Kow < 3)

and/or a low potential to cross biological membranes; or

– direct and indirect exposure of the aquatic compartment is unlikely.

7.3.3. Further information on adsorption/desorption

depending on the results of the study required in Annex VI

7.3.3. The study need not be conducted if:

– based on the physicochemical properties the substance can be expected to have a low

potential for adsorption (e.g. the substance has a low octanol water partition

coefficient); or

– the substance and its degradation products decompose rapidly.

7.4. Effects on terrestrial organisms

7.4. These studies do not need to be conducted if direct and indirect exposure of the soil

compartment is unlikely.

In the absence of toxicity data for soil organisms, the equilibrium partitioning method may

be applied to assess the exposure to soil organisms. The choice of the appropriate tests

depends on the outcome of the chemical safety assessment.

In particular for substances that have a high potential to adsorb to soil or that are very

persistent, the registrant shall consider long-term toxicity testing instead of short-term.

7.4.1. Short-term toxicity to invertebrates (preferred species,

earthworms)

7.4.2. Effects on soil micro-organisms

7.4.3. Short-term toxicity to plants

9. METHODS OF DETECTION AND ANALYSIS

Description of the analytical methods shall be provided on request, for the relevant compartments for which studies were performed using the

analytical method concerned. If the analytical methods are not available this shall be justified.

10 Note: conditions for not requiring a specific test that are set out in the appropriate test methods in Annex X itself that are not

repeated in column 2, also apply

ANNEX VIII

ADDITIONAL STANDARD INFORMATION REQUIREMENTS FOR

SUBSTANCES MANUFACTURED OR IMPORTED IN QUANTITIES OF 1000 TONNES OR MORE

At the level of this Annex, the registrant must submit a proposal and a time schedule for

fulfilling the information requirements of this Annex in accordance with Article 11 (1) (d).

Column 1 of this Annex establishes the standard information required for all substances

manufactured or imported in quantities of 1000 tonnes or more in accordance with

Article 11 (1) (d). Accordingly, the information required in column 1 of this Annex is

additional to that required in column 1 of Annexes V, VI and VII. Any other relevant

physicochemical, toxicological and ecotoxicological information that is available shall be

provided. Column 2 of this Annex lists specific rules according to which the registrant may

propose to omit the required standard information, replace it by other information, provide it

at a later stage or adapt it in another way. If the conditions are met under which column 2 of

this Annex allows an adaptation to be proposed, the registrant shall clearly state this fact and

the reasons for proposing each adaptation under the appropriate headings in the registration

dossier.

In addition to these specific rules, a registrant may propose to adapt the required standard

information set out in column 1 of this Annex according to the general rules contained in

Annex IX. In this case as well, he shall clearly state the reasons for any decision to propose

adaptations to the standard information under the appropriate headings in the registration

dossier referring to the appropriate specific rule(s) in column 2 or in Annexes IX or X10.

Before new tests are carried out to determine the properties listed in this Annex, all available

in vitro data, in vivo data, historical human data, data from valid (Q)SARs and data from

structurally related substances (read-across approach) shall be assessed first. In vivo testing

with corrosive substances at concentration/dose levels causing corrosivity shall be avoided.

Prior to testing, further guidance on testing strategies should be consulted in addition to this

Annex.

When, for certain endpoints, it is proposed not to provide information for other reasons than

those mentioned in column 2 of this Annex or in Annex IX, this fact and the reasons shall also

be clearly stated.

6. TOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

6.4. If there is a positive result in any of the in vitro genotoxicity studies in Annex V or VI, a

second in vivo somatic cell test may be necessary, depending on the quality and relevance of all

the available data.

If there is a positive result from in an in vivo somatic cell study available, the potential for germ

cell mutagenicity should be considered on the basis of all available data, including toxicokinetic

evidence. If no clear conclusions about germ cell mutagenicity can be made, additional

investigations shall be considered.

6.6.3. A long-term repeated toxicity study (= 12 months) may be proposed by the registrant or

required by the Agency in accordance with Articles 39 or 40 if the frequency and duration of

human exposure indicates that a longer term study is appropriate and one of the following

conditions is met:

– serious or severe toxicity effects of particular concern were observed in the 28 days or 90 days

study for which the available evidence is inadequate for toxicological evaluation or risk

characterisation; or

– effects shown in substances with a clear relationship in molecular structure with the substance

being studied were not detected in the 28 days or 90 days study; or

– the substance may have a dangerous property that cannot be detected in a 90 days study.

6.6.4 Further studies shall be proposed by the registrant or may be required by the Agency in

accordance with Articles 39 or 40in case of:

– toxicity of particular concern (e.g. serious/severe effects); or

– indications of an effect for which the available evidence is inadequate for toxicological

evaluation and/or risk characterisation; In such cases it may also be more appropriate to

perform specific toxicological studies that are designed to investigate these effects (e.g.

immunotoxicity, neurotoxicity); or

– particular concern regarding exposure (e.g. use in consumer products leading to exposure

levels which are close to the dose levels at which toxicity is observed).

6.7. Reproductive toxicity

6.7 The studies need not be conducted if:

– the substance is known to be a genotoxic carcinogen and appropriate risk management

measures are implemented; or

- the substance is known to be a germ cell mutagen and appropriate risk management measures

are implemented; or

- the substance is of low toxicological activity (no evidence of toxicity seen in any of the tests

available), it can be proven from toxicokinetic data that no systemic absorption occurs via

relevant routes of exposure (e.g. plasma/blood concentrations below detection limit using a

sensitive method and absence of the substance and of metabolites of the substance in urine, bile or

exhaled air) and there is no or no significant human exposure.

6.7.2. Developmental toxicity study, one species, most

appropriate route of administration, having regard to the likely

route of human exposure (Annex X B.31 or OECD 414).

6.7.3. Two-generation reproductive toxicity study, one

species, male and female, most appropriate route of

administration, having regard to the likely route of human

exposure, unless already provided as part of Annex VII

requirements

–

6.9.1 Carcinogenicity study

6.9.1 A carcinogenicity study may be proposed by the registrant or may be required by the

Agency in accordance with Articles 39 or 40if:

– the substance has a widespread dispersive use or there is evidence of frequent or long-term

human exposure; and

– the substance is classified as mutagen category 3 or there is evidence from the repeated dose

study(ies) that the substance is able to induce hyperplasia and/or pre-neoplastic lesions.

If the substances is classified as mutagen category 1 or 2, the default presumption would be that a

genotoxic mechanism for carcinogenicity is likely. In these cases, a carcinogenicity test will

normally not be required.

7. ECOTOXICOLOGICAL INFORMATION

COLUMN 1

STANDARD INFORMATION REQUIRED

COLUMN 2

SPECIFIC RULES FOR ADAPTATION FROM COLUMN 1

7.2. Degradation

7.2. Further biotic degradation testing shall be proposed if the chemical safety assessment

according to Annex I indicates the need to investigate further the degradation of the

substance and its degradation products. The choice of the appropriate test(s) depends on

the results of the chemical safety assessment.

7.2.1. Biotic

7.3. Fate and behaviour in the environment

7.3.4. Further information on the environmental fate and

behaviour of the substance and/ore degradation products

7.3.4 Further testing shall be proposed by the registrant or may be required by the Agency in

accordance with Article 39 or 40 if the chemical safety assessment according to Annex I indicates

the need to investigate further the fate and behaviour of the substance. The choice of the

appropriate test(s) depends on the results of the chemical safety assessment.

7.4. Effects on terrestrial organisms

7.4. Long-term toxicity testing shall be proposed by the registrant if the results of the chemical

safety assessment according to Annex I indicates the need to investigate further the effects

of the substance and/or degradation products on terrestrial organisms. The choice of the

appropriate test(s) depends on the outcome of the chemical safety assessment.

These studies do not need to be conducted if direct and indirect exposure of the soil

compartment is unlikely.

7.4.4. Long-term toxicity testing on invertebrates , unless

already provided as part of Annex VII requirements.

7.4.6. Long-term toxicity testing on plants, unless already

provided as part of Annex VII requirements.

7.5.1. Long-term toxicity to sediment organisms

7.5.1. Long-term toxicity testing shall be proposed by the registrant if the results of the chemical

safety assessment indicates the need to investigate further the effects of the substance

and/or relevant degradation products on sediment organisms. The choice of the appropriate

test(s) depends on the results of the chemical safety assessment.

7.6.1. Long-term or reproductive toxicity to birds

7.6.1. Any need for testing should be carefully considered taking into account the large

mammalian dataset that is usually available at this tonnage level.

9. METHODS OF DETECTION AND ANALYSIS

Description of the analytical methods shall be provided on request, for the relevant compartments for which studies were performed using the analytical

method concerned. If the analytical methods are not available this shall be justified.

ANNEX IX

GENERAL RULES FOR ADAPTATION OF THE STANDARD TESTING REGIME

SET OUT IN ANNEXES V TO VIII

Annexes V to VIII set out the information requirements for all substances manufactured

or imported in quantities of:

– 1 tonne or more in accordance with Article 11 (1) (a),

– 10 tonnes or more in accordance with Article 11 (1) (b),

– 100 tonnes or more in accordance with Article 11 (1) (c), and

– 1000 tonnes or more in accordance with Article 11 (1) (d).

In addition to the specific rules set out in Column 2 of Annexes V to VIII, a registrant

may adapt the standard testing regime in accordance with the general rules set out in

Section 1 of this Annex. Under evaluation competent authorities of evaluating Member

States may assess these adaptations to the standard testing regime.

1. TESTING DOES NOT APPEAR SCIENTIFICALLY NECESSARY

1.1. Use of existing data

1.1.1. Data on physical-chemical properties from experiments not carried out

according to GLP or Annex X

Data shall be considered to be equivalent to data generated by the corresponding test in

Annex X if the following conditions are met:

(1) adequacy for the purpose of classification and labelling and/or risk assessment,

(2) sufficient documentation is provided to assess the adequacy of the study and

(3) the data are valid for the endpoint being investigated and the study is performed

using an acceptable level of quality assurance.

1.1.2. Data from animal and non-animal experiments not carried out according to

GLP or Annex X

Data shall be considered to be equivalent to data generated by the corresponding test in

Annex X if the following conditions are met:

(1) adequacy for the purpose of classification and labelling and/or risk assessment,

(2) adequate and reliable coverage of the key parameters foreseen to be investigated

in the corresponding test in Annex X,

(3) exposure duration comparable to or longer than the corresponding test in

Annex X if exposure duration is a relevant parameter, and

(4) adequate and reliable documentation of the study is provided

1.1.3. Historical human data

Historical human data, such as epidemiological studies on exposed populations, accidental or

occupational exposure data and clinical studies, shall be considered.

The strength of the data for a specific health effect depends, among other things, on the type

of analysis and on the parameters covered and on the magnitude and specificity of the

response and consequently the predictability of the effect. Criteria for assessing the adequacy

of the data include:

(1) the proper selection and characterisation of the exposed and control groups,

(2) adequate characterisation of exposure,

(3) sufficient length of follow-up for disease occurrence,

(4) valid method for observing an effect,

(5) proper consideration of bias and confounding factors, and

(6) a reasonable statistical reliability to justify the conclusion.

In all cases adequate and reliable documentation shall be provided.

1.2. Weight of evidence

There may be sufficient weight of evidence from several independent sources of information

leading to the assumption/conclusion that a substance has or has not a particular dangerous

property, while the information from each single source alone is regarded insufficient to

support this notion.

There may be sufficient weight of evidence from the use of newly developed test methods,

not yet included in Annex X or from an international test method recognised by the Agency as

being equivalent, leading to the conclusion that a substance has or has not a particular

dangerous property.

Where sufficient weight of evidence for the presence or absence of a particular dangerous

property is available:

– further testing on vertebrate animals for that property shall be omitted,

– further testing not involving vertebrate animals may be omitted.

In all cases adequate and reliable documentation shall be provided.

1.3. Structure-activity relationship (SAR)

Results obtained from valid qualitative or quantitative structure-activity relationship models

((Q)SARs) may indicate the presence or absence of a certain dangerous property. Results of

(Q)SARs may be used instead of testing when the following conditions are met:

– results are derived from a (Q)SAR model whose scientific validity has been

established,

– the substance falls within the applicability domain of the (Q)SAR model,

– results are adequate for the purpose of classification and labelling and/or risk

assessment, and

– adequate and reliable documentation of the applied method is provided.

The Agency in collaboration with the Commission, Member States and interested parties shall

develop and provide guidance in assessing which (Q)SARs will meet these conditions and

provide examples.

1.4. In vitro methods

Results obtained from suitable in vitro methods may indicate the presence of a certain

dangerous property or may be important in relation to a mechanistic understanding, which

may be important for the assessment. In this context, “suitable” means sufficiently well

developed according to internationally agreed test development criteria (e.g. the ECVAM

criteria for the entry of a test into the prevalidation process). Depending on the potential risk,

immediate confirmation requiring testing beyond the information foreseen in Annex V or VI

or proposed confirmation requiring testing beyond the information foreseen in Annex VII or

VIII for the respective tonnage level may be necessary.

If the results obtained from the use of such in vitro methods do not indicate a certain

dangerous property, the relevant test shall nevertheless be carried out at the appropriate

tonnage level to confirm the negative result, unless testing is not required in accordance with

Annexes V to VIII or the other rules in Annex IX.

Such confirmation may be waived, if the following conditions are met:

(1) results are derived from an in vitro method whose scientific validity has been

established by a validation study, according to internationally agreed validation

principles,

(2) results are adequate for the purpose of classification and labelling and/or risk

assessment, and

(3) adequate and reliable documentation of the applied method is provided.

1.5. Grouping of substances and read-across approach

Substances whose physicochemical, toxicological and ecotoxicological properties are likely to

be similar or follow a regular pattern as a result of structural similarity may be considered as a

group, or "category" of substances. Application of the group concept requires that

physicochemical properties, human health effects and environmental effects or environmental

fate may be predicted from data for a reference substance within the group by interpolation to

other substances in the group (read-across approach). This avoids the need to test every

substance for every endpoint.

The similarities may be based on:

(1) a common functional group,

(2) the common precursors and/or the likelihood of common breakdown products via

physical and biological processes, which result in structurally similar chemicals, or

(3) a constant pattern in the changing of the potency of the properties across the

category.

If the group concept is applied, substances shall be classified and labelled on this basis.

In all cases results should:

– be adequate for the purpose of classification and labelling and/or risk assessment,

– have adequate and reliable coverage of the key parameters foreseen to be

investigated in the corresponding test in Annex X

– cover an exposure duration comparable to or longer than the corresponding test in

Annex X if exposure duration is a relevant parameter, and

– adequate and reliable documentation of the applied method shall be provided.

2. TESTING IS TECHNICALLY NOT POSSIBLE

Testing for a specific endpoint may be omitted, if it is technically not possible to conduct the

study as a consequence of the properties of the substance: e.g. very volatile, highly reactive or

unstable substances cannot be used, mixing of the substance with water may cause danger of

fire or explosion or the radio-labelling of the substance required in certain studies may not be

possible. The guidance given in the test guidelines of Annex X, more specifically on the

technical limitations of a specific method, shall always be respected.

3. SUBSTANCE-TAILORED EXPOSURE-DRIVEN TESTING

Testing in accordance with Annexes VII and VIII may be omitted, based on the exposure

scenario(s) developed in the Chemical Safety Report.

In all cases, adequate justification and documentation shall be provided.

ANNEX X

TEST METHODS

The Presidency is not proposing to amend the testing methods as they are agreed at OECD

level and uncontroversial. The Presidency is however minded to provide for these test

methods to be adopted through comitology instead, and would welcome the views of Member

States on this issue.

ANNEX XI

GENERAL PROVISIONS FOR DOWNSTREAM USERS

TO ASSESS SUBSTANCES AND PREPARE CHEMICAL SAFETY REPORTS

Introduction

The purpose of this Annex is to set out how downstream users are to assess and document that

the risks arising from the substance(s) they use are adequately controlled during their use for a

use not covered by the safety data sheet supplied to them and that other users further down the

supply chain can adequately control the risks. The assessment shall cover the life-cycle of the

substance, from its receipt by the downstream user, for his own uses and for his identified

uses further down the supply chain. The assessment shall consider the use of the substance on

its own, in a preparation or in an article.

In carrying out the chemical safety assessment and producing the Chemical Safety Report, the

downstream user shall take account of information received from the supplier of the chemical

in accordance with Article 29 and 30 of this Regulation. Where available and appropriate, an

assessment carried out under Community legislation, (e.g. risk assessments completed under

Regulation 793/93) shall be taken into account in the chemical safety assessment and be

reflected in the Chemical Safety Report. Deviations from such assessments shall be justified.

Assessments carried out under other international and national programmes may also be taken

into account.

The process which the downstream user goes through in carrying out the chemical safety

assessment and in producing his Chemical Safety Report, involves three steps:

Step 1: Development of exposure scenario(s)

The downstream user shall develop exposure scenarios for uses not covered in a safety data

sheet supplied to him in accordance with Section 5 of Annex I.

Step 2: If necessary, a refinement of the hazard assessment by the supplier;

If the downstream user considers the hazard and PBT assessments reported in the Safety Data

Sheet supplied to him to be appropriate, then no further hazard assessment or PBT and vPvB

assessment is necessary. In this case he shall use the relevant information reported by the

supplier for the risk characterisation. This shall be stated in the chemical safety report.

If the downstream user considers the assessments reported in the Safety Data Sheet supplied

to him to be inappropriate, then he shall carry out the relevant assessments in accordance with

Annex I, sections 1 through 4 as appropriate to him.

In those cases where the downstream user considers that information in addition to that

provided by the supplier is necessary for producing his Chemical Safety Report the

downstream user shall gather this information. Where this information can only be obtained

by testing on vertebrate animals, he shall submit a proposal for a testing strategy to the

Agency in accordance with Article 35. He shall explain why he considers that additional

information is necessary. While waiting for results of further testing, he shall record in his

chemical safety report the risk management measures intended to manage the risks being

explored that he has put in place.

On completion of any additional testing, the downstream user shall revise the Chemical

Safety Report, and his Safety Data Sheet if he is required to prepare one, as appropriate.

Step 3: Risk characterisation.

A risk characterisation shall be carried out for each new exposure scenario as prescribed in

section 6 of Annex I. The risk characterisation shall be presented under the relevant heading

of the Chemical Safety Report and summarised in the Safety Data Sheet under the relevant

heading(s).

When generating an exposure scenario it will be necessary to make initial assumptions about

the operating conditions and risk managements measures. If the initial assumptions lead to a

risk characterisation indicating inadequate protection of human health and the environment,

then it shall be necessary to carry out an iterative process with amendment of one or a number

of factors until adequate control can be demonstrated. This may require the generation of

additional hazard or exposure information or appropriate alteration of the process, operating

conditions or risk management measures. Therefore, iterations may be made between on the

one hand developing and revising an (initial) exposure scenario, which includes developing

and implementing risk management measures, and on the other hand generating further

information to produce the definitive exposure scenario. The purpose of generating further

information is to establish a more precise risk characterisation, based on a refined hazard

assessment and/or exposure assessment.

The downstream user shall produce a Chemical Safety Report detailing his chemical safety

assessment using part C, sections 5 and 6, of the format set out in Section 7 of Annex I and

the other sections of this format, if appropriate.

Part A of the Chemical Safety Report shall include a declaration that the risk management

measures outlined in the relevant exposure scenarios are implemented by the downstream user

for his own uses and that the risk management measures outlined in the exposure scenarios

for the identified uses are communicated down the supply chain.

ANNEX XII

CRITERIA FOR THE IDENTIFICATION OF PERSISTENT, BIOACCUMULATIVE AND TOXIC

SUBSTANCES, AND VERY PERSISTENT AND VERY BIOACCUMULATIVE SUBSTANCES

The UK Presidency is not proposing any amendments to the Commission proposal for this

Annex.

ANNEX XIII

LIST OF SUBSTANCES SUBJECT TO AUTHORISATION

The UK Presidency is not proposing any amendments to the Commission proposal for this

Annex.

ANNEX XIV

DOSSIERS

The purpose of this Annex is to lay down general principles for the Agency or the Member

States to submit and justify proposals for restrictions, harmonised classification and labelling

or identification of a substance as a PBT, a VPVB, or a substance of equivalent concern on a

Community-wide basis.

CONTENT OF DOSSIERS

Part A – Proposal

This shall include details of:

a) the restriction(s) proposed including the substance(s) concerned and the manufacture,

use(s) and/or placing on the market affected, or

b) the substance(s) concerned and the harmonised classification and labelling proposed,

or

c) the substance(s) concerned and whether it is proposed to be identified as a PBT

according to Article 54(d), a VPVB according to Article 54(e), or a substance of

equivalent concern according to Article 54(f).

Part B – Technical and Scientific Basis

An assessment of the hazard or risk.

For a restrictions proposal, the need for action on a Community wide basis beyond any

measures that are already in place shall be demonstrated. The relevant parts of Sections 1 – 6

of Annex I needed to support the proposal shall be completed.

For a classification and labelling proposal, the relevant parts of Sections 1 – 3 of Annex I

needed to support the proposal shall be completed.

For a proposal to identify a substance as a PBT or a VPVB, the relevant parts of Sections 1-4

of Annex I needed to support the proposal shall be completed.

Any relevant data from registration dossiers and any information available shall be

considered. Statements regarding lack of relevant information are not required.

For information which has not been previously submitted to the Agency and that is used in the

dossier, a robust study summary shall be prepared and submitted to the Agency in the format

specified by the Agency in accordance with Article 108.

The dossier may consider emissions of the substance from any source.

The dossier may develop exposure scenarios, which take into account the actual risk

management measures in place.

The dossier shall contain scientific justification of any grouping of substances.

The Member State submitting the dossier shall, on request, provide the Agency or the

Commission with any or all information on which the dossier was based or to which reference

is made in the dossier.

Part C – Justification for Action at Community Level for a Restriction Proposal

a) Evidence that implemented risk management measures (including those identified in

registrations under Articles 9 to 13) are not sufficient.

b) Justification for the proposal that action is required on a Community-wide basis.

c) Identification of the available options for addressing the concerns identified in Part

B. For restrictions, this includes evidence that alternative substances and/or processes

have been considered in the preparation of the proposal.

d) Identification of the administrative, legal or other tools by which the available

options can be implemented.

e) Justification for the option and implementation method selected. The options shall be

evaluated using the following criteria:

i) effectiveness: the action must be targeted to the effects or exposures that cause

the risks identified and must be capable of reducing these risks to a level where

the risk is adequately controlled within a reasonable period of time;

ii) practicality: the action must be implementable, enforceable and manageable.

Priority should be given to those measures that can be implemented with the

existing infrastructure;

iii) monitorability: the ability to monitor the result of the implementation of the

proposed action;

iv) a socio-economic assessment may be made of the costs and benefits and the

wider impact of the proposed action on the producers, importers, downstream

users and/or consumers of the substance and on other parties as well as the

environment. This assessment should follow Annex XV.

Part D – Other Information

a) A statement about which interested parties have been consulted about the proposed

action and, if appropriate, how their views have been taken into account.

b) Other relevant information.

ANNEX XV

SOCIO-ECONOMIC ANALYSIS

This Annex outlines the information that may be addressed by those submitting a socio-

economic analysis (SEA) with an application for authorisation, as specified in

Article 59 (5) (a), or in connection with a proposed restriction, as specified in

Article 66(3)(b).

The Agency shall prepare guidance for the preparation of SEAs. SEAs, or contributions to

them, shall be submitted in the format specified by the Agency in accordance with Article

108.

However, the level of detail and scope of the SEA, or contributions to them, shall be the

responsibility of the applicant for authorisation, or, in the case of a proposed restriction, the

interested party. The information provided can address the socio-economic impacts at any

level.

An SEA may include the following elements:

– Impact of a granted or refused authorisation on the applicant(s), or, in the case of a

proposed restriction, the impact on industry (e.g. manufacturers and importers). The impact

on all other actors in the supply chain, downstream users and associated businesses in terms

of commercial consequences such as impact on investment, research and development,

innovation, one-off and operating costs (e.g. compliance; transitional arrangements; changes

to existing processes, reporting and monitoring systems; installation of new technology etc)

taking into account general trends in the market and technology.

– Impacts of a granted or refused authorisation, or a proposed restriction, on

consumers. For example, product prices, changes in composition or quality or performance of

products, availability of products, consumer choice, as well as effects on health and the

environment to the extent that these affect consumers.

– Social implications of a granted or refused authorisation, or a proposed restriction.

For example job security and employment.

– Availability, suitability, and technical feasibility of alternative substances and/or

technologies, and economic consequences thereof, and information on the rates of, and

potential for, technological change in the sector(s) concerned. In the case of an application for

authorisation, the social and/or economic impacts of using any available alternatives

identified in Article 59(5)(b).

– Wider implications on trade, competition and economic development (in particular

for SMEs and in relation to third countries) of a granted or refused authorisation, or a

proposed restriction. This may include consideration of local, regional, national or

international aspects.

– In the case of a proposed restriction, proposals for other regulatory or non-regulatory

measures that could meet the aim of the proposed restriction (this shall take account of

existing legislation). This should include an assessment of the effectiveness and the costs

linked to alternative risk management measures.

– In the case of a proposed restriction or refused authorisation, the benefits for health

and the environment as well as the social and economic benefits of the proposed restriction.

For example, worker health, environmental performance and the distribution of these benefits,

for example, geographically, population groups.

– An SEA may also address any other issue that is considered to be relevant by the

applicant(s) or interested party.

ANNEX XVI

Restrictions on the manufacture, placing on the market and use of certain dangerous

substances, preparations and articles

The UK Presidency is not proposing any amendments to the Commission proposal for this

Annex.

ANNEX XVII

PERSISTENT ORGANIC POLLUTANTS (POPS)

The UK Presidency is not proposing any amendments to the Commission proposal for this

Annex.